| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0346 |
| --- | --- |

| Title: | AHG9: On Slice Header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Tel: Email: | +46 721 88 21 69 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution contains two proposals related to the slice header:

- Allow LMCS to be turned on and off at slice level.

- Allow selection at slice level if signalled scaling lists shall be applied or not.

# Introduction

When the Picture Header NAL unit was introduced in VVC, parameters related to LMCS and scaling lists where moved from the slice header to the picture header. In earlier drafts of VVC, LMCS and scaling lists could be turned on and off at the slice level, even though slice_lmcs_aps_id and slice_scaling_list_aps_id had to be the same in all slices of a picture. It is asserted that it is beneficial with slice level on/off control of these tools, and it is proposed to add two flags in the slice header to bring back support for this in the VVC draft.

# Proposal

The following is proposed:

- It is proposed to allow LMCS to be turned on or off on per slice basis if LMCS is enabled for the picture.

- It is proposed to allow scaling list to be chosen on per slice basis if scaling list is enabled for the picture.

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( pic_lmcs_enabled_flag ) |  |
| slice_lmcs_enabled_flag | u(1) |
| if( pic_scaling_list_enabled_flag ) |  |
| slice_scaling_list_present_flag | u(1) |
| if( entry_point_offsets_present_flag && NumEntryPoints > 0 ) { |  |
| offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| if( slice_header_extension_present_flag ) { |  |
| slice_header_extension_length | ue(v) |
| for( i = 0; i < slice_header_extension_length; i++) |  |
| slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| byte_alignment( ) |  |
| } |  |

slice_lmcs_enabled_flag equal to 1 specifies that luma mapping with chroma scaling is enabled for the current slice. slice_lmcs_enabled_flag equal to 0 specifies that luma mapping with chroma scaling is not enabled for the current slice. When slice_lmcs_enabled_flag is not present, it is inferred to be equal to 0.

slice_scaling_list_present_flag equal to 1 specifies that the scaling list data used for the current slice is derived based on the scaling list data contained in the referenced scaling list APS with aps_params_type equal to SCALING_APS  and and adaptation_parameter_set_id equal to pic_scaling_list_aps_id. slice_scaling_list_present_flag equal to 0 specifies that the scaling list data used for the current pictureis the default scaling list data derived specified in clause 7.4.3.18 (Scaling list data semantics). When not present, the value of slice_scaling_list_present_flag is inferred to be equal to 0.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).