| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0349-v2 |
| --- | --- |

| Title: | Combination of Triangle and Weighted-Prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | P.Bordes, T. Poirier, F. Le Léannec | Email: | philippe.bordes@interdigital.com tangi.poirier@interdigital.com fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In the current VTM-7.0 software, the weighted prediction (WP) is not enabled with TPM. In the last JVET meeting (Geneva, Oct-2019), it was agreed that combining WP and TPM was desirable feature. In this last meeting, it has been proposed to either cascade WP and TPM or to re-scale the blending weights with WP parameters prior the TPM blending stage. However, these methods have a latency issue since they add an additional stage and/or need an additional multiplier for the blending.

It is proposed to replace the combination of WP and TPM with either regular bi-prediction or regular weighted bi-prediction. It states that no additional process nor weighting stage is added to the inter-prediction pipeline compared to regular TPM or regular bi-prediction.

It is reported that the proposed method provides BD-rate gains of -0.12%(Y),-0.11%(U),-0.24%(V) / 0.09%(Y),-0.28%(U),0.14%(V) in RA / LDB configurations with fade-to-black sequences, and -0.28%(Y),-0.21%(U),-0.20%(V) / -0.17%(Y),-0.27%(U),-0.12%(V) in RA / LDB configurations with fade-to-white sequences.

# Introduction / Problem Statement

In the current VTM-7.0 software, the weighted prediction (WP) is not enabled with TPM. In the last JVET meeting (Geneva, Oct-2019), it was agreed that combining WP and TPM was a desirable feature. In this last meeting, it has been proposed to either cascade WP and TPM [1] or to re-scale the blending weights with WP parameters prior the TPM blending stage [2].

However, these methods have a latency issue since they add an additional stage and/or need an additional multiplier for the blending. Additionally, some participants commented that we should not add this extra complexity for a benefit that is only found on special content such as fading content.

# Proposal

In VVC, when the TPM mode is used, a CU is split evenly into two triangle-shaped partitions (A and B), using either the diagonal split (triangleDir=0) or the anti-diagonal split (triangleDir=1). The prediction samples are derived from uni-prediction motion compensation process, with MV (mvA, mvB) and reference indexes (refIdxA, refIdxB) derived from the triangle merge candidate list and candidate indexes.

In case the weighted prediction is enabled for at least one of the reference refIdxA, or refIdxB (WP enabled at slice level and at least one WP weights associated with refIdxA or refIdxB is not default), it is proposed to replace the two triangle-shaped partitions with regular bi-prediction (triangleDir=0) or regular weighted bi-prediction using WP weights (triangleDir=1). In case the weighted prediction is disabled for both the reference refIdxA, and refIdxB (WP disabled at slice level or WP weights associated with refIdxA and refIdxB are defaults) then the regular two triangle-shaped partitions applies (Figure 1).

Then, no additional weighting stage is added to the inter-prediction pipeline.

Figure 1 – The regular two triangle-shaped partitions with blending (left) or bi-prediction process (right) is selected depending on whether WP is disabled for both reference partitions or not.

# Experimental results

Since weighted prediction is not used on CTC, the experiments by using weighted prediction and fade-sequences have been conducted in this contribution.

The fade sequences are generated with tool provided in [3]. In our experiments, a linear fade is applied to the about first 2 seconds of the regular sequence set, and the experiments are conducted with them. A linear fade-out (sequence to black or white) is applied in the 1st half and a linear fade-in (respectively black or white to sequence) in the 2nd half. The fade strength is limited to the range [0.25, 0.75] instead of [0, 1] to avoid extreme PSNR numbers at the frame level that cause abnormal rate-distortion graphs.

The anchors are VTM-7.0 with WP enabled (incl. ibc bug-fix). The encoder method-0 is used for estimating WP parameters.

Table 1: summary of the proposed TPM mode combined with WP (fade-to-black and fade-to-white sequences).

|  | Proposal difference with VTM-7.0 (Fade to black) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.29% | -0.45% | -0.44% | 87% | 82% |  |  |  |  |  |
| Class A2 | 0.04% | 0.08% | 0.03% | 79% | 78% |  |  |  |  |  |
| Class B | -0.22% | 0.12% | -0.21% | 89% | 91% | 0.17% | -0.33% | -0.53% | 86% | 86% |
| Class C | -0.42% | -0.30% | -0.32% | 94% | 92% | 0.03% | 0.31% | 0.69% | 76% | 73% |
| Class E |  |  |  |  |  | 0.03% | -0.97% | 0.52% | 91% | 94% |
| Overall | -0.12% | -0.11% | -0.24% | 88% | 87% | 0.09% | -0.28% | 0.14% | 84% | 83% |
| Class D | -0.30% | -0.08% | 0.47% | 98% | 105% | -0.06% | 0.25% | -0.70% | 82% | 85% |
| Class F | -3.71% | -1.67% | -2.00% | 81% | 88% | -0.32% | -0.69% | -0.62% | 87% | 89% |
| TGM | -0.32% | -0.56% | -0.56% | 98% | 99% | -0.24% | -0.38% | -0.15% | 100% | 100% |

|  | Proposal difference with VTM-7.0 (Fade to white) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Random Access Main 10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.30% | -0.46% | 77% | 75% |  |  |  |  |  |
| Class A2 | -0.07% | 0.06% | 0.11% | 74% | 73% |  |  |  |  |  |
| Class B | -0.14% | -0.03% | -0.03% | 86% | 92% | -0.22% | -0.10% | 0.54% | 77% | 78% |
| Class C | -0.45% | -0.56% | -0.44% | 96% | 94% | -0.11% | -0.06% | -0.08% | 75% | 71% |
| Class E |  |  |  |  |  | -0.15% | -0.82% | -1.27% | 92% | 93% |
| Overall | -0.28% | -0.21% | -0.20% | 84% | 85% | -0.17% | -0.27% | -0.12% | 80% | 79% |
| Class D | -0.21% | 0.10% | -0.12% | 95% | 98% | -0.03% | 0.42% | 0.28% | 77% | 75% |
| Class F | -3.99% | -2.02% | -2.42% | 82% | 85% | -0.03% | -0.46% | -0.24% | 87% | 90% |
| TGM | -0.43% | -0.69% | -0.55% | 99% | 97% | -0.22% | -0.13% | -0.31% | 100% | 99% |

# Syntax modifications

The modification of the syntax is highlighted by yellow:

This process is invoked when decoding a coding unit with MergeTriangleFlag[ xCb ][ yCb ] equal to 1.

Inputs to this process are:

- a luma location ( xCb, yCb ) specifying the top-left sample of the current coding block relative to the top-left luma sample of the current picture,

- a variable cbWidth specifying the width of the current coding block in luma samples,

- a variable cbHeight specifying the height of the current coding block in luma samples,

- the luma motion vectors in 1/16 fractional-sample accuracy mvA and mvB,

- the chroma motion vectors mvCA and mvCB,

- the reference indices refIdxA and refIdxB,

- the prediction list flags predListFlagA and predListFlagB,

- the picture order count pocA and pocB

- the variable weightedPredFlag.

Outputs of this process are:

- an (cbWidth)x(cbHeight) array predSamplesL of luma prediction samples,

- an (cbWidth / SubWidthC)x(cbHeight / SubHeightC) array predSamplesCb of chroma prediction samples for the component Cb,

- an (cbWidth / SubWidthC)x(cbHeight / SubHeightC) array predSamplesCr of chroma prediction samples for the component Cr.

Let predSamplesLAL and predSamplesLBL be (cbWidth)x(cbHeight) arrays of predicted luma sample values and, predSamplesLACb, predSamplesLBCb, predSamplesLACr and predSamplesLBCr be (cbWidth / SubWidthC)x(cbHeight / SubHeightC) arrays of predicted chroma sample values.The predSamplesL, predSamplesCb and predSamplesCr are derived by the following ordered steps:

- For N being each of A and B, the following applies:

- The reference picture consisting of an ordered two-dimensional array refPicLNL of luma samples and two ordered two-dimensional arrays refPicLNCb and refPicLNCr of chroma samples is derived by invoking the process specified in clause 8.5.6.2 with X set equal to predListFlagN and refIdxX set equal to refIdxN as input.

- The array predSamplesLNL is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the luma coding block width sbWidth set equal to cbWidth, the luma coding block height sbHeight set equal to cbHeight, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvN and the reference array refPicLXL set equal to refPicLNL, the variable bdofFlag set euqal to FALSE, the variable cIdx is set equal to 0, and RefPicScale[ predListFlagN ][ refIdxN ] as inputs. The variable wpEnabledTriangle[0] is set equal to 1 if weightedPredFlag is equal to 1 and (predListFlagN is equal to 0 and luma_weight_l0_flag[ refIdxN ] is equal to 1) or (predListFlagN is equal to 1 and luma_weight_l1_flag[ refIdxN ] is equal to 1), otherwise wpEnabledTriangle[0] is set equal to 0.

- The array predSamplesLNCb is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding block width sbWidth set equal to cbWidth / SubWidthC, the coding block height sbHeight set equal to cbHeight / SubHeightC, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvCN, and the reference array refPicLXCb set equal to refPicLNCb, the variable bdofFlag set euqal to FALSE, the variable cIdx is set equal to 1, and RefPicScale[ predListFlagN ][ refIdxN ] as inputs. The variable wpEnabledTriangle[1] is set equal to 1 if weightedPredFlag is equal to 1 and (predListFlagN is equal to 0 and chroma_weight_l0_flag[ refIdxN ] is equal to 1) or (predListFlagN is equal to 1 and chroma_weight_l1_flag[ refIdxN ] is equal to 1), otherwise wpEnabledTriangle[1] is set equal to 0.

- The array predSamplesLNCr is derived by invoking the fractional sample interpolation process specified in clause 8.5.6.3 with the luma location ( xCb, yCb ), the coding block width sbWidth set equal to cbWidth / SubWidthC, the coding block height sbHeight set equal to cbHeight / SubHeightC, the motion vector offset mvOffset set equal to ( 0, 0 ), the motion vector mvLX set equal to mvCN, and the reference array refPicLXCr set equal to refPicLNCr, the variable bdofFlag set euqal to FALSE, the variable cIdx is set equal to 2, and RefPicScale[ predListFlagN ][ refIdxN ] as inputs. The variable wpEnabledTriangle[2] is set equal to 1 if weightedPredFlag is equal to 1 and (predListFlagN is equal to 0 and chroma_weight_l0_flag[ refIdxN ] is equal to 1) or (predListFlagN is equal to 1 and chroma_weight_l1_flag[ refIdxN ] is equal to 1), otherwise wpEnabledTriangle[2] is set equal to 0.

- The partition direction of merge triangle mode variable triangleDir is set equal to merge_triangle_split_dir[ xCb ][ yCb ].

- The variable wpTriangleMode[cIdx] is derived as follows:

  - If wpEnabledTriangle[cIdx] is equal to 1 and triangleDir is equal to 0, then wpTriangleMode[cIdx] is set equal to 0,

  - Otherwise if wpEnabledTriangle[cIdx] is equal to 1 and pocA is equal to pocB, then wpTriangleMode[cIdx] is set equal to 2,

  - Otherwise if wpEnabledTriangle[cIdx] is equal to 0, then wpTriangleMode[cIdx] is set equal to 2,

  - Otherwise (wpEnabledTriangle[cIdx] is equal to 1 and triangleDir is equal to 1) wpTriangleMode[cIdx] is set equal to 1.

- The prediction samples inside the current luma coding block, predSamplesL[ xL ][ yL ] with xL = 0..cbWidth − 1 and yL = 0..cbHeight − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 if wpTriangleMode[0] is equal to 2, specified in clause 8.5.6.6.3 if wpTriangleMode[0] is equal to 1, and specified in clause 8.5.6.6.2 with bcwIdw equal to 0 if wpTriangleMode[0] is equal to 0, with the coding block width nCbW set equal to cbWidth, the coding block height nCbH set equal to cbHeight, the sample arrays predSamplesLAL and predSamplesLBL, and the variables triangleDir, and cIdx equal to 0 as inputs.

- The prediction samples inside the current chroma component Cb coding block, predSamplesCb[ xC ][ yC ] with xC = 0..cbWidth / SubWidthC − 1 and yC = 0..cbHeight / SubHeightC − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 if wpTriangleMode[1] is equal to 2, specified in clause 8.5.6.6.3 if wpTriangleMode[1] is equal to 1, specified in clause 8.5.6.6.2 with bcwIdw equal to 0 if wpTriangleMode[1] is equal to 0, with the coding block width nCbW set equal to cbWidth / SubWidthC, the coding block height nCbH set equal to cbHeight / SubHeightC, the sample arrays predSamplesLACb and predSamplesLBCb, and the variables triangleDir, and cIdx equal to 1 as inputs.

- The prediction samples inside the current chroma component Cr coding block, predSamplesCr[ xC ][ yC ] with xC = 0..cbWidth / SubWidthC − 1 and yC = 0..cbHeight / SubHeightC − 1, are derived by invoking the weighted sample prediction process for triangle merge mode specified in clause 8.5.7.2 if wpTriangleMode[2] is equal to 2, specified in clause 8.5.6.6.3 if wpTriangleMode[2] is equal to 1, specified in clause 8.5.6.6.2 with bcwIdw equal to 0 if wpTriangleMode[2] is equal to 0, with the coding block width nCbW set equal to cbWidth / SubWidthC, the coding block height nCbH set equal to cbHeight / SubHeightC, the sample arrays predSamplesLACr and predSamplesLBCr, and the variables triangleDir, and cIdx equal to 2 as inputs.

- The motion vector storing process for merge triangle mode specified in clause 8.5.7.3 is invoked with the luma coding block location ( xCb, yCb ), the luma coding block width cbWidth, the luma coding block height cbHeight, the partition direction triangleDir, the luma motion vectors mvA and mvB, the reference indices refIdxA and refIdxB, and the prediction list flags predListFlagA and predListFlagB as inputs.

# Subjective tests

One provides in Figure 2 two examples extracted from the same frame in anchor and proposal (RA, FadeWhite, D_RaceHorses, poc=25, qp=37). They illustrate the coding artefacts that may be caused by using TPM with reference samples which are not compensated with WP weights (VTM-7.0 anchor, left) and how this is corrected with proposal (right).

Figure 2 – On the left, TPM mode applied without WP weights (VTM-7.0, anchor) causes luminance artefacts, while on the right the proposed method has no luminance artefact.

# Conclusion

It is proposed to enable the weighted prediction with TPM. It is stated that the proposed method does not add any supplemental process nor weighting stage to the inter-prediction pipeline compared to the regular TPM or the regular bi-prediction modes.

# References

- T.Chujoh, E.Sasaki, T.Ikai, “Non-CE4: On a simplification for triangle merge mode,” 16th Meeting: Geneva, CH, 1–11 October 2019.

- A.Filippov, V.Rufitskiy, “Non-CE4: On TPM merge mode in the presence of weighted prediction,” 16th Meeting: Geneva, CH, 1–11 October 2019.

- P. Bordes, E. François and F. L. Léannec, AHG13 - Weighted Prediction vs Generalized Bi-prediction with Fade sequences, Joint Video Exploration Team Document, JVET-L0201, Macao, Oct. 2018.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).