| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0350_r1 |
| --- | --- |

| Title: | Non-CE: Enable CIIP for 4x8/8x4 block without SCIPU violation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Seung-hwan Kim Jaehyun Lim 19, Ynagjae-daero 11-gil, Seocho-gu, Seoul, 06772, Korea | Tel: Email: | +82-10-8505-0793 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In this contribution, it is proposed to allow CIIP for 4x8/8x4 blocks, and the corresponding chroma block, which is smaller than SCIPU, uses only inter prediction block without chroma intra prediction in order to keep SCIPU restriction. The experimental results show 0.02%, 0.03% BD-rate saving in CTC RA and LDB configuration, respectively. The BD-rate saving in the low resolution is relatively high. 0.09% and 0.10% for classD in RA and LDB.

In the revision1, the LDB experimental result is updated.

Introduction

In the current VVC draft version 7, the intra prediction for chroma component is bypassed for 2xN chroma block in order to consider SCIPU restriction and CIIP is allowed only if all of the following conditions are true.

- sps_ciip_enabled_flag is equal to 1.

- general_merge_flag[ x0 ][ y0 ] is equal to 1.

- merge_subblock_flag[ x0 ][ y0 ] is equal to 0.

- regular_merge_flag[ x0 ][ y0 ] is equal to 0.

- cu_skip_flag[ x0 ][ y0 ] is equal to 0.

- cbWidth is less than 128.

- cbHeight is less than 128.

- cbWidth * cbHeight is greater than or equal to 64.

By the above condition, CIIP is disallowed for 4x8/8x4 block. BD-rate saving can be achieved as shown in Table 1 by allowing CIIP for those blocks while the corresponding chroma block becomes smaller than SCIPU. Because VVC draft version 7 partioanlly covered SCIPU restriction for CIIP.

Proposed

In the proposed method, it is suggested to consider SCIPU restriction for the chroma intra prediction during CIIP decoding process and to allow CIIP for 4x8/8x4 coding block.

Experimental result

The table 1 shows 4x8/8x4 CIIP enabling without SCIPU restriction only for CIIP decoding process under CTC configuration. BD-rate saving becomes 0.03% at the overall sequences and in it shows -0.11% at the ClassD especially without Encoding complexity increment.

Table 1 Test results on 4x8/8x4 enabling without SCIPU restriction

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 CTC |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.13% | -0.01% | 100% | 99% |
| Class A2 | -0.03% | 0.03% | -0.04% | 100% | 99% |
| Class B | -0.03% | 0.00% | 0.01% | 101% | 100% |
| Class C | -0.04% | 0.02% | 0.00% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.02% | -0.01% | 100% | 100% |
| Class D | -0.11% | 0.00% | -0.11% | 100% | 101% |
| Class F | -0.04% | -0.03% | -0.06% | 100% | 100% |
| Class SCC | -0.02% | -0.03% | 0.00% | 100% | 100% |

Table 2 shows the experimental results of the proposed method.

Table 2 Test results on the proposed method

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.05% | -0.03% | 100% | 99% |
| Class A2 | -0.01% | 0.01% | 0.02% | 100% | 100% |
| Class B | -0.02% | 0.10% | 0.15% | 100% | 98% |
| Class C | -0.06% | -0.08% | -0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.01% | 0.04% | 100% | 99% |
| Class D | -0.09% | -0.02% | -0.23% | 100% | 100% |
| Class F | -0.03% | 0.03% | -0.01% | 100% | 100% |
| Class SCC | -0.04% | -0.02% | -0.03% | 100% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.01% | -0.06% | -0.13% | 101% | 100% |
| Class C | -0.02% | -0.27% | -0.16% | 101% | 99% |
| Class E | -0.09% | 0.52% | 0.14% | 100% | 99% |
| Overall | -0.03% | 0.01% | -0.07% | 100% | 100% |
| Class D | -0.10% | -0.04% | -0.24% | 100% | 99% |
| Class F | 0.02% | -0.25% | 0.32% | 100% | 98% |
| Class SCC | -0.04% | -0.09% | -0.12% | 101% | 98% |

Conclusion

It is suggested to adopt the proposed method in the next VTM.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).