| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0351_r1 |
| --- | --- |

| Title: | Non-CE: Report on experimental result for fixing BDPCM Chroma mismatch |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hyeongmun Jang Junghak Nam Naeri Park Seung-hwan Kim Jaehyun Lim 19, Ynagjae-daero 11-gil, Seocho-gu, Seoul, 06772, Korea | Tel: Email: | +82-10-8505-0793 hm.jang@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

This contribution reports the mismatch issues on BDPCM chroma and also inform the test result after aligning SW with the specification. The abovementioned mismatch issues on BDPCM chroma is occurred according to syntax signaling position and context. The test result shows 0.25%, 0.07 and 0.11% BD-rate saving for 4:4:4 contents in AI, RA and LDB respectively after aligning SW with VVC draft.

In the revision1, the LDB experimental result tables for aspect 2 and aspect 3 are updated.

Introduction

The BDPCM chorma is adopted at the last JVET meeting. In order to enable BDPCM for chroma component, BDPCM flag and direction are transmitted. In the specification, the BDPCM flag and direction uses different context between luma component and chroma component. Additionally, the syntaxes related to BDPCM chroma is signaled between luma intra prediction mode and chroma intra prediction mode at the single tree structure. But the context for BDPCM flag and direction is shared for luma and chroma component in the current VTM implementation. And also the BDPCM chroma syntaxes are transmitted before intra prediction mode for luma component.

Experimental result

The experimental shows test result on fixing SW to align with Spec. Different contexts are used for luma and choma component. The syntax for BDPCM chroma also transmitted before chroma intra prediction mode to align SW with specification.

Table 1. Align SW with SPEC regarding “Separate BDPCM Ctx for each Channel”

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| 444 color format | -0.02% | -0.07% | -0.05% | 100% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| 444 color format | 0.02% | 0.05% | 0.04% | 99% | 98% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| 444 color format | -0.01% | -0.04% | 0.05% | 100% | 100% |

Table 2. Align SW with SPEC regarding “Signaling BDPCM Chroma syntax before Chroma intra prediction mode”

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| 444 color format | -0.18% | -0.21% | -0.24% | 101% | 98% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| 444 color format | -0.05% | -0.01% | -0.04% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| 444 color format | -0.12% | 0.05% | 0.11% | 100% | 100% |

Table 3. Align SW with SPEC regarding “table 1 + table 2”

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| 444 color format | -0.25% | -0.21% | -0.25% | 101% | 99% |
|  | Random access Main10 |  |  |  |  |
|  | Over VTM-7.0 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| 444 color format | -0.07% | 0.01% | -0.05% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-7.0 4:4:4 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| 444 color format | -0.11% | -0.04% | 0.05% | 100% | 99% |

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

“