| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0356 |
| --- | --- |

| Title: | AHG16: Parallel Merge Estimation for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Han Gao, Biao Wang, Anand Meher Kotra, Elena Alshina (Huawei) Yu-Ling Hsiao, Chun-Chia Chen, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei (MediaTek) | Email: | semih.esenlik@huawei.com {yuling.hsiao, chunchia.chen, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | Huawei Technologies Co.,Ltd., MediaTek Inc. |  |  |

_____________________________

# Abstract

Merge estimation region (MER), which is part of HEVC, divides a picture into equally sized square regions and allows estimation of the costs of merging candidates of multiple CUs inside the same region in parallel. Parallel merge estimation is an encoder tool, wherein the MER size can be adjusted or completely switched off by the encoder. This contribution proposes to introduce MER in VVC. Two alternative methods have been proposed. The first method introduces a normative spatial candidate availability check similar to HEVC. In the second method an additional HMVP LUT update rule is introduced to enable usage of HMVP in parallel merge estimation.

# Introduction

Merge estimation region (MER) in HEVC lets the encoder estimate the costs of merging candidates of multiple coding blocks within square region in parallel. MER lets a spatial merging candidate to be added into merging candidate list only when the current CB and the neighboring CB are in the different MER. In this contribution it is proposed to make the feature available in VVC by introducing as little normative changes as possible.

# Proposed method

Two alternative methods are prosed which are presented in the subsections below.

## Alternative method 1

### Spatial Merge candidate availability check (normative)

In method one, the same MER condition of HEVC is introduced in VVC, which can be described as follows:

“A neighbor block is considered unavailable for prediction in merge list construction if the two blocks are inside the same square MER”.

As an example, assume that the coordinates of the bottom-left merge candidate is (xNBL, yNBL) and coordinates of the current block is (xCb, yCb). The bottom-left merge candidate is set unavailable if both of the following conditions are satisfied:

- xNBL>>log2MERsize == xCb>>log2MERsize

- yNBL>>log2MERsize == yCb>>log2MERsize

As it can be inferred from the conditions above, the MER region is always square in shape and its size is controlled by the parameter log2MERsize which is signalled in the SPS.

The VTM software already includes the suggested spatial merge candidate availability check (which is carried over from HM software). Therefore the contribution only modifies the VVC specification and no modification is necessary in the VTM software.

The proposed spatial merge candidate availability check is applied to regular merge candidate list and Sub-block merge candidate lists, and not the IBC merge candidate list. In other words it affects 4 merge modes including inter regular merge mode, merge with motion vector difference (MMVD) mode, combined inter/intra prediction (CIIP) mode, and triangular partitioning mode (TPM).

### BT and TT split constraints in encoder (non-normative)

The following split constraints are applied in a non-normative fashion to prevent the splits that hinder parallel merge estimation.

- When the current slice is inter slice and when current CB width is larger than MER size or current CB height is larger than MER size

  - When CB height smaller than or equal to MER size, disallow horizontal BT split

  - When CB width smaller than or equal to MER size, disallow vertical BT split

  - When CB height smaller than or equal to 2 times MER size, disallow horizontal TT split

  - When CB width smaller than or equal to 2 times MER size, disallow vertical TT split

The contribution JVET-L0216 exemplifies possible block splits that would hinder parallel merge estimation in the encoder. The above split constraints guarantee that none of the examples that are presented in JVET-L0216 can happen.

In other words according to the split constraints above, a coding block or any of its parent blocks are guaranteed to;

- either encapsulate one or more complete MERs,

- or it is encapsulated completely by a MER.

### HMVP candidate usage restriction (non-normative)

History-based merging candidates (HMVP) and merging candidates after HMVP are not used when current CU width is smaller than MER size or current CU height is smaller than MER size. The constraint is used to break the dependency caused by updating HMVP table between different CUs in one MER.

## Alternative method 2

Alternative method 2 is similar to method 1, except for the HMVP restriction, which is implemented as a normative modification. The method 2 can be summarized as follows:

### Spatial Merge candidate availability check (normative)

Identical to alternative method 1.

### BT and TT split constraints in encoder (non-normative)

Identical to alternative method 1.

### HMVP LUT update rule (normative)

An additional condition check is introduced to disable updating of HMVP table if the following condition is satisfied:

“Do not update HMVP LUT if bottom-right coordinate of the CB does not coincide with the bottom-right coordinate of a MER.”

The above condition can be implemented using the following single IF condition check:

Update HMVP table if(((xCb + width) & (1<<log2MERSize – 1) == 0) && ((yCb + height) & (1<<log2MERSize – 1) == 0)), where “&” operand is bitwise AND operation.

The condition above guarantees that HMVP LUT is updated only at MER boundaries, and only when the bottom and right boundaries of a CB coincides with MER boundaries. Therefore there are no dependencies between 2 CBs inside the same MER that would hinder parallelism.

# Experiment results

## Experimental results for alternative method 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.47% | 0.39% | 0.65% | 86% | 100% |
| Class A2 | 1.06% | 1.68% | 1.75% | 84% | 100% |
| Class B | 1.07% | 1.54% | 1.54% | 83% | 100% |
| Class C | 2.38% | 3.00% | 3.18% | 78% | 100% |
| Class E |  |  |  |  |  |
| Overall | 1.29% | 1.73% | 1.84% | 82% | 100% |
| Class D | 2.54% | 3.41% | 3.43% | 77% | 100% |
| Class F (optional) | 1.92% | 2.52% | 2.62% | 86% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.19% | 1.74% | 2.11% | 82% | 96% |
| Class C | 2.61% | 3.43% | 3.61% | 77% | 99% |
| Class E | 1.23% | 2.20% | 1.07% | 92% | 100% |
| Overall | 1.67% | 2.42% | 2.35% | 83% | 98% |
| Class D | 3.16% | 4.85% | 4.68% | 74% | 100% |
| Class F (optional) | 3.91% | 4.65% | 6.48% | 82% | 99% |

Table 1 Results of the proposed method with MER equals to 8x8

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.77% | 2.06% | 2.74% | 65% | 99% |
| Class A2 | 3.59% | 4.93% | 4.82% | 65% | 100% |
| Class B | 2.95% | 3.56% | 3.62% | 64% | 100% |
| Class C | 3.86% | 4.23% | 4.50% | 61% | 101% |
| Class E |  |  |  |  |  |
| Overall | 3.08% | 3.71% | 3.92% | 64% | 100% |
| Class D | 3.28% | 3.38% | 3.69% | 64% | 100% |
| Class F (optional) | 2.93% | 3.39% | 3.58% | 75% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 3.34% | 4.64% | 4.60% | 63% | 97% |
| Class C | 4.52% | 5.53% | 5.48% | 59% | 101% |
| Class E | 3.70% | 4.59% | 3.41% | 77% | 101% |
| Overall | 3.82% | 4.92% | 4.60% | 65% | 99% |
| Class D | 4.12% | 4.85% | 5.26% | 62% | 101% |
| Class F (optional) | 5.64% | 6.68% | 7.71% | 70% | 101% |

Table 2 Results of the proposed method with MER equals to 16x16

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.51% | 2.74% | 3.39% | 62% | 100% |
| Class A2 | 3.81% | 4.40% | 4.40% | 69% | 99% |
| Class B | 3.18% | 3.12% | 3.14% | 70% | 99% |
| Class C | 3.42% | 3.24% | 3.54% | 75% | 100% |
| Class E |  |  |  |  |  |
| Overall | 3.23% | 3.33% | 3.55% | 69% | 100% |
| Class D | 2.99% | 2.84% | 3.10% | 82% | 100% |
| Class F (optional) | 2.29% | 2.41% | 2.61% | 81% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 3.67% | 4.28% | 4.46% | 66% | 98% |
| Class C | 3.92% | 4.31% | 4.55% | 75% | 100% |
| Class E | 3.93% | 4.57% | 3.49% | 73% | 102% |
| Overall | 3.82% | 4.36% | 4.25% | 70% | 100% |
| Class D | 3.72% | 4.29% | 4.39% | 82% | 101% |
| Class F (optional) | 4.00% | 4.63% | 5.36% | 76% | 101% |

Table 3 Results of the proposed method with MER equals to 32x32

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.87% | 1.51% | 2.13% | 101% | 99% |
| Class A2 | 3.10% | 3.50% | 3.68% | 105% | 98% |
| Class B | 3.30% | 2.73% | 2.76% | 105% | 97% |
| Class C | 4.08% | 4.11% | 4.47% | 104% | 100% |
| Class E |  |  |  |  |  |
| Overall | 3.18% | 3.01% | 3.27% | 104% | 98% |
| Class D | 3.83% | 3.83% | 4.16% | 102% | 100% |
| Class F (optional) | 2.07% | 2.28% | 2.33% | 104% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 3.67% | 3.09% | 3.27% | 102% | 96% |
| Class C | 4.89% | 5.06% | 5.51% | 101% | 98% |
| Class E | 2.66% | 3.00% | 2.11% | 100% | 98% |
| Overall | 3.82% | 3.72% | 3.73% | 101% | 97% |
| Class D | 4.84% | 5.16% | 7.05% | 100% | 98% |
| Class F (optional) | 3.21% | 3.84% | 4.29% | 101% | 99% |

Table 4 Results of the proposed method with MER equals to 64x64

## Experimental results for alternative method 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.43% | 0.42% | 0.71% | 87% | 100% |
| Class A2 | 1.06% | 1.64% | 1.68% | 84% | 100% |
| Class B | 1.05% | 1.52% | 1.49% | 83% | 101% |
| Class C | 2.31% | 3.02% | 3.19% | 78% | 101% |
| Class E |  |  |  |  |  |
| Overall | 1.27% | 1.72% | 1.82% | 83% | 100% |
| Class D | 2.43% | 3.26% | 3.23% | 77% | 101% |
| Class F (optional) | 1.87% | 2.46% | 2.54% | 87% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 1.14% | 1.80% | 1.91% | 82% | 97% |
| Class C | 2.49% | 3.23% | 3.54% | 77% | 100% |
| Class E | 1.25% | 1.58% | 1.16% | 92% | 100% |
| Overall | 1.62% | 2.22% | 2.27% | 83% | 99% |
| Class D | 3.07% | 4.33% | 4.40% | 74% | 101% |
| Class F (optional) | 3.79% | 4.67% | 5.67% | 83% | 101% |

Table 5 Results of the proposed method with MER equals to 8x8

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.66% | 2.07% | 2.48% | 66% | 100% |
| Class A2 | 3.21% | 4.23% | 4.13% | 66% | 100% |
| Class B | 2.49% | 3.15% | 3.20% | 65% | 101% |
| Class C | 3.20% | 3.62% | 3.90% | 62% | 102% |
| Class E |  |  |  |  |  |
| Overall | 2.66% | 3.27% | 3.43% | 65% | 101% |
| Class D | 2.73% | 2.71% | 2.74% | 66% | 102% |
| Class F (optional) | 2.55% | 2.93% | 3.10% | 76% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.90% | 4.28% | 4.15% | 63% | 99% |
| Class C | 3.77% | 4.67% | 4.93% | 60% | 102% |
| Class E | 3.50% | 4.44% | 3.34% | 78% | 100% |
| Overall | 3.34% | 4.45% | 4.21% | 65% | 100% |
| Class D | 3.41% | 4.68% | 4.31% | 63% | 103% |
| Class F (optional) | 4.83% | 6.28% | 6.79% | 71% | 102% |

Table 6 Results of the proposed method with MER equals to 16x16

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 2.04% | 2.23% | 2.75% | 64% | 100% |
| Class A2 | 3.03% | 3.35% | 3.39% | 71% | 100% |
| Class B | 2.24% | 2.52% | 2.55% | 72% | 101% |
| Class C | 2.37% | 2.20% | 2.46% | 79% | 101% |
| Class E |  |  |  |  |  |
| Overall | 2.39% | 2.54% | 2.73% | 72% | 101% |
| Class D | 2.23% | 2.09% | 2.29% | 87% | 101% |
| Class F (optional) | 1.70% | 1.77% | 1.85% | 83% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.65% | 3.67% | 3.73% | 67% | 98% |
| Class C | 2.83% | 2.93% | 3.31% | 77% | 99% |
| Class E | 3.28% | 4.07% | 3.19% | 74% | 101% |
| Overall | 2.87% | 3.53% | 3.46% | 72% | 99% |
| Class D | 2.75% | 3.21% | 3.56% | 86% | 101% |
| Class F (optional) | 3.16% | 3.24% | 4.38% | 78% | 101% |

Table 7 Results of the proposed method with MER equals to 32x32

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 1.22% | 1.04% | 1.33% | 106% | 100% |
| Class A2 | 2.24% | 2.49% | 2.70% | 108% | 98% |
| Class B | 2.26% | 2.10% | 2.18% | 108% | 99% |
| Class C | 3.12% | 3.16% | 3.33% | 110% | 100% |
| Class E |  |  |  |  |  |
| Overall | 2.28% | 2.25% | 2.42% | 108% | 99% |
| Class D | 3.24% | 3.04% | 3.39% | 110% | 101% |
| Class F (optional) | 1.50% | 1.66% | 1.73% | 107% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM4.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 2.40% | 1.94% | 1.95% | 104% | 97% |
| Class C | 3.68% | 3.40% | 3.82% | 107% | 98% |
| Class E | 1.85% | 1.99% | 0.86% | 103% | 98% |
| Overall | 2.69% | 2.44% | 2.30% | 105% | 98% |
| Class D | 4.04% | 4.08% | 4.83% | 107% | 100% |
| Class F (optional) | 2.34% | 2.49% | 3.08% | 105% | 99% |

Table 8 Results of the proposed method with MER equals to 64x64

# Tabular Summary of experimental results

Below table summarizes the experimental results for first and the second methods.

|  | MER size | BDR-Y | BDR-U | BDR-Y | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| Method 1 | 8x8 | 1.29% | 1.73% | 1.84% | 82% | 100% |
|  | 16x16 | 3.08% | 3.71% | 3.92% | 64% | 100% |
|  | 32x32 | 3.23% | 3.33% | 3.55% | 69% | 100% |
|  | 64x64 | 3.18% | 3.01% | 3.27% | 104% | 98% |
| Method 2 | 8x8 | 1.27% | 1.72% | 1.82% | 83% | 100% |
|  | 16x16 | 2.66% | 3.27% | 3.43% | 65% | 101% |
|  | 32x32 | 2.39% | 2.54% | 2.73% | 72% | 101% |
|  | 64x64 | 2.28% | 2.25% | 2.42% | 108% | 99% |

Table 9. Summary of the results for RA configuration

|  | MER size | BDR-Y | BDR-U | BDR-Y | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| Method 1 | 8x8 | 1.67% | 2.42% | 2.35% | 83% | 98% |
|  | 16x16 | 3.82% | 4.92% | 4.60% | 65% | 99% |
|  | 32x32 | 3.82% | 4.36% | 4.25% | 70% | 100% |
|  | 64x64 | 3.82% | 3.72% | 3.73% | 101% | 97% |
| Method 2 | 8x8 | 1.62% | 2.22% | 2.27% | 83% | 99% |
|  | 16x16 | 3.34% | 4.45% | 4.21% | 65% | 100% |
|  | 32x32 | 2.87% | 3.53% | 3.46% | 72% | 99% |
|  | 64x64 | 2.69% | 2.44% | 2.30% | 105% | 98% |

Table 10. Summary of the results for LDB configuration

# Conclusion

In this contribution, it is proposed to add MER to VVC to support encoding merge modes in parallel. With MER, the tight cycle budget at encoder side could be eased to make hardware friendlier. Two methods have been proposed, it is suggested to include one of the methods in VVC specification draft.

# References

[1] Y.-K. Wang, B. Bross, and V. Drugeon, “AHG2: Editorial input on VVC draft text,” Document of Joint Video Experts Team, JVET-Q0041.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-N1010.

# Patent rights declaration(s)

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).