| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0358 |
| --- | --- |

| Title: | AHG9: Constraints on ALF APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma Haitao Yang | Tel: Email: | maxiang6@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

There are three types of adaptation parameter set (APS): ALF_APS, LMCS_APS, and SCALING_APS. For each type of APS, there is an ID in picture header (PH) to specify the APS ID of the APS that the slices associated with the PH refers to. In the current VVC draft specification, for APS NAL units with aps_params_type equal to LMCS_APS and SCALING_APS, there are constraints on the TemporalId of the APS NAL unit and the TemporalId of the picture associated with PH, while for APS NAL units with aps_params_type equal to ALF_APS, there is no such kind of constraint. It is proposed to add the TemporalId constraint between the ALF_APS NAL unit and the the picture associated with PH.

Introduction

There are three types of adaptation parameter set (APS): ALF_APS, LMCS_APS, and SCALING_APS. For each type of APS, there is an ID in picture header (PH) to specify the APS ID of the APS that the slices associated with the PH refers to. In the current VVC draft specification, for APS NAL units with aps_params_type equal to LMCS_APS and SCALING_APS, there are constraints on the TemporalId of the APS NAL unit and the TemporalId of the picture associated with PH, while for APS NAL units with aps_params_type equal to ALF_APS, there is no such kind of constraint.

Picture header

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| … |  |
| if( sps_lmcs_enabled_flag ) { |  |
| pic_lmcs_enabled_flag | u(1) |
| if( pic_lmcs_enabled_flag ) { |  |
| pic_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| pic_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_scaling_list_enabled_flag ) { |  |
| pic_scaling_list_present_flag | u(1) |
| if( pic_scaling_list_present_flag ) |  |
| pic_scaling_list_aps_id | u(3) |
| } |  |
| … |  |
| } |  |

pic_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slices associated with the PH refers to.

The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to pic_alf_aps_id_luma[ i ] shall be equal to 1.

…

pic_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slices associated with the PH refers to.

The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to pic_alf_aps_id_chroma shall be equal to 1.

…

pic_lmcs_aps_id specifies the adaptation_parameter_set_id of the LMCS APS that the slices associated with the PH refers to. The TemporalId of the APS NAL unit having aps_params_type equal to LMCS_APS and adaptation_parameter_set_id equal to pic_lmcs_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

…

pic_scaling_list_aps_id specifies the adaptation_parameter_set_id of the scaling list APS. The TemporalId of the APS NAL unit having aps_params_type equal to SCALING_APS and adaptation_parameter_set_id equal to pic_scaling_list_aps_id shall be less than or equal to the TemporalId of the picture associated with PH.

…

Slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| … |  |
| } |  |

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_luma[ i ] is not present, the value of slice_alf_aps_id_luma[ i ] is inferred to be equal to the value of pic_alf_aps_id_luma[ i ].

The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] shall be equal to 1.

…

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slice refers to. The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be less than or equal to the TemporalId of the coded slice NAL unit. When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_chroma is not present, the value of slice_alf_aps_id_chroma is inferred to be equal to the value of pic_alf_aps_id_chroma.

The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to slice_alf_aps_id_chroma shall be equal to 1.

…

Constraint analysis

From the semantics of slice_alf_aps_id_luma[ i ] and slice_alf_aps_id_chroma, it can be seen that when slice_alf_enabled_flag is equal to 1, but this two syntaxes are not presented, they will infer to the relative value from PH. So, it is necessary to add the TemporalId constraint between the ALF_APS NAL units and the picture associated with PH.

On the other hand, even this two syntaxes are presented, which means they will not infer to the relative value from PH, the NAL units with ID contained in the PH also need to conform to the constraint. Because, it does not make sense to contain an ID of APS NAL unit of which cannot be inferred by the slice in the picture associated with PH.

Proposed text

It is proposed to add the TemporalId constraint between the ALF_APS NAL unit and the the picture associated with PH:

pic_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slices associated with the PH refers to.

The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to pic_alf_aps_id_luma[ i ] shall be equal to 1.

The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to pic_alf_aps_id_luma[ i ] shall be less than or equal to the TemporalId of the picture associated with PH.

pic_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slices associated with the PH refers to.

The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to pic_alf_aps_id_chroma shall be equal to 1.

The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and adaptation_parameter_set_id equal to pic_alf_aps_id_chroma shall be less than or equal to the TemporalId of the picture associated with PH.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).