| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0359 |
| --- | --- |

| Title: | AHG12: Constraints on tile signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiang Ma Haitao Yang | Tel: Email: | maxiang6@huawei.com |
| Source: | Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

In the current VVC draft specification, there is no constraints on the value of tile_column_width_minus1[ i ] and tile_row_height_minus1[ i ], which are used to signal the width and height of the i-th tile column in units of CTBs, respectively. Meanwhile, the constraint on the tile_idx_delta[ i ], which used to signal the difference in tile index between the i-th rectangular slice and the ( i + 1 )-th rectangular slice, is not complete. It is proposed to add some constrains on value of these syntaxes to make the signaled values of these syntaxes valid.

Introduction

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| for( j = 0; j < num_slices_in_tile_minus1[ i ]; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |
| } |  |
| … |  |

…

num_exp_tile_columns_minus1 plus 1 specifies the number of explicitly provided tile column widths. The value of num_exp_tile_columns_minus1 shall be in the range of 0 to PicWidthInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_exp_tile_columns_minus1 is inferred to be equal to 0.

num_exp_tile_rows_minus1 plus 1 specifies the number of explicitly provided tile row heights. The value of num_exp_tile_rows_minus1 shall be in the range of 0 to PicHeightInCtbsY − 1, inclusive. When no_pic_partition_flag is equal to 1, the value of num_tile_rows_minus1 is inferred to be equal to 0.

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs for i in the range of 0 to num_exp_tile_columns_minus1 − 1, inclusive. tile_column_width_minus1[ num_exp_tile_columns_minus1 ] is used to derive the width of the tile columns with index greater than or equal to num_exp_tile_columns_minus1 as specified in clause 6.5.1. When not present, the value of tile_column_width_minus1[ 0 ] is inferred to be equal to PicWidthInCtbsY − 1.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs for i in the range of 0 to num_exp_tile_rows_minus1 − 1, inclusive. tile_row_height_minus1[ num_exp_tile_rows_minus1 ] is used to derive the height of the tile rows with index greater than or equal to num_exp_tile_rows_minus1 as specified in clause 6.5.1. When not present, the value of tile_row_height_minus1[ 0 ] is inferred to be equal to PicHeightInCtbsY − 1.

…

tile_idx_delta[ i ] specifies the difference in tile index between the i-th rectangular slice and the ( i + 1 )-th rectangular slice. The value of tile_idx_delta[ i ] shall be in the range of –NumTilesInPic + 1 to NumTilesInPic − 1, inclusive. When not present, the value of tile_idx_delta[ i ] is inferred to be equal to 0. In all other cases, the value of tile_idx_delta[ i ] shall not be equal to 0.

…

Propose 1: on tile_column_width_minus1[ i ], tile_row_height_minus1[ i ]

Since the Descriptor of these two syntaxes is ‘ue(v)’, so, it can be derived that the values of these two syntaxes shall not less than 0, which means the width or the height of each tile shall larger than or equal to 1 CTB. But also, it is nessary to constrains the width or the height of each tile shall not larger than the width of the picture and the height of the picture in unit of CTB, respectively.

Meanwhile, it is nessary to constrains that the sum of the all tile width and the sum of all tile height should less than or equal to the width or height of the picture in unit of CTB, respectively. Because, a tile outside of the picture does not make sense.

Propose 2: on tile_idx_delta[ i ]

The logic to add constraint to tile_idx_delta[ i ] is similar with above two syntaxes. To limit that all the rect slice is included in the picture, it is nessary to constrains that the sum of the tile_idx_delta[ i ] should less than of the number of the tiles in the picture.

Proposed text

Propose 1: on tile_column_width_minus1[ i ], tile_row_height_minus1[ i ]

tile_column_width_minus1[ i ] plus 1 specifies the width of the i-th tile column in units of CTBs for i in the range of 0 to num_exp_tile_columns_minus1 − 1, inclusive. tile_column_width_minus1[ num_exp_tile_columns_minus1 ] is used to derive the width of the tile columns with index greater than or equal to num_exp_tile_columns_minus1 as specified in clause 6.5.1. When not present, the value of tile_column_width_minus1[ 0 ] is inferred to be equal to PicWidthInCtbsY − 1. For i in the range of 0 to num_exp_tile_columns_minus1 − 1 , the sum of tile_column_width_minus1[ i ] shall less than PicWidthInCtbsY.

tile_row_height_minus1[ i ] plus 1 specifies the height of the i-th tile row in units of CTBs for i in the range of 0 to num_exp_tile_rows_minus1 − 1, inclusive. tile_row_height_minus1[ num_exp_tile_rows_minus1 ] is used to derive the height of the tile rows with index greater than or equal to num_exp_tile_rows_minus1 as specified in clause 6.5.1. When not present, the value of tile_row_height_minus1[ 0 ] is inferred to be equal to PicHeightInCtbsY − 1. For i in the range of 0 to num_exp_tile_rows_minus1 − 1 , the sum of tile_row_height_minus1 [ i ] shall less than PicHeightInCtbsY.

Propose 2: on tile_idx_delta[ i ]

tile_idx_delta[ i ] specifies the difference in tile index between the i-th rectangular slice and the ( i + 1 )-th rectangular slice. The value of tile_idx_delta[ i ] shall be in the range of –NumTilesInPic + 1 to NumTilesInPic − 1, inclusive. When not present, the value of tile_idx_delta[ i ] is inferred to be equal to 0. In all other cases, the value of tile_idx_delta[ i ] shall not be equal to 0. For i in the range of 0 to num_slices_in_pic_minus1-1 , the sum of tile_idx_delta[ i ] shall less than NumTilesInPic.

Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).