| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0360 |
| --- | --- |

| Title: | AHG9: Slice level control of coding tools |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Anand Meher Kotra, Han Gao, Biao Wang, Alexey Filippov, Vasily Rufitskiy, Elena Alshina | Email: | semih.esenlik@huawei.com |
| Source: | Huawei Technologies Co., Ltd |  |  |

_____________________________

# Abstract

During the 16th JVET meeting which was held in Geneva, picture header (PH) was decided to be included in the draft VVC specification. Accordingly most of the syntax elements that were present in the SH have been moved to PH. In this contribution it is proposed to move the enabling control flags of 3 coding tools (namely BDOF, DMVR, PROF) from PH back to SH. It is asserted that proposed modifications provide finer encoder control and improved support for mixed content and local lossless coding application scenarios.

# Proposal

According to the proposal it is proposed to provide control to enable or disable the following 3 coding tools in the slice header:

- DMVR

- BDOF

- PROF

In VVC draft 7 it is not possible to control DMVR, BDOF and PROF at the CU level explicitly, since those tools are enabled automatically when a coding block is coded with merge mode or affine mode. Including CU level control for these tools have been tested in previous meetings in the context of CE9, and no action was taken due to the coding loss associated with such change. According to the contribution these 3 tools are controlled at the second higher level (per slice). As a result, an encoder can decide to switch off those tools for parts of the frame, by initiating a new slice or using sub-picture partitioning.

Furthermore it is asserted that the listed coding tools assume specific motion models (translational motion and sample level affine), which are common in natural content but those assumptions might not hold well in screen content. In the case of mixed content, it is common practice to devide the picture into multiple slices and encapsulate different content types in different slices (e.g. due to different source generating the content). Therefore it is desirable to be able to control the listed tools independently for each slice in the case of mixed content applications.

Finally in the very high bit-rate regime DMVR and BDOF are asserted to provide very small compression efficiency, since the importance of precise inter prediction loses importance and transform and residual coding starts taking up more encoding time. In the case of local (near-) lossless coding scenarios, it would be beneficial to switch off those tools to direct the encoding resources into other coding tools. The contribution JVET-P0521 reports the performance results of BDOF and DMVR in lossless coding scenario.

The next sub-section presents the proposed syntax and semantic changes.

## Proposed Specification Changes

Yellow text indicate flags that are moved from PH to SH.

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( sps_bdof_slice_present_flag && slice_type = = B) |  |
| slice_disable_bdof_flag | u(1) |
| if( sps_dmvr_slice_present_flag && slice_type = = B ) |  |
| slice_disable_dmvr_flag | u(1) |
| if( sps_prof_slice_present_flag && slice_type ! = I) |  |
| slice_disable_prof_flag | u(1) |
| slice_qp_delta | se(v) |
| … |  |

7.3.2.6	Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |

7.4.3.4	Picture parameter set RBSP semantics

…

pic_disable_bdof_flag equal to 1 specifies that bi-directional optical flow inter prediction based inter bi-prediction is disabled in the slices associated with the PH. pic_disable_bdof_flag equal to 0 specifies that bi-directional optical flow inter prediction based inter bi-prediction may or may not be enabled in the slices associated with the PH.

When pic_disable_bdof_flag is not present, the following applies:

- If sps_bdof_enabled_flag is equal to 1, the value of pic_disable_bdof_flag is inferred to be equal to 0.

- Otherwise (sps_bdof_enabled_flag is equal to 0), the value of pic_disable_bdof_flag is inferred to be equal to 1.

pic_disable_dmvr_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is disabled in the slices associated with the PH. pic_disable_dmvr_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction may or may not be enabled in the slices associated with the PH. When not present, the value of pic_disable_dmvr_flag is inferred to be 1.

When pic_disable_dmvr_flag is not present, the following applies:

- If sps_dmvr_enabled_flag is equal to 1, the value of pic_disable_dmvr_flag is inferred to be equal to 0.

- Otherwise (sps_dmvr_enabled_flag is equal to 0), the value of pic_disable_dmvr_flag is inferred to be equal to 1.

pic_disable_prof_flag equal to 1 specifies that prediction refinement with optical flow is disabled in the slices associated with the PH. pic_disable_prof_flag equal to 0 specifies that prediction refinement with optical flow may or may not be enabled in the slices associated with the PH. When not present, the value of pic_disable_prof_flag is inferred to be 1.

When pic_disable_prof_flag is not present, the following applies:

- If sps_affine_prof_enabled_flag is equal to 1, the value of pic_disable_prof_flag is inferred to be equal to 0.

- Otherwise (sps_affine_prof_enabled_flag is equal to 0), the value of pic_disable_prof_flag is inferred to be equal to 1.

7.4.8	Slice header semantics

7.4.8.1	General slice header semantics

slice_disable_bdof_flag equal to 1 specifies that bi-directional optical flow inter prediction based inter bi-prediction is disabled in the slice associated with the SH. slice_disable_bdof_flag equal to 0 specifies that bi-directional optical flow inter prediction based inter bi-prediction may or may not be enabled in the slice associated with the SH.

When slice_disable_bdof_flag is not present, the following applies:

- If sps_bdof_enabled_flag is equal to 1, the value of slice_disable_bdof_flag is inferred to be equal to 0.

- Otherwise (sps_bdof_enabled_flag is equal to 0), the value of slice_disable_bdof_flag is inferred to be equal to 1.

slice_disable_dmvr_flag equal to 1 specifies that decoder motion vector refinement based inter bi-prediction is disabled in the slice associated with the SH. pic_disable_dmvr_flag equal to 0 specifies that decoder motion vector refinement based inter bi-prediction may or may not be enabled in the slice associated with the SH. When not present, the value of slice_disable_dmvr_flag is inferred to be 1.

When slice_disable_dmvr_flag is not present, the following applies:

- If sps_dmvr_enabled_flag is equal to 1, the value of slice_disable_dmvr_flag is inferred to be equal to 0.

- Otherwise (sps_dmvr_enabled_flag is equal to 0), the value of slice_disable_dmvr_flag is inferred to be equal to 1.

slice_disable_prof_flag equal to 1 specifies that prediction refinement with optical flow is disabled in the slice associated with the SH. pic_disable_prof_flag equal to 0 specifies that prediction refinement with optical flow may or may not be enabled in the slice associated with the SH. When not present, the value of slice_disable_prof_flag is inferred to be 1.

When slice_disable_prof_flag is not present, the following applies:

- If sps_affine_prof_enabled_flag is equal to 1, the value of slice_disable_prof_flag is inferred to be equal to 0.

- Otherwise (sps_affine_prof_enabled_flag is equal to 0), the value of slice_disable_prof_flag is inferred to be equal to 1.

# Conclusion

It is proposed to move the 3 disabling control flags of DMVR, BDOF and PROF from PH to SH, in order to facilitate control at the slice level. It is asserted that proposed modifications provide finer encoder control and improved support for mixed content and local lossless coding application scenarios.

# References

[1] T. Ma, Y. Chen, X. Xiu, H. Jhu, X. Wang, “AHG18: Disabling DMVR and BDOF for lossless coding”, JVET- P0521, the 16th JVET meeting, October 2019, Geneva, CH

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).