| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0362 |
| --- | --- |

| Title: | Non-normative aspects for cleanup of DMVR specification and software |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Semih Esenlik, Biao Wang, Anand Meher Kotra, Han Gao, Elena Alshina (Huawei) Jeeva Raj Arumugam, Sagar Kotecha, Shailesh Ramamurthy (Ittiam) | Email: | semih.esenlik@huawei.com Jeeva.Raj@ittiam.com |
| Source: | Huawei Technologies Co., Ltd, Ittiam Systems Private Limited |  |  |

_____________________________

# Abstract

Two non-normative improvements for the VVC draft 7 and one bugfix for VTM7.0 software have been proposed corresponding to the description and implementation of DMVR. Furthermore DMVR external memory access measurement results have been reported as information in response to the AhG request from previous meeting, which recommended fixing of a bug in the memory bandwidth measurement tool.

# Proposal

In this contribution 2 modifications to the VVC specification are proposed to align spec text with software and to cleanup the text.

Furthermore one software bug-fix is proposed to fix an issue in 4:0:0 color format coding. It is reported that the VTM software does not work properly when the input video has 4:0:0 format due to software bugs allegedly at multiple places in the code. In this contribution a bug related to the DMVR module is reported and suggested to be fixed.

## Spec cleanup proposal 1

In section “8.3.5.1 Derivation process for delta motion vector component offset” of the VVC draft 7 a pseudo code is provided as part of parametric motion vector derivation process of DMVR. The provided pseudo code is one way of the implementation of integer division with truncation operation that is already defined in VVC specification text.

The pseudo code that is provided is just one of the alternative implementations of the integer division operation. It is suggested to replace this section with intiger division “/” operand to make it clear that the provided pseudo code is not mistaken as mandatory process. The proponents believe that this would make the specification more clear and cleanup the text.

Below is the proposed modifications, where green is used to highlight additions, yellow is used to highlight removals. The suggested changes are purely editorial.

#### 8.5.3.5 Parametric motion vector refinement process Inputs to this process are: - a 3x3 2-D array sadArray[ dX + 1 ][ dY + 1 ] with dX = −1..1 and dY = −1..1, - a delta luma motion vector dMvL0. Output of this process is the modified delta luma motion vector dMvL0. The variable dMvX is derived by invoking the derivation process for delta motion vector component offset specified in clause 8.5.3.5.1 with the SAD values sadMinus, sadCenter and sadPlus set equal to sadArray[ 0 ][ 1 ], sadArray[ 1 ][ 1 ], and sadArray[ 2 ][ 1 ] as inputs, and dMvX set equal to the output dMVc. The variable dMvY is derived by invoking the derivation process for delta motion vector component offset specified in clause 8.5.3.5.1 with the SAD values sadMinus, sadCenter and sadPlus set equal to sadArray[ 1 ][ 0 ], sadArray[ 1 ][ 1 ], and sadArray[ 1 ][ 2 ] as inputs, and dMvY set equal to the output dMVc. The delta luma motion vector dMvL0 is modified as follows: dMvL0[ 0 ] += dMvX (654) dMvL0[ 1 ] += dMvY (655) NOTE – dMvC with C being X or Y is constrained to be between −8 and 8 since sadMinus, sadCenter, and sadPlus are all positive, and sadCenter is the smallest value among the three. This allows the division to be performed with up to 4 quotient bits and can be implemented using compares, shifts, and subtractions. #### 8.5.3.5.1 Derivation process for delta motion vector component offset Inputs to this process are 3 SAD values sadMinus, sadCenter, and sadPlus. Output of this process is the delta motion vector component correction offset dMvC. The offset dMVc is derived using the following pseudo codeas follows: denom = ( ( sadMinus + sadPlus ) − ( sadCenter << 1 ) ) << 3 if( denom = = 0 ) dMvC = 0 else { num = ( sadMinus − sadPlus ) << 3if( sadMinus = = sadCenter ) dMvC = num / denomdMvC = −8 else if( sadPlus = = sadCenter ) dMvC = 8 else { num = ( sadMinus − sadPlus ) << 4 signNum = 0 if( num < 0 ) { num = −num signNum = 1 } quotient = 0 (656) counter = 3 while( counter > 0 ) { counter = counter − 1 quotient = quotient << 1 if( num >= denom ) { num = num − denom quotient = quotient + 1 } denom = ( denom >> 1 ) } if( signNum = = 1 ) dMvC = −quotient else dMvC = quotient } } NOTE – dMvC is constrained to be between −8 and 8 since sadMinus, sadCenter, and sadPlus are all positive, and sadCenter is the smallest value among the three. This allows the division to be performed with up to 4 quotient bits and can be implemented using compares, shifts, and subtractions. NOTE 1– The above process is equivalent to an integer division of num = ( sadMinus − sadPlus ) << 3 by denom = ( sadMinus + sadPlus − ( sadCenter << 1 ). Given the fact that sadMinus, sadCenter, and sadPlus are all positive, and sadCenter is the smallest value among the three, the value is limited to be in the range of −8 to 8, inclusive.

## Spec cleanup proposal 2

In DMVR, 25 search points are compared with each other to determine the integer refinement offset. In software implementations the center point can be checked first to trigger early termination, which is how DMVR is implemented in VTM software. Otherwise all points are checked in raster scan order.

In the current VVC specification, it appears to be that the SAD cost must be computed twice for the center point, first one for the early termination decision, second one in section 8.5.3.4, though the second one is unnecessary.

It is proposed to align the VVC specification with software, such that the center point is checked only once as the first point. It is believed that this would reduce chance of inefficient implemention of DMVR. The suggested change is purely editorial, it is presented below.

#### 8.5.3.4 Array entry selection process Inputs to this process are: - a 2-D array of sum of absolute differences values sadArray[ dX + 2 ][ d Y + 2 ] with dX = −2..2 and dY = −2..2, - an integer sample offset ( intOffX, intOffY ), - a variable minSad. Outputs of this process are: - the modified integer sample ( intOffX, intOffY ), - a variable dmvrSad. The following steps are applied to modify the integer sample offset ( intOffX, intOffY ): for( dY = −2; dY <= 2; dY++ ) { for( dX = −2; dX <= 2; dX++ ) { if( !(dX ==0 && dY==0) && sadArray[ dX + 2 ][ dY + 2 ] < minSad ) { minSad = sadArray[ dX + 2 ][ dY + 2 ] (653) intOffX = dX intOffY = dY } } } The variable dmvrSad is set equal to minSad.

## Software bug-fix for 4:0:0 color format coding

An encoder crash is observed in the current VTM software while encoding with YUV 4:0:0. The reason behind the crash is that the various coding tools assumes Y,U and V components are always present irrespective of color format. This contribution provides the fix for DMVR by using “getNumberValidComponents(chromaFormat)” in place of  “MAX_NUM_COMPONENT”(3) to support 4:0:0 coding. Fix is required in the components looping code of  reference integer pixel copy, padding and motion compensation. Patch for this bug-fix on top of VTM7.0 is provided with this contribution. The suggested fix is only for the VTM software and the current specification draft is aligned with this fix.

# Memory access measurement test results for DMVR

In the 16th JVET meeting in Geneva it was reported that the memory bandwidth measurement tool is broken for DMVR. Between the two meeting cycles the issue has been resolved by merge request 1131 by the authors of this contribution. The measurement results with the bug-fix is reported below for information purposes.

# Conclusion

Modifications targeting cleanup of VVC draft specification and VTM software have been proposed. It is suggested to adopt the suggested modifications to improve clarity and readability of specification and software.

# Patent rights declaration(s)

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Ittiam Systems Pvt. Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).