| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0366 |
| --- | --- |

| Title: | Simplification of interpolation filtering for intra prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gagan Rath, Franck Galpin, Fabrice Urban 975 Avenue des champs blancs 35576 Cesson-Sévigné, FRANCE | Tel: Email: | +33.2.56.85.62.76 gagan.rath@interdigital.com, franck.galpin@interdigital.com fabrice.urban@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This document presents a simplification of the interpolation filtering for luma intra prediction. It is proposed to replace the 4-tap smoothing filter in VTM 7.0, that replaced the Gaussian filter in VTM 6.0, by linear interpolation with a prior filtering of reference samples by [1 2 1]/4 filter. It is reported that the proposed simplification yields 0.02% coding loss in AI over VTM-7.0, with 100% encoding and 96% decoding time, and that it yields 0.02% coding loss in RA over VTM-7.0, with 100% encoding and 100% decoding time.

# Introduction

In VTM 7.0 intra prediction, the predictor samples in angular modes with non-integer slopes are interpolated with either a 4-tap cubic filter or a 4-tap smoothing filter. The 4-tap smoothing filter replaced the 4-tap Gaussian filter in VTM 6.0. The filter to be used is decided based on the value of an interpolationFlag, whose derivation depends on refFilterFlag, another flag which decides if the reference samples need to be filtered with the [1 2 1]/4 filter.

As [1] has mentioned, the new 4-tap smoothing filters can be derived by convolving a linear filter with the [1 2 1]/4 filter. Therefore, instead of interpolating with the 4-tap smoothing filters, similar predictors can be obtained with linear interpolation after pre-filtering the reference filters by [1 2 1]/4 filter.

# Problem definition

In VTM 7.0, a flag called refFilterFlag is set as 1 if the reference samples for intra prediction are to be filtered with the [1 2 1]/4 filter. Thus, it is set as 1 when all the following conditions hold true:

(i) Luma CU, (ii) MRL index is 0, (iii) no CU split in ISP (iv) no MIP (v) H * W > 32, (vi) prediction mode is either PLANAR or a mode with integer slope (i.e., predIntraAngle is equal to 32 or a multiple thereof).

As the condition (vi) indicates, for all directional modes with non-integer slopes, the reference samples are never filtered.  Since these modes have non-integer slopes, the predictor samples for all target samples need to be interpolated. The interpolation is performed using either a 4-tap cubic filter or a 4-tap smoothing filter. The choice between the two filters is indicated with another flag called interpolationFlag, which is set as 1 if conditions (i)-(v) hold, and (vi) the prediction mode satisfies a preset angularity threshold condition based on the CU size. If the interpolationFlag is 1, a 4-tap smoothing filter is used for interpolation, else a 4-tap cubic filter is used.

# Proposal

In this proposal, the refFilterFlag is set as 1 when all the following conditions hold true:

(i) Luma CU, (ii) MRL index is 0, (iii) no CU split in ISP (iv) no MIP (v) H * W > 32, (vi) prediction mode is either PLANAR, or a prediction mode that satisfies the preset angularity threshold depending on the CU size, irrespective of having integer slope or not.

If the refFilterFlag is set as 1, the reference samples are filtered with the [1 2 1]/4 filter, and the interpolation is performed using linear interpolation in the case of modes with non-integer slopes. Otherwise, the reference samples are not filtered, and the interpolation of predictors is performed using the 4-tap cubic filter. For the case of angular modes with integer slopes, the filtering of reference samples and the prediction are kept unchanged.

The above proposal requires the following Specification text changes in section 8.4.5.2.5 of [2]:

The variable refFilterFlag is derived as follows:

- If predModeIntra is equal to one of the following values: 0, −14, −12, −10, −6, 2, 34, 66, 72, 76, 78, 80, then refFilterFlag is set equal to 1.

- Otherwise, refFilterFlag is set equal to 0.

- refFilterFlag is set equal to 1 if all of the following conditions are true:

- refIdx is equal to 0

- nTbW * nTbH is greater than 32

- cIdx is equal to 0

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT

  - predModeIntra is either equal to 0,  or is greater than 1 where the following applies:

    - The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

    - The variable intraHorVerDistThres[ nTbS ] is specified in Table 25.

    - minDistVerHor is greater than intraHorVerDistThres[ nTbS ]

- Otherwise, refFilterFlag is set equal to 0.

Specification text changes in section 8.4.5.2.9 of [2]:

The variable filterFlag is derived as follows:

- If all of the following conditions are true, filterFlag is set equal to 1:

- refIdx is equal to 0

- nTbW * nTbH is greater than 32

- cIdx is equal to 0

- IntraSubPartitionsSplitType is equal to ISP_NO_SPLIT

- refFilterFlag is equal to 1

- Otherwise, filterFlag is set equal to 0.

For the derivation of the reference samples p[ x ][ y ] the following applies:

- If filterFlag refFilterFlag is equal to 1, the filtered sample values p[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 are derived as follows:

Specification text changes in section 8.4.5.2.12 of [2]:

The variable filterFlag is derived as follows:

- If one or more of the following conditions is true, filterFlag is set equal to 0:

- refFilterFlag is equal to 1

- refIdx is not equal to 0

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- Otherwise, the following applies:

- The variable minDistVerHor is set equal to Min( Abs( predModeIntra − 50 ), Abs( predModeIntra − 18 ) ).

- The variable intraHorVerDistThres[ nTbS ] is specified in Table 25.

- The variable filterFlag is derived as follows:

- If minDistVerHor is greater than intraHorVerDistThres[ nTbS ], filterFlag is set equal to 1.

- Otherwise, filterFlag is set equal to 0.

- If cIdx is equal to 0, and refFilterFlag is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = fC[ iFact ][ j ]	   	(332)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1( ( ( i=03fT[ i ]*ref[ x+iIdx+i ] ) + 32 )  >>  6 )	(333)

- Otherwise (cIdx is not equal to 0, or refFilterFlag is equal to 1), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ x + iIdx + 1 ] + iFact * ref[ x + iIdx + 2 ] + 16 )  >>  5	(334)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ x + iIdx + 1 ]		(335)

- If cIdx is equal to 0, and refFilterFlag is equal to 0, the following applies:

- The interpolation filter coefficients fT[ j ] with j = 0..3 are derived as follows:

fT[ j ] = fC[ iFact ][ j ]		(342)

- The value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = Clip1( ( ( i=03fT[ i ]*ref[ y+iIdx+i ] ) + 32 )  >>  6 )	(343)

- Otherwise (cIdx is not equal to 0, or refFilterFlag is equal to 1), depending on the value of iFact, the following applies:

- If iFact is not equal to 0, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = 
	( ( 32 − iFact ) * ref[ y + iIdx + 1 ] + iFact * ref[ y + iIdx + 2 ] + 16 )  >>  5	(344)

- Otherwise, the value of the prediction samples predSamples[ x ][ y ] is derived as follows:

predSamples[ x ][ y ] = ref[ y + iIdx + 1 ]		(345)

# Experimental Results

Simulations were performed according to the CTC described in [3]. AI and RA results are reported. Note that timings results are not fully reliable.

Table 1. Results of proposed method over VTM 7.0, AI configuration

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.06% | 0.23% | 100% | 99% |
| Class A2 | 0.02% | 0.00% | 0.00% | 101% | 98% |
| Class B | 0.02% | 0.08% | -0.01% | 100% | 97% |
| Class C | -0.01% | 0.10% | 0.03% | 100% | 93% |
| Class E | 0.01% | -0.06% | 0.18% | 100% | 95% |
| Overall | 0.02% | 0.03% | 0.07% | 100% | 96% |
| Class D | 0.00% | -0.03% | 0.01% | 99% | 94% |
| Class F | 0.01% | 0.03% | 0.07% | 100% | 93% |

Table 2. Results of proposed method over VTM 7.0, RA configuration

|  |  |  | Random access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | -0.05% | -0.01% | 99% | 99% |
| Class A2 | -0.03% | -0.02% | 0.16% | 100% | 98% |
| Class B | 0.03% | 0.12% | 0.24% | 99% | 101% |
| Class C | 0.03% | -0.11% | -0.02% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.00% | 0.10% | 100% | 100% |
| Class D | 0.02% | -0.05% | -0.18% | 100% | 99% |
| Class F | 0.03% | 0.06% | 0.08% | 99% | 105% |

# Conclusion

In this document, a simplification of the interpolation filtering for intra prediction is proposed. The proposed method replaces the case of 4-tap smoothing filter by pre-filtering of reference samples by [1 2 1]/4 filter and then linear interpolation at the interpolation stage. The proposed design eliminates the usage of interpolationFlag as used in the VTM 7.0 and uses only the value of refFilterFlag to decide on both the reference sample filtering and  the interpolation method used. Experimental results show the proposed simplification yields about the same coding performance in both AI and RA configurations as VTM 7.0. It is proposed to adopt the method into the future versions of VTM.

# References

- A. Filippov and V. Rufitskiy, “Cleanup of interpolation filtering for intra prediction”, JVET-P0599-v2, Geneva, CH, Oct. 2019.

- B. Bross et al,  “Versatile video coding (Draft 7) ”, JVET-P2001-vE, Geneva, CH, Oct. 2019.

- F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, Geneva, CH, March 2019.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).