| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels,BE, 7–17January 2020 | Document: JVET-Q0369-v1 |
| --- | --- |

| Title: | Restricting MIP for ACT coded CUs |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Wenting Cai, Jianqing Zhu, Kimihiko Kazui | Tel: Email: | +86-21-63350606 caiwenting@cn.fujitsu.com zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Source: | Fujitsu |  |  |

_____________________________

- Abstract

This document proposes two modifications to improve the performance of ACT:

Aspect 1: fix a bug by skipping BDPCM signaling for Chroma when ACT is used in the software to align the VTM-7.1 with VVC Draft 7

Aspect 2: disallow MIP when ACT is used for an intra CU to enhance the throughput.

Based on the same test sequences used for HEVC SCC, simulation results for the single-tree partition on top of VTM-7.1 reportedly show that Aspect 1 provides average {G, B, R} BD-rate changes {-0.03%, -0.02%, -0.03%}, {-0.01%, -0.01%, 0.00%} and average {Y, U, V} BD-rate changes {0.00%, 0.00%, 0.01%}, {0.00%, 0.00%, 0.00%} in AI and RA configurations, respectively; Aspect 2 combined with Aspect 1 provides average {G, B, R} BD-rate changes {-0.04%, -0.02%, 0.00%}, {0.02%, 0.00 %, 0.01 %} and average {Y, U, V} BD-rate changes {0.02%, -0.01%, 0.00%}, {0.01%, 0.03%, 0.02%} in AI and RA configurations.

Introduction

On Aspect 1

BDPCM for Chroma has been adopted into the VVC (Draft 7[1]) along with ACT in the last meeting, which are both only enabled for sequences in 4:4:4 format. In the current WD7 [1], BDPCM for Chroma is not signalled when an intra CU is decided to use ACT as follows in Table 1:

Table 1 Syntax related to ACT

7.3.9.5 Coding unit syntax ……. if( !cu_act_enabled_flag ) {. if( cbWidth <= MaxTsSize && cbHeight <= MaxTsSize && sps_bdpcm_chroma_enabled_flag ) {. intra_bdpcm_chroma_flag; ae(v). if( intra_bdpcm_chroma_flag ). intra_bdpcm_chroma_dir_flag; ae(v). } ....... }. …….

However, a mismatch occurs in VTM-7.1[2] that BDPCM for Chroma will always be signalled no matter if ACT is utilized for an intra CU.

It is proposed to fix this bug in the software by skipping BDPCM signaling for Chroma when ACT is used in order to align the VTM-7.1 SW with VVC Draft 7.

On Aspect 2

In VTM7.1, for an intra CU in the single tree, when the luma intra prediction mode is not PLT and BDPCM, MIP will be signaled before other regular modes. Table 2 is the statistics of MIP hit rate with MIP flag(0 or 1) signaled which is parsed from the bit stream with the encoding condition that IBC, PLT, BDPCM for both luma and chroma, inter HashME and ACT are enabled on top of VTM-7.1 in AI configuration. Data shows that,

- For RGB sequences, when cu_act_enabled_flag is 1, the ratio of CU with MIP flag signaled (RoMF) is much more than 50%, but the hit rate of MIP among them is less than 3%.

- For YUV sequences, when cu_act_enabled_flag is 1, although the hits of MIP is higher (3%~25%) , the ratio of CU with MIP flag signaled is very small (<1%).

- When cu_act_enabled_flag is 0, MIP is applied by a large number of intra CUs for YUV sequences.

Table 2 the hit rate of MIP when ACT is enabled

|  | ACT = 1 |  |  | ACT = 0 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | MIP = 1 | MIP = 0 | RoMF | MIP = 1 | MIP = 0 | RoMF |
| RGB, text & graphics with motion, 1080p & 720p | 0.59% | 99.41% | 51.34% | 0.66% | 99.34% | 48.66% |
| RGB, mixed content, 1440p & 1080p | 0.43% | 99.57% | 84.35% | 0.39% | 99.61% | 15.65% |
| RGB, Animation, 720p | 1.51% | 98.49% | 89.95% | 1.73% | 98.27% | 10.05% |
| RGB, camera captured, 1080p | 2.65% | 97.35% | 90.83% | 4.53% | 95.47% | 9.17% |
| YUV, text & graphics with motion, 1080p & 720p | 3.72% | 96.28% | 0.07% | 4.30% | 95.70% | 99.93% |
| YUV, mixed content, 1440p & 1080p | 7.95% | 92.05% | 0.05% | 10.52% | 89.48% | 99.95% |
| YUV, Animation, 720p | 14.03% | 85.97% | 0.02% | 7.41% | 92.59% | 99.98% |
| YUV, camera captured, 1080p | 24.38% | 75.63% | 0.01% | 22.75% | 77.25% | 99.99% |

Where RoMF means the ratio of CU with MIP flag signaled.

Proposed method

It is proposed to disallow MIP when ACT is used. The syntax change is as the highlighted below.

7.3.9.5	Coding unit syntax

| …… |  |
| --- | --- |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_act_enabled_flag && treeType = = SINGLE_TREE ) |  |
| cu_act_enabled_flag | ae(v) |
| if( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA ¦ ¦ CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_PLT ) { |  |
| if( treeType = = SINGLE_TREE ¦ ¦ treeType = = DUAL_TREE_LUMA ) { |  |
| if( pred_mode_plt_flag ) |  |
| palette_coding( x0, y0, cbWidth, cbHeight, treeType ) |  |
| else { |  |
| if( sps_bdpcm_enabled_flag && cbWidth <= MaxTsSize && cbHeight <= MaxTsSize ) |  |
| intra_bdpcm_luma_flag | ae(v) |
| if( intra_bdpcm_luma_flag ) |  |
| intra_bdpcm_luma_dir_flag | ae(v) |
| else { |  |
| if( sps_mip_enabled_flag && !cu_act_enable_flag ) |  |
| intra_mip_flag[ x0 ][ y0 ] | ae(v) |
| if( intra_mip_flag[ x0 ][ y0 ] ) { |  |
| intra_mip_transposed[ x0 ][ y0 ] | ae(v) |
| intra_mip_mode[ x0 ][ y0 ] | ae(v) |
| }…… |  |
| …… |  |

Experiment results

The proposed method has been implemented on top of VTM-7.1, and tested under AI and RA of the tested following the test conditions used by the core experiment (CE)8[3] in the Gothenburg meeting. In the simulation, IBC, PLT, BDPCM for both luma and chroma, inter HashME and ACT are enabled for both anchor (VTM-7.1) and tests. Results with separate-tree disabled are provided in Table 3 and Table 4.

Performance of Aspect 1

Table 3 Coding performance of the Aspect 1

|  | All Intra |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | -0.04% | -0.03% | -0.02% |
| RGB, mixed content, 1440p & 1080p | -0.06% | -0.04% | -0.03% |
| RGB, Animation, 720p | -0.01% | 0.02% | -0.04% |
| RGB, camera captured, 1080p | -0.02% | -0.02% | -0.01% |
| YUV, text & graphics with motion, 1080p & 720p | 0.00% | -0.01% | 0.00% |
| YUV, mixed content, 1440p & 1080p | 0.00% | 0.02% | 0.02% |
| YUV, Animation, 720p | 0.00% | 0.00% | 0.00% |
| YUV, camera captured, 1080p | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

|  | Randon Access |  |  |
| --- | --- | --- | --- |
|  | G/Y | B/U | R/V |
| RGB, text & graphics with motion, 1080p & 720p | 0.00% | -0.01% | -0.02% |
| RGB, mixed content, 1440p & 1080p | -0.02% | -0.02% | -0.03% |
| RGB, Animation, 720p | -0.04% | 0.00% | -0.02% |
| RGB, camera captured, 1080p | 0.04% | 0.00% | 0.06% |
| YUV, text & graphics with motion, 1080p & 720p | 0.00% | 0.01% | 0.00% |
| YUV, mixed content, 1440p & 1080p | 0.02% | 0.02% | 0.02% |
| YUV, Animation, 720p | 0.00% | 0.00% | -0.02% |
| YUV, camera captured, 1080p | 0.00% | 0.00% | 0.00% |
| Enc Time[%] | 100% |  |  |
| Dec Time[%] | 100% |  |  |

Performance of combining Aspect 1 and Aspect 2

Table 4 Coding performance of combining Aspect 1 and Aspect 2

|  |  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | G/Y |  | B/U |  | R/V |  |
| RGB, text & graphics with motion, 1080p & 720p |  | -0.06% |  | -0.02% |  | 0.03% |  |
| RGB, mixed content, 1440p & 1080p |  | -0.04% |  | -0.04% |  | -0.01% |  |
| RGB, Animation, 720p |  | -0.08% |  | 0.05% |  | -0.01% |  |
| RGB, camera captured, 1080p |  | 0.02% |  | -0.06% |  | -0.03% |  |
| YUV, text & graphics with motion, 1080p & 720p |  | 0.00% |  | 0.00% |  | 0.00% |  |
| YUV, mixed content, 1440p & 1080p |  | 0.00% |  | 0.06% |  | 0.03% |  |
| YUV, Animation, 720p |  | -0.01% |  | -0.02% |  | 0.01% |  |
| YUV, camera captured, 1080p |  | 0.00% |  | 0.00% |  | 0.01% |  |
| Enc Time[%] |  | 98% |  |  |  |  |  |
| Dec Time[%] |  | 100% |  |  |  |  |  |
|  |  | Random Access |  |  |  |  |  |
|  |  | G/Y |  | B/U |  | R/V |  |
| RGB, text & graphics with motion, 1080p & 720p |  | 0.04% |  | 0.00% |  | 0.02% |  |
| RGB, mixed content, 1440p & 1080p |  | -0.01% |  | -0.05% |  | -0.05% |  |
| RGB, Animation, 720p |  | -0.10% |  | 0.00% |  | -0.02% |  |
| RGB, camera captured, 1080p |  | 0.15% |  | 0.02% |  | 0.05% |  |
| YUV, text & graphics with motion, 1080p & 720p |  | 0.00% |  | -0.01% |  | 0.00% |  |
| YUV, mixed content, 1440p & 1080p |  | 0.05% |  | 0.10% |  | 0.10% |  |
| YUV, Animation, 720p |  | -0.02% |  | 0.01% |  | 0.03% |  |
| YUV, camera captured, 1080p |  | 0.02% |  | 0.04% |  | -0.06% |  |
| Enc Time[%] |  | 100% |  |  |  |  |  |
| Dec Time[%] |  | 100% |  |  |  |  |  |

Conclusions

It is recommended to adopt the proposed changes into the next VTM/VVC WD.

References

- B. Bross, J. Chen, and etc, “Versatile Video Coding (Draft 7),” JVET-P2001, Nov 2019.

- VTM-7.1 in https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

- X. Xu, Y.-C. Chao, Y.-C. Sun, J. Xu, “Description of Core Experiment 8 (CE8): 4:4:4 Screen Content Coding Tools”, JVET-O2028, July 2019, Gothenburg, Sweden.

Patent rights declaration(s)

Fujitsu may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).