| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0371-v3 |
| --- | --- |

| Title: | Unification of intra reference sample generation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Dae-Yeon Kim 11/12/13F, V&S Tower, 26 Samseong-ro 85-gil, Gangnam-gu, Seoul, 06194, KOREA Sung-Chang Lim Jinho Lee Jungwon Kang 218 Gajeong-ro, Yuseong-gu, Daejeon, 34129, KOREA | Tel: Email: | +82-10-3060-3361 brad.kim@chipsnmedia.com sclim@etri.re.kr jinosoul@etri.re.kr jungwon@etri.re.kr |
| Source: | Chips&Media and Electronics and Telecommunications Research Institute (ETRI) |  |  |

_____________________________

- Abstract

This contribution points out a mismatch between text and software in regard to intra reference sample generation for MIP. Whereas the VVC7 text generates the reference samples of above and left for MIP, the VTM7 software generates the reference samples of above, above-right, above-left, left, and left-bottom like general intra sample prediction modes. It is found that it would occur a mismatch on the reconstructed samples between text and software in case where multiple slices are used. To solve the problem, this proposal suggests to align the reference sample generation of the MIP to that of the general intra sample prediction.

Problem statement

The MIP generates the reference samples to predict the sample values of the current block. In regard to the reference sample generation, the VVC7 text[1] defines the MIP only generates above samples having tuWidth size, left samples having tuHeight size, and one corner sample which is not used. On the other hands, the VTM7[2] generates above samples having 2 * tuWidth size, left samples having 2 * tuHeight  that means it generates above-right and left-bottom samples as well as the above and the left samples. It is found that it would occur a mismatch between outputs by the text and outputs by the software in case where multiple slices are used and a CTU width is equal to a transform width. Fig. 1 (a) shows an example of the case where the mismatch would occur. In this case, the above-right samples are only available, because the above samples are inside a different slice, and the left samples are outside a picture. If the reference sample generation of the VVC7 text is used, then all neighbor samples are substituted with a medium value as shown in Fig. 1 (b). On the other hands, if the reference sample generation of the VTM7 software is used, then all neighbor samples are substituted with an above-right sample (A) as shown in Fig. 1 (c). Therefore, the prediction sample values of MIP would be different between the output of text and that of software.

Fig. 1 (a). An example of the case where the mismatch would occur.

Fig. 1 (b). The result when the reference sample generation of the VVC7 text is applied to the Fig. 1(a).

Fig. 1 (c). The result when the reference sample generation of the VTM7 software is applied to the Fig. 1(a).

Proposed method

To solve the issue, two methods can be considered as below.

Method #1. Unify with the general intra prediction (Align the text to the software)

Method #2. Align the software to the text

In this contribution, the method #1 is suggested which the reference sample generation of the text is aligned to the software’s behavior like the general intra mode. The reasons why the method #1 is suggested are that the coding efficiency impact of the two methods would be minor, and the method #1 is more encoder-friendly method. Specifically, the reference sample generation has to be performed for each modes, MIP and the other modes in a loop on an intra mode decision if the method #2 is used, whereas the MIP can re-uses the reference samples generated by the other modes in case of the method #1.

Proposed text changes

8.4.5.2.1 Matrix-based intra sample prediction … The variables refW and refH are derived as follows: refW = nTbW * 2 refH = nTbH * 2 For the generation of the reference samples refT[ x ] with x = 0..refW − 1 and refL[ y ] with y = 0..refH − 1, the following applies: - The reference sample availability marking process as specified in clause 8.4.5.2.7 is invoked with the sample location ( xTbCmp, yTbCmp ), reference line index equal to 0, the reference sample width refW, the reference sample height refH, colour component index equal to 0 as inputs, and the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1 as output. - When at least one sample refUnfilt[ x ][ y ] with x = −1, y = −1.. refH − 1 and x = 0.. refW − 1, y = −1 is marked as "not available for intra prediction", the reference sample substitution process as specified in clause 8.4.5.2.8 is invoked with reference line index 0, the reference sample width refW, the reference sample height refH, the reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..refH − 1 and x = 0..refW − 1, y = −1, and colour component index 0 as inputs, and the modified reference samples refUnfilt[ x ][ y ] with x = −1, y = −1..nTbH − 1 and x = 0..nTbW − 1, y = −1 as output.

Conclusions

This contribution points out a mismatch between text and software in regard to intra reference sample generation for MIP. It is found that it would occur a mismatch on the reconstructed samples between text and software in case where multiple slices are used and a CTU width is equal to a transform width. To solve the problem, this proposal suggests to align the reference sample generation of the MIP to that of the general intra sample prediction.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 7),” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET-P2001, Oct. 2019

- https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-7.1

Patent rights declaration(s)

Chips&Media may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).