| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0373 |
| --- | --- |

| Title: | AHG12: On signalling of slice |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jihoon Do Yong-Uk Yoon Dohyeon Park Jae-Gon Kim (KAU) | Tel: Email: | +82-2-300-0414 jhdo@kau.kr yuyoon@kau.kr dhpark@kau.kr jgkim@kau.ac.kr |
| Source: | Korea Aerospace University |  |  |

_____________________________

- Abstract

VVC (Draft 7) [1] includes signaling for tile and slices in PPS.

This contribution proposes some modifications on signaling of slice information in the PPS as follows.

- Proposal 1: It is proposed to infer the syntax element of slice_height_in_ctu_minus1 as value of 0 when the corresponding tile’s height is the same as the number of slice in the tile in the unit of CTU.

- Proposal 2: It is proposed to infer the syntax element of num_slices_in_pic_minus1 as value of 0 when the corresponding tile’s height is equal to 1 in the unit of CTU.

- Proposal 3: It is proposed to replace the syntax elements of num_slices_in_pic_minus1 as num_exp_slices_in_pic_minus1 which represents the number of last explicitly signaled slice height.

Proposal 1

In VVC Draft 7, when one or multiple slices exist in one tile, the syntax element of num_slices_in_pic_minus1 and slice_height_in_ctu_minus1 is signaled.

When multiple slices exist in one tile, the syntax element of num_slices_in_tile_minus1[ i ] is signaled to indicate the number of slices in the tile. If the number of slice and the height of the tile are the same, the height of slice is always 1.

In this contribution, the value of syntax elements slice_height_in_ctu_minus1[ i++ ] is inferred as 0 when the height of tile and number of slices in tile are the same. The proposed syntax changes are shown highlighted below comapred with VVC Draft 7.

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| if (Rowheight[tileY] != numSlicesInTileMinus1+1) |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |

The proposed modifications on semantics are shown below comapred with VVC Draft 7. The syntax element of slice_height_in_ctu_minus1[ i++ ] is not signaled when the Rowheight[tileY] is equal to numSlicesInTileMinus1+1 and it is inferred as 0.

- 7.4.3.4 Picture parameter set RBSP semantics

…

slice_height_in_ctu_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of CTU rows for the case where the i-th slice contains a subset of CTU rows from a single tile. The value of slice_height_in_ctu_minus1[ i ] shall be in the range of 0 to RowHeight[tileY ] − 1, inclusive, where tileY is the tile row index containing the i-th slice. When not present, the value of slice_height_in_ctu_minus1 [ i ] is inferred to be equal to 0.

6.5.1	CTB raster scanning, tile scanning, and subpicture scanning processes

tileIdx = 0
		for( i = 0; i  <=  num_slices_in_pic_minus1; i++ )
			NumCtuInSlice[ i ] = 0
		for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
			tileX = tileIdx % NumTileColumns
			tileY = tileIdx / NumTileColumns
			if( i  = =  num_slices_in_pic_minus1 ) {
				slice_width_in_tiles_minus1[ i ] = NumTileColumns − 1 − tileX
				slice_height_in_tiles_minus1[ i ] = NumTileRows − 1 − tileY
				num_slices_in_tile_minus1[ i ] = 0
			}
			if( slice_width_in_tiles_minus1[ i ]  = =  0  &&  slice_height_in_tiles_minus1[ i ]  = =  0 )
			{	
				ctbY = tileRowBd[ tileY ]
				numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ]
				if( Rowheight[tileY] != numSlicesInTileMinus1+1)
				{
					for( j = 0; j < numSlicesInTileMinus1; j++ ) {
						AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
								ctbY, ctbY + slice_height_in_ctu_minus1[ i ] + 1 )
						ctbY  +=  slice_height_in_ctu_minus1[ i ] + 1
						i++
					}
					AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ], 
							ctbY, tileRowBd[ tileY + 1 ] )
				} else {
					for( j = 0; j < numSlicesInTileMinus1; j++ ) {
						AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ], ctbY, ctbY + 1 )
					ctbY  += 1
					i++
					}
					AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ], ctbY, 
							tileRowBd[ tileY + 1 ] )
				} 
			} else
				for( j = 0; j  <=  slice_height_in_tiles_minus1[ i ]; j++ )
					for( k = 0; k  <=  slice_width_in_tiles_minus1[ i ]; k++ )
						AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ],
								tileRowBd[ tileY + j ], tileRowBd[ tileY + j + 1 ] )

Proposal 2

In VVC Draft 7, when one or multiple slices exist in one tile, the syntax element of num_slices_in_pic_minus1 and slice_height_in_ctu_minus1 are signaled.

In the case of multiple slices in one tile, the syntax of num_slices_in_pic_minus1 and slice_height_in_ctu_minus1 are signaled to indicate each slice’s height value in units of CTU.

In the case of one slice in one tile, only the syntax of num_slices_in_pic_minus1 is signaled and the slice_height_in_ctu_minus1 is inferred as the corresponding tile’s height value in units of CTU. If the height of that tile is equal to 1, the tile can have one slice only since the slice has height in units of CTU. That is, when the height of the tile is equal to 1, the size of num_slices_in_pic_minus1 is always fixed to 0.

In this contribution, the syntax elements of num_slices_in_pic_minus1 is inferred as 0 when the height of the tile is equal to 1. The proposed syntax changes are shown highlighted below comapred with VVC Draft 7.

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && !Rowheight[tileY]) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |

The proposed modifications on semantics are shown below comapred with VVC Draft 7. The syntax element of num_slices_in_tile_minus1[ i ] is not signaled when the Rowheight[tileY] is equal to 1 and it is inferred as 0.

- 7.4.3.4 Picture parameter set RBSP semantics

6.5.1	CTB raster scanning, tile scanning, and subpicture scanning processes

tileIdx = 0
		for( i = 0; i  <=  num_slices_in_pic_minus1; i++ )
			NumCtuInSlice[ i ] = 0
		for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
			tileX = tileIdx % NumTileColumns
			tileY = tileIdx / NumTileColumns
			if( i  = =  num_slices_in_pic_minus1 ) {
				slice_width_in_tiles_minus1[ i ] = NumTileColumns − 1 − tileX
				slice_height_in_tiles_minus1[ i ] = NumTileRows − 1 − tileY
				num_slices_in_tile_minus1[ i ] = 0
			}
		if( slice_width_in_tiles_minus1[ i ]  = =  0  &&  slice_height_in_tiles_minus1[ i ]  = =  0
			 && !Rowheight[tileY]) {	
			ctbY = tileRowBd[ tileY ]
			numSlicesInTileMinus1 = num_slices_in_tile_minus1[ i ]
			for( j = 0; j < numSlicesInTileMinus1; j++ ) {
				AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
						ctbY, ctbY + slice_height_in_ctu_minus1[ i ] + 1 )
				ctbY  +=  slice_height_in_ctu_minus1[ i ] + 1
				i++
			}
			AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ], ctbY, tileRowBd[ tileY + 1 ] )
		} 
		else if( Rowheight[tileY] )
		{
			ctbY = tileRowBd[ tileY ]
			AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ], ctbY, tileRowBd[ tileY + 1 ] )
		}
		else
			for( j = 0; j  <=  slice_height_in_tiles_minus1[ i ]; j++ )
				for( k = 0; k  <=  slice_width_in_tiles_minus1[ i ]; k++ )
					AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ],
						tileRowBd[ tileY + j ], tileRowBd[ tileY + j + 1 ] )

Proposal 3

In JVET-P0096 [2], the number of tile columns and rows are explicitly signaled without flags that indicate uniform and non-uniform tile.

In VVC Draft 7, the number of slices is signaled by the syntax of num_slices_in_tile_minus1. In this contribution, as the same concept with JVET-P0096, num_slices_in_tile_minu1 is replaced as num_exp_slices_in_tile_minus1 and the semantics are changed.

- 7.3.2.4 Picture parameter set RBSP syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < num_slices_in_pic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ] | ue(v) |
| slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 && !Rowheight[tileY]) { |  |
| num_exp_slices_in_tile_minus1[ i ] | ue(v) |
| numSlicesInTileMinus1 = num_exp_slices_in_tile_minus1[ i ] |  |
| for( j = 0; j < numSlicesInTileMinus1; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| } |  |
| if( tile_idx_delta_present_flag && i < num_slices_in_pic_minus1 ) |  |
| tile_idx_delta[ i ] | se(v) |
| } |  |

- 7.4.3.4 Picture parameter set RBSP semantics

…

num_slices_in_tile_minus1[ i ] plus 1 specifies the number of slices in the current tile for the case where the i-th slice contains a subset of CTU rows from a single tile. The value of num_slices_in_tile_minus1[ i ] shall be in the range of 0 to RowHeight[ tileY ] − 1, inclusive, where tileY is the tile row index containing the i-th slice. When not present, the value of num_slices_in_tile_minus1[ i ] is inferred to be equal to 0.

num_exp_slices_in_tile_minus1[ i ] plus 1 specifies the number of explicitly provided slices in the current tile for the case where the i-th slice contains a subset of CTU rows from a single tile. The value of num_slices_in_tile_minus1[ i ] shall be in the range of 0 to RowHeight[ tileY ] − 1, inclusive, where tileY is the tile row index containing the i-th slice. When not present, the value of num_slices_in_tile_minus1[ i ] is inferred to be equal to 0.

slice_height_in_ctu_minus1[ i ] plus 1 specifies the height of the i-th rectangular slice in units of CTU rows for the case where the i-th slice contains a subset of CTU rows from a single tile. The value of slice_height_in_ctu_minus1[ i ] shall be in the range of 0 to RowHeight[ tileY ] − 1, inclusive, where tileY is the tile row index containing the i-th slice. plus 1 specifies the height of the i-th rectangular slice inthe units of CTU rows for i in the range of 0 to num_exp_slices_in_tile_minus1− 1, inclusive. slice_height_in_ctu_minus1[ num_exp_slices_in_tile_minus1 ] is used to derive the height of the slice with index greater than or equal to num_exp_slices_in_tile_minus1 as specified in clause 6.5.1. The value of slice_height_in_ctu_minus1[ i ] shall be in the range of 0 to RowHeight[ tileY ] − 1, inclusive, where tileY is the tile row index containing the i-th slice.

6.5.1	CTB raster scanning, tile scanning, and subpicture scanning processes

tileIdx = 0
		for( i = 0; i  <=  num_slices_in_pic_minus1; i++ )
			NumCtuInSlice[ i ] = 0
		for( i = 0; i  <=  num_slices_in_pic_minus1; i++ ) {
			tileX = tileIdx % NumTileColumns
			tileY = tileIdx / NumTileColumns
			if( i  = =  num_slices_in_pic_minus1 ) {
				slice_width_in_tiles_minus1[ i ] = NumTileColumns − 1 − tileX
				slice_height_in_tiles_minus1[ i ] = NumTileRows − 1 − tileY
				num_slices_in_tile_minus1[ i ] = 0
			}
			if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0)
			{	
				ctbY = tileRowBd[ tileY ]
				numSlicesInTileMinus1 = num_exp_slices_in_tile_minus1[ i ]
				for( j = 0; j < numSlicesInTileMinus1; j++ ) {
					AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
							ctbY, ctbY + slice_height_in_ctu_minus1[ i ] + 1 )
					ctbY  +=  slice_height_in_ctu_minus1[ i ] + 1
					i++
				}
				uniformSliceHeight = slice_height_in_ctu_minus1 [ numSlicesInTileMinus1] + 1
				while (ctbY + uniformSliceHeight <= tileRowBd[ tileY + 1])
				{
					AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
							ctbY, ctbY + slice_height_in_ctu_minus1[ numSlicesInTileMinus1 ] + 1 )
					ctbY  +=  slice_height_in_ctu_minus1[ numSlicesInTileMinus1 ] + 1
					i++
				}
				if( tileRowBd[ tileY+1 ] – ctbY > 0)
					AddCtbsToSlice( i, tileColBd[ tileX ], tileColBd[ tileX + 1 ],
							ctbY, tileRowBd[ tileY + 1 ] )
			}else
				for( j = 0; j  <=  slice_height_in_tiles_minus1[ i ]; j++ )
					for( k = 0; k  <=  slice_width_in_tiles_minus1[ i ]; k++ )
						AddCtbsToSlice( i, tileColBd[ tileX + k ], tileColBd[ tileX + k + 1 ],
							tileRowBd[ tileY + j ], tileRowBd[ tileY + j + 1 ] )

References

- B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7),” JVET-P2001, Oct. 2019.

- M. M. Hannuksela, “AHG12: On signalling of tile and brick partitioning,” JVET-P0096, Oct. 2019.

Patent rights declaration(s)

Korea Aerospace University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).