| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0377 |
| --- | --- |

| Title: | AHG12: On “tile-fraction” slices and signaling of slices per subpicture |  |  |
| --- | --- | --- | --- |
| Status: | JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naël Ouedraogo Guillaume Laroche Patrice Onno Canon Research Centre France Rue de la Touche Lambert 35510 CESSON-SEVIGNE, FRANCE | Tel: Email: | +33 299876800 nael.ouedraogo@crf.canon.fr |
| Source: | Canon Research Centre France. |  |  |

_____________________________

# Abstract

This contribution proposes to signal the slice partitioning per subpicture instead of at picture level in the PPS. Secondly, it proposes to limit the usage of slices representing a fraction of tiles in combination with subpictures. It is asserted that this limitation is reasonable since the initial concept of “tile-fraction” slices targets bitstream extraction and merging operations for omnidirectional content for which subpictures where adopted in VVC.

The proposed syntax removes three syntax elements from the PPS and adds a new one, which is asserted as a simplification of the PPS syntax. It is reported that it provides a better coding efficiency of the slice signalling in particular when tiles are subdivided in multiple slices.

# Introduction

The signalling of the slice structure in VVC draft 7 is made at picture level. Slices partition is made on top of the tile structure without reference to subpicture. On the other hand, the specification constrains subpictures to contain one or more slices and the slice header describes the slice_address syntax element which is the index of the slice relatively to its subpicture. We think that current syntax is not enforcing all these constraints which is prone to generating bitstreams which are not in conformance with VVC requirements.

In addition, the requirement to define multiple slices (or initially bricks) within a tile (named “tile-fraction” slices in this document) addresses OMAF use cases (as represented in the figure 1) that require particular patterns of regions specific bitstream for extraction and merging operations.. It was identified, that there is a need to define slices within a tile to address such use cases.

Figure 1: Example of regions pattern for OMAF

In such approaches, we think each "tile-fraction" slice would be probably in different subpictures to allow the merging operations. From compression point of view, we think it makes not so much sense to have a subpicture containing one complete tile with multiple “tile-fraction” slices. For this reason, we think it is not necessary to define subpicture containing several tile-fraction slices. Moreover, in current specification tile-fraction slices are not allowed in raster scan slice mode which shows that tile-fraction slices are envisaged only for subpicture approaches.

In this contribution, we propose to signal the slice partitioning at subpicture level instead of picture level in the PPS. The width and height of the slices are signalled in tile units. It is possible to define a subpicture as a fraction of tile and in that case it is constrained to contain a single slice of size equal to the subpicture size. With this approach, it is possible to define "tile-fraction" slice when it covers a complete subpicture which is also a fraction of a tile.

# Proposal

The PPS syntax is modified as follows:

- num_slices_in_pic_minus1, num_slices_in_tile_minus1[ i ] and slice_height_in_ctu_minus1[ i ] are removed

- A new syntax element num_slices_in_subpic_minus1[ i ] specifies the number of slices per subpicture

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| […] |  |
| no_pic_partition_flag | u(1) |
| if( !no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| num_exp_tile_columns_minus1 | ue(v) |
| num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= num_exp_tile_columns_minus1; i++ ) |  |
| tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= num_exp_tile_rows_minus1; i++ ) |  |
| tile_row_height_minus1[ i ] | ue(v) |
| rect_slice_flag | u(1) |
| if( rect_slice_flag ) { |  |
| single_slice_per_subpic_flag | u(1) |
| if( rect_slice_flag && !single_slice_per_subpic_flag ) { |  |
| num_slices_in_pic_minus1 | ue(v) |
| tile_idx_delta_present_flag | u(1) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) { |  |
| num_slices_in_subpic_minus1[ i ] | ue(v) |
| for( i = 0; i < num_slices_in_subpic_minus1; i++ ) { |  |
| slice_width_in_tiles_minus1[ i ][ j ] | ue(v) |
| slice_height_in_tiles_minus1[ i ][ j ] | ue(v) |
| if( slice_width_in_tiles_minus1[ i ] = = 0 && slice_height_in_tiles_minus1[ i ] = = 0 ) { |  |
| num_slices_in_tile_minus1[ i ] | ue(v) |
| for( j = 0; j < num_slices_in_tile_minus1[ i ]; j++ ) |  |
| slice_height_in_ctu_minus1[ i++ ] | ue(v) |
| } |  |
| if( tile_idx_delta_present_flag && ij < num_slices_in_subpic_minus1 ) |  |
| tile_idx_delta[ i ][ j ] | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| loop_filter_across_tiles_enabled_flag | u(1) |
| loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| […] |  |
| } |  |

With the following modifications for the semantics:

num_slices_in_subpic_minus1[ i ] plus 1 specifies the number of rectangular slices in the i-th subpicture. The value of num_slices_in_subpic_minus1 shall be in the range of 0 to MaxSlicesPerPicture − 1, inclusive, where MaxSlicesPerPicture is specified in Annex A. When no_pic_partition_flag is equal to 1, the value of num_slices_in_subpic_minus1[ 0 ] is inferred to be equal to 0.

slice_width_in_tiles_minus1[ i ][ j ] plus 1 specifies the width of the ij-th rectangular slice in units of tile columns in the i-th subpicture. The value of slice_width_in_tiles_minus1[ i ][ j ] shall be in the range of 0 to NumTileColumns − 1, inclusive. When not present, the value of slice_width_in_tiles_minus1[ i ][ j ] is inferred as specified in clause 6.5.1.

slice_height_in_tiles_minus1[ i ][ j ] plus 1 specifies the height of the ij-th rectangular slice in units of tile rows in the i-th subpicture. The value of slice_height_in_tiles_minus1[ i ][ j ] shall be in the range of 0 to NumTileRows − 1, inclusive. When not present, the value of slice_height_in_tiles_minus1[ i ][ j ] is inferred as specified in clause 6.5.1.

tile_idx_delta[ i ][ j ] specifies the difference in tile index between the ij-th rectangular slice and the ( ij + 1 )-th rectangular slice of the i-th subpicture. The value of tile_idx_delta[ i ][ j ] shall be in the range of –NumTilesInPic + 1 to NumTilesInPic − 1, inclusive. When not present, the value of tile_idx_delta[ i ][ j ] is inferred to be equal to 0. In all other cases, the value of tile_idx_delta[ i ][j ] shall not be equal to 0.

The modification of the scanning processes of section 6.5.1 is provided in the attached specification text.

# Conclusion

This proposal simplifies the slice signalling by reducing the number of syntax elements of the PPS. In addition, using information from the subpicture layout definition to specify the slice partition may provide a better coding efficiency of the slice signalling in particular when tiles are subdivided in multiple slices.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).