| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET- Q0380 |
| --- | --- |

| Title: | LFNST restriction based on MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ankit Kumar Sandeep Shrestha Bumshik Lee Yonggil Lee Junsuk Park | Email: | ankitku@chosun.kr sandeepshrestha407@chosun.kr bslee@chosun.ac.kr yglee@humaxdigital.com parkjs@humaxdigital.com |
| Source: | Chosun University and Humax |  |  |

_____________________________

- Abstract

In VTM-7.0 [1], MIP [2] and LFNST [3] have been harmonized, where LFNST is allowed for the MIP blocks with width and height greater than or equal to 16. In addition, it has been decided that MIP is applied to all CU blocks without any restrictions of block sizes [5] in VTM7.0. Hence, we propose to modify the number of block sizes to apply LFNST for MIP blocks. In this contribution, we propose two sets of block sizes to apply LFNST on MIP blocks. First, where either height or width are greater than or equal to 16 or (maximum(height, width) greater than or equal to 32 && minimum(height, width) greater than or equal to 4), which includes 4×32, 4×64, 8×32, 8×64, 16×16, 16×32, 16×64, 32×4, 32×8, 32×16, 32×32, 32×64, 64×4, 64×8, 64×16, 64×32 and 64×64 block sizes. In the second test, where either height or width both are greater than or equal to 16 or (H>=4 && W>=64) and vice-versa or (H>=8 && W>=32) and vice versa, which includes all right diagonal blocks as 4×64, 8×32, 8×64, 16×16, 16×32, 16×64, 32×8, 32×16, 32×32, 32×64, 64×4, 64×8, 64×16, 64×32 and 64×64 block sizes.

Proposed Method

The Fig. 1 shows the types of blocks to apply LFNST on MIP blocks in VTM 7.0. As shown in Fig. 1, LFNST is only applied to the MIP blocks with width and height greater than or equal to 16. In the 16-th JVET meeting in Geneva, it has been decided that the restriction of MIP block size has been removed, resulting in that MIP is applied to all CU blocks. Thus, 4×32, 4×64, 8×64, 32×4, 64×4 and 64×8 blocks are newly added for MIP in VTM 7.0. These added blocks can also be taken into consideration to apply LFNST.

| 4×4 | 8×4 | 16×4 | 32×4 | 64×4 |
| --- | --- | --- | --- | --- |
| 4×8 | 8×8 | 16×8 | 32×8 | 64×8 |
| 4×16 | 8×16 | 16×16 | 32×16 | 64×16 |
| 4×32 | 8×32 | 16×32 | 32×32 | 64×32 |
| 4×64 | 8×64 | 16×64 | 32×64 | 64×64 |

Fig. 1. LFNST is applied on the marked MIP blocks in VTM-7.0

Method 1:

In this test, we propose to apply LFNST to MIP blocks where either of height or width both are greater than or equal to 16 or (maximum(height, width) greater than or equal to 32 && minimum(height, width) greater than or equal to 4). Fig. 2 shows the block sizes taken in consideration for proposed method 1.

| 4×4 | 8×4 | 16×4 | 32×4 | 64×4 |
| --- | --- | --- | --- | --- |
| 4×8 | 8×8 | 16×8 | 32×8 | 64×8 |
| 4×16 | 8×16 | 16×16 | 32×16 | 64×16 |
| 4×32 | 8×32 | 16×32 | 32×32 | 64×32 |
| 4×64 | 8×64 | 16×64 | 32×64 | 64×64 |

Fig 2. LFNST is applied on the marked MIP coded blocks for proposed Test 1

Method 2:

In this test, we propose to apply LFNST to MIP coded blocks whose either of height or width both are greater than or equal to 16 or (H>=4 && W>=64) and vice-versa or (H>=8 && W>=32) and vice versa. Fig. 3 shows the block sizes taken in consideration for proposed method 2.

| 4×4 | 8×4 | 16×4 | 32×4 | 64×4 |
| --- | --- | --- | --- | --- |
| 4×8 | 8×8 | 16×8 | 32×8 | 64×8 |
| 4×16 | 8×16 | 16×16 | 32×16 | 64×16 |
| 4×32 | 8×32 | 16×32 | 32×32 | 64×32 |
| 4×64 | 8×64 | 16×64 | 32×64 | 64×64 |

Fig 3. LFNST is applied on the marked MIP coded blocks for proposed Test 2

Proposed Text

The spec changes for the proposed method for Test 1:

- 7.3.9.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ¦¦ ( Max( lfnstWidth, lfnstHeight)>=32 && Min(lfnstWidth, lfnstHeight)>=4 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

The spec changes for the proposed method 2:

- 7.3.9.5 Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType != DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ¦¦ (lfnstWidth>=4 && lfnstHeight>=64) ¦¦ (lfnstWidth>=64 && lfnstHeight>=4) ¦¦ (lfnstWidth==8 && lfnstHeight==32) ¦¦ (lfnstWidth==32 && lfnstHeight==8) && Max( cbWidth, cbHeight ) <= MaxTbSizeY) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 ) |  |
| lfnst_idx | ae(v) |
| } |  |

Experimental results

The proposed method has been implemented on VTM-7.0 reference software and evaluated for VTM test according to common test conditions for AI and RA configurations [4].

For Proposed method 1:

Table 1. Experimental results for AI configurations over VTM-7.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.14% | 0.11% | 98% | 99% |
| Class A2 | 0.01% | 0.00% | -0.07% | 98% | 99% |
| Class B | 0.01% | -0.01% | -0.05% | 98% | 98% |
| Class C | 0.00% | -0.03% | 0.01% | 99% | 96% |
| Class E | 0.00% | -0.01% | 0.10% | 98% | 95% |
| Overall | 0.00% | -0.03% | 0.01% | 98% | 97% |
| Class D | 0.00% | -0.05% | 0.17% | 99% | 95% |
| Class F | -0.02% | 0.03% | 0.10% | 97% | 95% |

Table 2. Experimental results of RA configurations over VTM-7.0

|  | Random access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

For Proposed method 2:

Table 3. Experimental results for AI configurations over VTM-7.0

|  | All Intra Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.11% | 0.09% | 96% | 99% |
| Class A2 | -0.01% | 0.03% | -0.06% | 95% | 100% |
| Class B | 0.00% | -0.03% | -0.10% | 97% | 98% |
| Class C | 0.00% | 0.00% | -0.07% | 98% | 95% |
| Class E | 0.01% | -0.08% | 0.09% | 98% | 95% |
| Overall | 0.00% | -0.03% | -0.02% | 97% | 97% |
| Class D | 0.01% | -0.03% | 0.07% | 100% | 95% |
| Class F | -0.01% | -0.01% | 0.03% | 97% | 95% |

Table 4. Experimental results of RA configurations over VTM-7.0

|  | Random access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 7),” JVET-P2001, 16th Meeting: Geneva, CH, 1–11 Oct. 2019

- J. Pfaff et al., “CE3: Affine linear weighted intra prediction (CE3-4.1, CE3-4.2)”, JVET-N0217, Geneva, Switzerland, March 2019

- M. Koo et al., “CE6: Reduced Secondary Transform (RST)(CE6-3.1)”, JVET-N0193, 14th meeting, Geneva, CH, 19-27 March 2019.

- F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, 14th JVET meeting, Geneva, 19–27 Mar. 2019.

- J. Choi, J. Heo, J. Lim, “Non-CE3: Removal of block size restriction in MIP”, JVET-P0535, 16th meeting, Geneva, CH, 1-11 October 2019.

Patent rights declaration(s)

Chosun University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Humax may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).