| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0382-v1 |
| --- | --- |

| Title: | CE5-related: On high level syntax of CC-ALF |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fangdong Chen Liying Xu Li Wang | Tel: Email: | +86 15156063604 chenfangdong@hikvision.com |
| Source: | Hikvision |  |  |

_____________________________

- Abstract

This proposal reports the modifications on the high level syntax of cross component adaptive loop filter (CC-ALF). First, it introduces a new SPS level syntax for CC-ALF to decouple of high level syntax control of ALF and CCALF. Second, a bug that CC-ALF related syntax in picture header (PH) is still signalled even if two chroma arrays are unavailable is fixed. Third, the numbers of filters for CC-ALF in slice and picture header are removed.

Proposed Modification

Modification 1

Currently, ALF and CC-ALF share the same SPS level syntax. However, it is found that the CC-ALF can achieve the similar coding gains even if ALF is disabled. Hence, it is reasonable to introduce a new SPS level syntax for CC-ALF to decouple of high level syntax control of ALF and CCALF. This modification will help to provide encoders with better flexibility.

Modification 2

Moreover, the CC-ALF related syntax in picture header (PH) is still signalled even if two chroma arrays are unavailable (ChromaArrayType equals 0). This bug is also fixed in the proposal.

Modification 3

The numbers of filters for CC-ALF in slice and picture header are unnecessary. Hence it is proposed to remove the numbers of filters for CC-ALF in slice and picture header, i.e., the syntax elements pic_cross_component_cb_filters_signalled_minus1, pic_cross_component_cr_filters_signalled_minus1, slice_cross_component_cb_filters_signalled_minus1, slice_cross_component_cr_filters_signalled_minus1 should be removed.

Related syntax modifications

The first modifications on top of the CCALF specification text provided in JVET-Q0058 [1] are marked with green, and the second ones are marked with red, and the third ones are marked with yellow.

7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| …… |  |
| sps_alf_enabled_flag | u(1) |
| sps_cross_component alf_enabled_flag | u(1) |

7.3.3.2	 General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| …. |  |
| no_alf_constraint_flag | u(1) |
| no_cross_component alf_constraint_flag | u(1) |

7.3.2.6	Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| …….. |  |
| if( sps_alf_enabled_flag ) { |  |
| pic_alf_enabled_present_flag | u(1) |
| if( pic_alf_enabled_present_flag ) { |  |
| pic_alf_enabled_flag | u(1) |
| if( pic_alf_enabled_flag ) { |  |
| pic_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < pic_num_alf_aps_ids_luma; i++ ) |  |
| pic_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| pic_alf_chroma_idc | u(2) |
| if( pic_alf_chroma_idc ) |  |
| pic_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| } |  |
| if( sps_ccalf_enabled_flag && ChromaArrayType != 0 ) { |  |
| pic_cross_component_alf_enabled_present_flag | u(1) |
| if( pic_cross_component_alf_enabled_present_flag) { |  |
| pic_cross_component_alf_cb_enabled_flag | u(1) |
| if( pic_cross_component_alf_cb_enabled_flag ) { |  |
| pic_cross_component_alf_cb_aps_id | u(3) |
| pic_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| pic_cross_component_alf_cr_enabled_flag | u(1) |
| if( pic_cross_component_alf_cr_enabled_flag ) { |  |
| pic_cross_component_alf_cr_aps_id | u(3) |
| pic_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |
| } |  |

7.3.7.1	General slice header syntax

| slice_header( ) { | Descriptor |
| --- | --- |
| ……. |  |
| if( sps_alf_enabled_flag && !pic_alf_enabled_present_flag ) { |  |
| slice_alf_enabled_flag | u(1) |
| if( slice_alf_enabled_flag ) { |  |
| slice_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < slice_num_alf_aps_ids_luma; i++ ) |  |
| slice_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) |  |
| slice_alf_chroma_idc | u(2) |
| if( slice_alf_chroma_idc ) |  |
| slice_alf_aps_id_chroma | u(3) |
| } |  |
| } |  |
| if( sps_cross_component_alf_enabled_flag && !pic_cross_component_alf_enabled_present_flag ) { |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cb_enabled_flag | u(1) |
| if( slice_cross_component_alf_cb_enabled_flag ) { |  |
| slice_cross_component_alf_cb_aps_id | u(3) |
| slice_cross_component_cb_filters_signalled_minus1 | ue(v) |
| } |  |
| if( ChromaArrayType != 0 ) |  |
| slice_cross_component_alf_cr_enabled_flag | u(1) |
| if( slice_cross_component_alf_cr_enabled_flag ) { |  |
| slice_cross_component_alf_cr_aps_id | u(3) |
| slice_cross_component_cr_filters_signalled_minus1 | ue(v) |
| } |  |
| } |  |

The semantics of the newly introduced syntax elements are as follows:

sps_cross_component_alf_enabled_flag equal to 0 specifies that the cross component adaptive loop filter is disabled. sps_cross_component_alf_enabled_flag equal to 1 specifies that the cross component adaptive loop filter is enabled.

no_cross_component_alf_constraint_flag equal to 1 specifies that sps_cross_component_alf_enabled_flag shall be equal to 0.  no_cross_component_alf_constraint_flag equal to 0 does not impose such a constraint.

pic_cross_component_alf_enabled_present_flag equal to 1 specifies that pic_cross_component_alf_cb_enabled_flag, pic_cross_component_alf_cb_aps_id, pic_cross_component_alf_cb_filter_count_minus1, pic_cross_component_alf_cr_enabled_flag, pic_cross_component_alf_cr_aps_id and pic_cross_component_alf_cr_filter_count_minus1 are present in the PH(picture header). pic_alf_enabled_present_flag equal to 0 specifies that pic_cross_component_alf_cb_enabled_flag, pic_cross_component_alf_cb_aps_id, pic_cross_component_alf_cb_filter_count_minus1, pic_cross_component_alf_cr_enabled_flag, pic_cross_component_alf_cr_aps_id and pic_cross_component_alf_cr_filter_count_minus1 are not present in the PH. When pic_ccalf_enabled_present_flag is not present, it is inferred to be equal to 0.

pic_alf_enabled_present_flag equal to 1 specifies that pic_alf_enabled_flag, pic_num_alf_aps_ids_luma, pic_alf_aps_id_luma[ i ], pic_alf_chroma_idc, pic_alf_aps_id_chroma, pic_cross_component_alf_cb_enabled_flag, pic_cross_component_alf_cb_aps_id,  pic_cross_component_cb_filters_signalled_minus1, pic_cross_component_alf_cr_enabled_flag, pic_cross_component_alf_cr_aps_id, and pic_cross_component_cr_filters_signalled_minus1 are present in the PH. pic_alf_enabled_present_flag equal to 0 specifies that pic_alf_enabled_flag, pic_num_alf_aps_ids_luma, pic_alf_aps_id_luma[ i ], pic_alf_chroma_idc, pic_alf_aps_id_chroma, pic_cross_component_alf_cb_enabled_flag, pic_cross_component_alf_cb_aps_id,  pic_cross_component_cb_filters_signalled_minus1, pic_cross_component_alf_cr_enabled_flag, pic_cross_component_alf_cr_aps_id, and pic_cross_component_cr_filters_signalled_minus1 are not present in the PH. When pic_alf_enabled_present_flag is not present, it is inferred to be equal to 0.

pic_cross_component_cb_filters_signalled_minus1 plus 1 specifies the number of cross component Cb filters. The value of pic_cross_component_cb_filters_signalled_minus1 shall be in the range 0 to 3

When pic_cross_component_alf_cb_enabled_flag equal to 1, it is a requirement of bitstream conformance that pic_cross_component_cb_filters_signalled_minus1 shall be less than or equal to the value of alf_cross_component_cb_filters_signalled_minus1 in the referenced ALF APS referred to by pic_cross_component_alf_cb_aps_id.

pic_cross_component_cr_filters_signalled_minus1 plus 1 specifies the number of cross component Cr filters. The value of pic_cross_component_cr_filters_signalled_minus1 shall be in the range 0 to 3

When pic_cross_component_alf_cr_enabled_flag equal to 1, it is a requirement of bitstream conformance that pic_cross_component_cr_filters_signalled_minus1 shall be less than or equal to the value of alf_cross_component_cr_filters_signalled_minus1 in the referenced ALF APS referred to by pic_cross_component_alf_cr_aps_id.

slice_cross_component_cb_filters_signalled_minus1 plus 1 specifies the number of cross component Cb filters. The value of slice_cross_component_cb_filters_signalled_minus1 shall be in the range 0 to 3. When slice_cross_component_alf_cb_enabled_flag is equal to 1 and slice_cross_component_cb_filters_signalled_minus1 is not present, the value of slice_cross_component_cb_filters_signalled_minus1 is inferred to be equal to the value of pic_cross_component_cb_filters_signalled_minus1.

When slice_cross_component_alf_cb_enabled_flag equal to 1, it is a requirement of bitstream conformance that slice_cross_component_cb_filters_signalled_minus1 shall be less than or equal to the value of alf_cross_component_cb_filters_signalled_minus1 in the ALF APS referred to by slice_cross_component_alf_cb_aps_id of current slice.

slice_cross_component_cr_filters_signalled_minus1 plus 1 specifies the number of cross component Cr filters. The value of slice_cross_component_cr_filters_signalled_minus1 shall be in the range 0 to 3. When slice_cross_component_alf_cr_enabled_flag is equal to 1 and slice_cross_component_cr_filters_signalled_minus1 is not present, the value of slice_cross_component_cr_filters_signalled_minus1 is inferred to be equal to the value of pic_cross_component_cr_filters_signalled_minus1.

When slice_cross_component_alf_cr_enabled_flag equal to 1, it is a requirement of bitstream conformance that slice_cross_component_cr_filters_signalled_minus1 shall be less than or equal to the value of alf_cross_component_cr_filters_signalled_minus1 in the referenced ALF APS referred to by slice_cross_component_alf_cr_aps_id of current slice.

Table 123 – Syntax elements and associated binarizations

| coding_tree_unit( ) | alf_ctb_flag[ ][ ][ ] | FL | cMax = 1 |
| --- | --- | --- | --- |
|  | alf_use_aps_flag | FL | cMax = 1 |
|  | alf_luma_fixed_filter_idx | TB | cMax = 15 |
|  | alf_luma_prev_filter_idx | TB | cMax = slice_num_alf_aps_ids_luma − 1 |
|  | alf_ctb_filter_alt_idx[ ][ ][ ] | TR | cMax = alf_chroma_num_alt_filters_minus1, cRiceParam = 0 |
|  | alf_ctb_cross_component_cb_idc[ ][ ] | TR | cMax = alf_cross_component_cb_filters_signalled_minus1 ( slice_cross_component_cb_filters_signalled_minus1 + 1 ), cRiceParam = 0 |
|  | alf_ctb_cross_component_cr_idc[ ][ ] | TR | cMax = alf_cross_component_cr_filters_signalled_minus1 ( slice_cross_component_cr_filters_signalled_minus1 + 1 ), cRiceParam = 0 |

Reference

- K. Misra, et al., “CE5 Common Base: Cross Component Adaptive Loop Filter”, JVET-Q0058, Brussels, BE, 7–17 January 2020.

Patent rights declaration(s)

Hikvision Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).