| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0385 |
| --- | --- |

| Title: | Simplification of intra prediction in CIIP mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ramin Ghaznavi-Youvalari Jani Lainema Krit Panusopone | Email: | ramin.ghaznavi-youvalari@nokia.com jani.lainema@nokia.com krit.panusopone@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution proposes to simplify the intra prediction part of the combined inter intra prediction (CIIP) mode by disabling or limiting the usage of the PDPC filter. Three alternatives are proposed in this contribution. The first method is to disable the PDPC filter always when the block is coded in CIIP mode. The second method enables the PDPC filter only when at least one of the neighboring blocks is coded in intra prediction mode. The third method enables the PDPC when at least one of the neighboring blocks is coded in intra mode or includes residual coding. The simulation results in VTM-7.0 are as below:

Method 1:

RA:  0.01%, -0.03%, 0.01%

LB: -0.01%, -0.02%, -0.05%

Method 2:

RA:  0.00%, -0.01%, -0.01%

LB: -0.01%, 0.03%, -0.07%

Method 3:

RA:  0.01%, 0.00%, 0.03%

LB: -0.03%, -0.16%, -0.04%

The proposed methods reduce the complexity of CIIP mode in terms of number of calculations with almost no impact in coding performance and execution times.

# Proposed changes

The combined inter intra prediction (CIIP) tool uses planar prediction for intra prediction part. By default, the planar prediction includes applying the PDPC filters to the predicted intra block. This contribution proposes to simplify this process by disabling the PDPC filter always or applying it to the intra prediction part depending on the neighboring block’s (the same neighboring blocks that are used for weight derivation in CIIP) prediction modes. Three methods have been considered as below:

Method 1: Disable the PDPC filter always when the block is in CIIP mode.

Method 2: Enable the PDPC filter when at least one of the neighboring blocks (i.e., top and left) is coded in intra prediction mode.

Method 3: Enable the PDPC filter when at least one of the neighboring blocks (i.e., top and left) is coded in intra prediction mode or consists residual coding.

Figure 1. Top and left neighboring blocks used in PDPC usage decision

# Results

The objective performances of the proposed methods are shown in below tables in comparison to VTM-7.0 using CTC.

Table 1. BD-Rate (%) result of method #1 with RA and LB configurations

|  | Random access |  |  |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00 % | -0.08 % | -0.03 % | 100 % | 101 % |  |  |  |  |  |
| Class A2 | -0.03 % | -0.01 % | 0.05 % | 100 % | 101 % |  |  |  |  |  |
| Class B | 0.01 % | -0.04 % | 0.01 % | 100 % | 101 % | 0.01 % | -0.19 % | 0.04 % | 100 % | 100 % |
| Class C | 0.03 % | 0.01 % | 0.02 % | 100 % | 100 % | 0.01 % | -0.16 % | 0.07 % | 100 % | 101 % |
| Class E |  |  |  |  |  | -0.07 % | 0.47 % | -0.37 % | 100 % | 101 % |
| Overall | 0.01 % | -0.03 % | 0.01 % | 100 % | 101 % | -0.01 % | -0.02 % | -0.05 % | 100 % | 101 % |
| Class D | -0.03 % | -0.07 % | 0.07 % | 100 % | 102 % | -0.02 % | -0.04 % | -0.18 % | 100 % | 103 % |
| Class F | -0.01 % | -0.05 % | 0.00 % | 99 % | 101 % | 0.01 % | -0.57 % | 0.61 % | 100 % | 101 % |

Table 2. BD-Rate (%) result of method #2 with RA and LB configurations

|  | Random access |  |  |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02 % | -0.06 % | -0.04 % | 100 % | 100 % |  |  |  |  |  |
| Class A2 | -0.01 % | -0.04 % | 0.03 % | 100 % | 100 % |  |  |  |  |  |
| Class B | 0.00 % | 0.01 % | -0.05 % | 99 % | 100 % | 0.02 % | -0.09 % | 0.06 % | 99 % | 100 % |
| Class C | 0.00 % | 0.01 % | 0.03 % | 100 % | 96 % | 0.05 % | -0.07 % | 0.12 % | 99 % | 101 % |
| Class E |  |  |  |  |  | -0.13 % | 0.37 % | -0.53 % | 99 % | 101 % |
| Overall | 0.00 % | -0.01 % | -0.01 % | 100 % | 99 % | -0.01 % | 0.03 % | -0.07 % | 99 % | 101 % |
| Class D | 0.01 % | -0.08 % | -0.10 % | 99 % | 98 % | -0.09 % | -0.01 % | -0.35 % | 99 % | 101 % |
| Class F | -0.04 % | -0.01 % | 0.02 % | 100 % | 97 % | 0.00 % | -0.27 % | 0.62 % | 99 % | 99 % |

Table 3. BD-Rate (%) result of method #3 with RA and LB configurations

|  | Random access |  |  |  |  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.02 % | -0.06 % | 0.00 % | 101 % | 100 % |  |  |  |  |  |
| Class A2 | -0.03 % | 0.06 % | -0.02 % | 101 % | 100 % |  |  |  |  |  |
| Class B | 0.00 % | 0.03 % | 0.06 % | 100 % | 100 % | -0.01 % | -0.24 % | -0.08 % | 100 % | 99 % |
| Class C | 0.03 % | -0.04 % | 0.05 % | 99 % | 99 % | -0.04 % | -0.24 % | 0.05 % | 99 % | 100 % |
| Class E |  |  |  |  |  | -0.06 % | 0.07 % | -0.10 % | 99 % | 99 % |
| Overall | 0.01 % | 0.00 % | 0.03 % | 100 % | 100 % | -0.03 % | -0.16 % | -0.04 % | 100 % | 99 % |
| Class D | 0.00 % | -0.01 % | 0.11 % | 100 % | 100 % | -0.11 % | 0.43 % | -0.26 % | 99 % | 100 % |
| Class F | -0.02 % | -0.01 % | -0.02 % | 99 % | 99 % | 0.00 % | -0.57 % | -0.18 % | 99 % | 98 % |

# Proposed specification text

TBD

# Conclusion

The proposed methods simplify the intra prediction scheme of CIIP tool. Furthermore, the required changes in the spec and software are very minimal.  It is proposed to adopt one of the described alternatives to the VVC draft and VTM software.

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).