| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0386 |
| --- | --- |

| Title: | AHG16: Feature-rich implementation of VVC real-time decoding and playback on ARM based mobile clients |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Jeeva Raj A Sagar Kotecha Shailesh Ramamurthy Anil Kumar Bedgujar Neha Mary Thomas Abhishek Chelawat Jayasanker J Saurabh Agrawal Vijayakumar GR Kaustubh Patankar | Tel: Email: | +91-8066601146 +91-8066601027 jeeva.raj@ittiam.com sagar.kotecha@ittiam.com Shailesh.Ramamurthy@ittiam.com |
| Source: | Ittiam systems private limited |  |  |

_____________________________

# Abstract

A real time decoder is the most important enabler for widespread adoption of any video coding technology or Standard. In this informative input contribution to JVET, We provide salient aspects of a feature-rich implementation of a VVC software decoder running on ARM based mobile clients. This implementation encompasses all tools that are essential for random-access VVC configuration. The software decoder has been implemented by Ittiam, with all critical modules optimized for ARM, is compatible with the syntax of VTM 7.0

# Introduction

One of Ittiam’s major focus areas is embedded software implementations, optimized for the widely-prevalent ARM based mobile platform.

Using Single Instruction Multiple Data (SIMD) on ARM, the performance achieved in our full-fledged implementation of VVC decoder demo is characterized below.

- 1920x1080@24fps on with 4 cores of Cortex-A75 clocked at 2.5GHz. 4 Tiles have been used towards a multi-threaded decoding application

- All tools essential for random access configuration of VVC is included in this implementation. All the critical modules/tools are optimized for ARM architecture.

- For effective parallelization during VVC decoding, a general recommendation is to have multiple tiles configured during VVC video encoding and stream creation.

# Bit stream Characteristics

Bit streams used for real time decoding are generated using VTM 7.0 with following configurations

- All random access configuration tools are enabled

- Input and Internal bit depth of 8 bit

- 4 tiles with uniform spacing

# Playback with Software Decoder on Mobile devices

The said VVC software player can be demonstrated running on a mobile device. Pre-encoded bit streams are used for playback on this device. Some salient details have been indicated below.

- Device: Google Pixel 3 (OS: Android 9.0 Pie Processor: ARM Cortex-A75) or One plus 7T (OS: Android 10.0 Processor: ARM Cortex-A76). The performance reported is for the former platform. The latter platform shows even further improved performance.

- This implementation and its demo includes media player and renderer

- This implementation and its demo runs on 4 cores of ARM.

# A demonstration of VVC playback can be shown during the JVET meeting.

# Profiling information for the full-fledged implementation of VVC decoder on ARM

Profiling of software decoder on ARM was performed using GPROF. Please find below the module level profile data for BasketballDrive 1080p and MarketPlace 1080p, both with QP27. The module names and abbreviations have their standard connotations as per VVC Specification.

# Conclusion

Ittiam has implemented the full-fledged version of VVC software decoder on ARM Cortex-A75/76. The results confirm that a real-time implementation of VVC decoding on mobile clients, exercising the complete tools of VTM7.0, is immensely feasible.

# References

- “AHG16:Early Implementation of VVC software player and Demonstration on Mobile device”, JVET-P0307,  JVET 16th meeting, Geneva, CH, 1-11	October 2019

- Frank Bossen, “On software complexity”, JCTVC-G757, JCT-VC 7rd Meeting, Geneva, November, 2010

- Frank Bossen et al, “HEVC Complexity and Implementation Analysis”, IEEE Transactions on Circuits and Systems for Video Technology, Vol. 22, No. 12, December 2012