| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0387-v2 |
| --- | --- |

| Title: | AHG14: Throughput and coding efficiency report of JVET-P0300 on VTM-7.0 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer, Benjamin Bross, Tung Nguyen, Detlev Marpe, Heiko Schwarz, Thomas Wiegand | Tel: Email: | +49 30 31002-184 heiner.kirchhoffer @hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution evaluates the throughput and coding efficiency of the high throughput CABAC mode proposed in JVET-P0300 based on VTM-7.0. The method was implemented into the throughput evaluation testbed of JVET-L1025 (CE5 subtest 2) in order to evaluate the coding engine outside of VTM-7.0. In this testbed, the coding engine of JVET-P0300 increases the throughput by approximately 70 to 100% compared to the VTM-7.0 engine. For three generalized bypass probability values (1/4, 1/8, and 1/16) and QPs 2-17, the BD-rates of the VTM-7.0-based implementation are 4.55% for AI, 5.22% for RA, 4.53% for LB, and 4.66% for LP. The corresponding throughput of the decoder (in mbit per second) is increased by 20% for AI, 18% for RA, 16% for LB, and 18% for LP.

# VTM-7.0-based configuration

The high throughput CABAC mode of JVET-P0300 was implemented into VTM-7.0. The number of generalized bypass shifts (numGB) is set to 3 and context model initialization values have jointly been trained in the same way as described in JVET-P0300 based on the VTM-7.0 bitstreams using common test conditions, the low QP setup (QPs 2, 7, 12, 17), a high QP setup (QPs 42, 47, 52, 57), and the lossless setup. For all bitstreams, the normalized rate d() of JVET-N1021 has been calculated for the VTM-7.0 initialization and entropy coding method as well as for the initialization and entropy coding method of JVET-P0300. Normalized rates of the VTM-7.0 methods have been multiplied with 4 while normalized rates of the proposed method have been multiplied by 1 before proceeding with the procedure of JVET-N1021.

In this way, normalized rates of VTM-7.0 have a 4 times higher importance in the selection process of the initialization values than the ones for the proposed method.

# Throughput evaluation testbed

The high throughput CABAC mode has been implemented into the throughput evaluation testbed (subtest 2) of JVET-L1025, which contains an optimized implementation of the VTM-7.0 and the HEVC CABAC engine. A loop similar to the following was used for decoding and its run time measured:

for (i = 0; i < 50000000; i++)

{

ctx = ctxs[i][lastBin];

lastBin = decodeBin(ctx);

bins[i] = lastBin;

}

The byte sequence to be decoded was generated by a similar encoding loop.

The following configuration was used:

- A sequence of 50.000.000 bins is drawn from a Bernoulli distribution. The following distribution parameters p are tested: 0.00, 0.1, 0.2, …, 1.0.

- For each drawn bin, an associated context model index in the interval [0, 401] is drawn from a uniform distribution. For VTM-7.0, the indexes are stored in array ctxs while for JVET-P0300, the states of context models are stored. This shall reflect the aspect that a context model of JVET-P0300 is represented by a constant state.

The following table lists the measured throughput values using an Intel Xeon W 2155 processor with 64GB or RAM. Simulations were executed using one thread with no other noticeable load on the computer. Hyperthreading and the turbo mode were both enabled.

Each simulation was executed three times and the highest throughput value was picked.

|  | Throughput in mbit/s |  |  | Throughput increase over VTM-7.0 |  |
| --- | --- | --- | --- | --- | --- |
| P | VTM-7.0 | JVET-P0300 | HEVC | JVET-P0300 | HEVC |
| 0 | 125,4 | 222,9 | 152,1 | 77,7% | 21,3% |
| 0,1 | 122,4 | 218,8 | 146,2 | 78,7% | 19,4% |
| 0,2 | 118,5 | 207,7 | 140,7 | 75,3% | 18,7% |
| 0,3 | 116,4 | 201,3 | 137,9 | 72,9% | 18,4% |
| 0,4 | 117,7 | 240,0 | 141,1 | 103,9% | 19,9% |
| 0,5 | 121,6 | 240,0 | 146,3 | 97,3% | 20,3% |
| 0,6 | 117,7 | 239,7 | 140,9 | 103,7% | 19,7% |
| 0,7 | 116,4 | 200,8 | 138,0 | 72,5% | 18,6% |
| 0,8 | 118,4 | 208,0 | 140,5 | 75,6% | 18,6% |
| 0,9 | 122,6 | 219,3 | 146,5 | 78,8% | 19,5% |
| 1 | 128,8 | 235,7 | 157,5 | 83,0% | 22,3% |

As can be seen from the table, the achievable throughput for the JVET-P0300 coding engine is approximately 70-100% higher than for the VTM-7.0 engine.

# Source code

The accompanying source code contains the testbed implementation as well as the VTM-7.0-based implementation of JVET-P0300 (as patch file relative to VTM-7.0).

The testbed is configured using the preprocessor define “ENGINE” in source/App/utils/testbed/testbed.cpp.

In the VTM-7.0-based implementation, the high throughput CABAC mode (JVET-P0300) can be enabled by setting the config file parameter “FastCabac” to 1.

# Experimental results

The coding efficiency has been evaluated using common test conditions, lossless, and the low QP setting. In addition, throughput numbers have been calculated (8 * bitstream size / ( 1,000,000 * decoding time ) ) and are reported in megabits per second. Columns EncTP and DecTP show the relative throughput (test throughput / reference throughput) as percentage.

Encoding times are not very accurate due to heterogenous simulation conditions. Decoding times should be accurate since all bitstreams have been decoded on an isolated computer with no other work load on it (only allowing one thread at a time).

## QP 22-37 (common test conditions)

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 QP22-37 |  |  |  |  |  |  |
|  | Y | U | V | EncTP | DecTP | EncT | DecT |
| Class A1 | 6,40% | 4,25% | 4,51% | 94% | 109% | 110% | 95% |
| Class A2 | 5,26% | 4,08% | 5,46% | 105% | 109% | 97% | 94% |
| Class B | 3,30% | 3,98% | 2,90% | 104% | 109% | 98% | 94% |
| Class C | 2,69% | 3,49% | 3,15% | 111% | 108% | 92% | 94% |
| Class E | 2,89% | 3,97% | 3,21% | 109% | 107% | 95% | 97% |
| Overall | 3,94% | 3,93% | 3,70% | 105% | 108% | 98% | 95% |
| Class D | 2,13% | 2,87% | 2,69% | 111% | 107% | 92% | 96% |
| Class F | 5,92% | 5,84% | 5,75% | 109% | 109% | 96% | 96% |
| Class TGM | 9,68% | 10,07% | 9,98% | 112% | 109% | 97% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over VTM-7.0 QP22-37 |  |  |  |  |  |  |
|  | Y | U | V | EncTP | DecTP | EncT | DecT |
| Class A1 | 7,05% | 7,70% | 7,96% | 115% | 104% | 91% | 100% |
| Class A2 | 4,63% | 3,76% | 5,96% | 109% | 101% | 93% | 100% |
| Class B | 4,76% | 3,80% | 2,93% | 108% | 103% | 95% | 100% |
| Class C | 3,69% | 3,65% | 3,06% | 109% | 104% | 95% | 99% |
| Class E |  |  |  |  |  |  |  |
| Overall | 4,91% | 4,53% | 4,57% | 110% | 103% | 94% | 100% |
| Class D | 3,31% | 2,70% | 2,68% | 108% | 104% | 95% | 99% |
| Class F | 6,34% | 6,53% | 6,27% | 108% | 105% | 98% | 100% |
| Class TGM | 10,26% | 10,43% | 10,68% | 114% | 106% | 95% | 103% |
|  | Low delay B Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 QP22-37 |  |  |  |  |  |  |
|  | Y | U | V | EncTP | DecTP | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 4,42% | 2,20% | -1,04% | 106% | 104% | 95% | 97% |
| Class C | 2,84% | 2,56% | 0,79% | 106% | 103% | 95% | 98% |
| Class E | 3,37% | 3,48% | 0,17% | 104% | 100% | 98% | 102% |
| Overall | 3,63% | 2,64% | -0,13% | 106% | 103% | 96% | 99% |
| Class D | 3,09% | 0,59% | 1,14% | 106% | 103% | 96% | 98% |
| Class F | 6,61% | 7,20% | 6,68% | 110% | 104% | 96% | 102% |
| Class TGM | 10,28% | 10,88% | 10,92% | 113% | 101% | 96% | 107% |
|  | Low delay P Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 QP22-37 |  |  |  |  |  |  |
|  | Y | U | V | EncTP | DecTP | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 4,42% | 2,49% | -1,33% | 106% | 105% | 96% | 96% |
| Class C | 2,92% | 2,64% | 0,48% | 105% | 104% | 97% | 98% |
| Class E | 3,77% | 2,86% | 0,32% | 106% | 102% | 96% | 100% |
| Overall | 3,75% | 2,63% | -0,32% | 106% | 104% | 96% | 98% |
| Class D | 3,14% | 0,11% | 0,64% | 107% | 104% | 95% | 98% |
| Class F | 6,65% | 7,44% | 6,67% | 108% | 104% | 97% | 102% |
| Class TGM | 10,19% | 10,92% | 11,06% | 115% | 102% | 94% | 106% |

## Lossless setting (100 frames)

|  | All Intra |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | ratio |  |  |  | bit-rate savings |  |
|  |  | VTM-7.0 |  | JVET-P0300 |  |  |  |
| Class A1 |  | 2,2 |  | 2,1 |  | 5,52% |  |
| Class A2 |  | 1,6 |  | 1,5 |  | 5,71% |  |
| Class B |  | 2,2 |  | 2,1 |  | 5,59% |  |
| Class C |  | 1,9 |  | 1,8 |  | 5,51% |  |
| Class D |  | 1,9 |  | 1,8 |  | 5,23% |  |
| Class E |  | 2,8 |  | 2,5 |  | 10,51% |  |
| Class F |  | 5,3 |  | 4,9 |  | 6,87% |  |
| TGM |  | 11,8 |  | 11,0 |  | 6,92% |  |
| Overall |  | 2,1 |  | 2,0 |  | 6,40% |  |
| Dec Throughput [%] |  | 115% |  |  |  |  |  |
| Enc Time[%] |  | 93% |  |  |  |  |  |
| Dec Time[%] |  | 92% |  |  |  |  |  |

|  | Random Access |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-7.0 | JVET-P0300 |  |
| Class A1 | 2,2 | 2,1 | 7,69% |
| Class A2 | 1,7 | 1,6 | 8,22% |
| Class B | 2,3 | 2,2 | 6,44% |
| Class C | 2,4 | 2,3 | 5,52% |
| Class D | 2,8 | 2,6 | 6,46% |
| Class E |  |  |  |
| Class F | 33,7 | 30,1 | 9,37% |
| TGM | 107,1 | 96,1 | 10,42% |
| Overall | 2,2 | 2,1 | 6,80% |
| Dec Throughput [%] | 117% |  |  |
| Enc Time[%] | 97% |  |  |
| Dec Time[%] | 91% |  |  |

|  | Low delay B |  |  |
| --- | --- | --- | --- |
|  | ratio |  | bit-rate savings |
|  | VTM-7.0 | JVET-P0300 |  |
| Class A1 |  |  |  |
| Class A2 |  |  |  |
| Class B | 2,3 | 2,2 | 6,61% |
| Class C | 2,4 | 2,3 | 5,48% |
| Class D | 2,7 | 2,6 | 6,38% |
| Class E | 3,1 | 3,0 | 5,59% |
| Class F | 50,7 | 43,0 | 11,01% |
| TGM | 124,9 | 109,3 | 12,62% |
| Overall | 2,6 | 2,4 | 5,98% |
| Dec Throughput [%] | 116% |  |  |
| Enc Time[%] | 95% |  |  |
| Dec Time[%] | 90% |  |  |

## QP 2-17 (low QP setting, 100 frames)

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 QP2-17 |  |  |  |  |  |  |
|  | Y | U | V | EncTP | DecTP | EncT | DecT |
| Class A1 | 5,84% | 4,46% | 4,97% | 100% | 122% | 104% | 85% |
| Class A2 | 4,71% | 4,60% | 4,66% | 113% | 124% | 92% | 84% |
| Class B | 5,27% | 5,30% | 5,95% | 107% | 119% | 98% | 88% |
| Class C | 3,50% | 3,26% | 3,45% | 108% | 116% | 95% | 89% |
| Class E | 3,31% | 3,98% | 4,76% | 109% | 121% | 95% | 85% |
| Overall | 4,55% | 4,37% | 4,82% | 107% | 120% | 97% | 86% |
| Class D | 3,11% | 2,72% | 2,95% | 101% | 116% | 102% | 89% |
| Class F | 5,45% | 4,14% | 4,37% | 105% | 114% | 100% | 91% |
| Class TGM | 6,39% | 6,21% | 6,37% | 106% | 108% | 101% | 98% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over VTM-7.0 QP2-17 |  |  |  |  |  |  |
|  | Y | U | V | EncTP | DecTP | EncT | DecT |
| Class A1 | 5,83% | 2,92% | 3,53% | 101% | 125% | 102% | 83% |
| Class A2 | 5,09% | 4,15% | 4,46% | 107% | 121% | 96% | 84% |
| Class B | 5,85% | 3,84% | 1,97% | 98% | 118% | 105% | 88% |
| Class C | 4,07% | 4,43% | 3,16% | 110% | 112% | 93% | 91% |
| Class E |  |  |  |  |  |  |  |
| Overall | 5,22% | 3,87% | 3,10% | 104% | 118% | 99% | 87% |
| Class D | 3,45% | 3,28% | 2,53% | 114% | 111% | 90% | 93% |
| Class F | 6,97% | 5,22% | 4,66% | 107% | 105% | 97% | 99% |
| Class TGM | 8,24% | 7,43% | 7,54% | 107% | 100% | 101% | 108% |
|  | Low delay B Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 QP2-17 |  |  |  |  |  |  |
|  | Y | U | V | EncTP | DecTP | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 5,37% | 5,50% | 5,35% | 100% | 118% | 104% | 88% |
| Class C | 3,73% | 4,47% | 3,17% | 107% | 112% | 96% | 91% |
| Class E | 4,19% | 3,71% | 3,35% | 107% | 119% | 95% | 85% |
| Overall | 4,53% | 4,71% | 4,12% | 104% | 116% | 99% | 88% |
| Class D | 3,03% | 2,88% | 2,71% | 109% | 111% | 94% | 92% |
| Class F | 5,91% | 5,69% | 4,85% | 109% | 108% | 95% | 95% |
| Class TGM | 7,53% | 7,40% | 7,51% | 112% | 103% | 96% | 104% |
|  | Low delay P Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 QP2-17 |  |  |  |  |  |  |
|  | Y | U | V | EncTP | DecTP | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 5,38% | 5,16% | 5,04% | 103% | 118% | 100% | 88% |
| Class C | 4,02% | 3,96% | 2,73% | 106% | 116% | 96% | 88% |
| Class E | 4,34% | 2,62% | 2,91% | 106% | 122% | 96% | 84% |
| Overall | 4,66% | 4,13% | 3,74% | 105% | 118% | 98% | 87% |
| Class D | 3,22% | 2,57% | 2,64% | 105% | 113% | 98% | 91% |
| Class F | 5,90% | 5,35% | 4,60% | 110% | 110% | 94% | 94% |
| Class TGM | 7,34% | 7,25% | 7,40% | 111% | 103% | 97% | 104% |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).