| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0388-v4 |
| --- | --- |

| Title: | CE4-related: GEO memory reduction in weighting masks by fixed shifting |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu, Christopher Hollmann, Ruoyang Yu, Jacob Ström Torshamnsgatan 23 16480 Stockholm Sweden | Email: | {du.liu, christopher.hollmann, ruoyang.yu, jacob.strom} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

Geometric partitioning (GEO) allows a block to be partitioned into two parts, where the prediction is calculated from a weighted sum of the two parts. If GEO is not calculated on the fly, weighting masks are needed. Currently, the largest block size allowed in GEO is 128×128. The weighting masks have a size of 224×224 such that the blocks can be shifted in both vertical and horizontal directions to obtain the weights.

This contribution proposes to specify the shift direction for each angle, such that only one shifting direction is allowed for each mask. It is reported that the mask size then can be reduced to 224×128 or 128×224. It is further asserted that the total memory requirement for all weighting masks then can be reduced by 35%. It is claimed that the same memory reduction percentage (-35%) holds even if the largest block size is limited to 64×64.

The proposed method was reportedly tested under VTM-7.0 CE4 common base and random access (RA) and low delay B (LDB) common test configuration. The contribution includes three tests. Memory reduction and the BD-rate impact are provided:

Test 1: Specifying the shift direction for each angle:

weighting mask memory reduction -35%, Y-BDR -0.02% (RA), 0.00% (LDB).

Test 2: Test 1 on top of CE4-1 (64 modes):

weighting mask memory reduction -44%, Y-BDR -0.01% (RA), 0.02% (LDB).

Test 3: Test 1 plus further reduction in the number of modes (54 modes),

weighting mask memory reduction -53%, Y-BDR 0.02% (RA), 0.06% (LDB).

# Introduction

Geometric partitioning (GEO), as described in JVET-P0884 [1], is a partitioning mode that allows a block to be partitioned into two parts, where the partition is described by different angles and distances. There are 24 angles defined in GEO. The angles with absolute tangent values {0, 1/4, 1/2, 1, 2, 4, ∞} are used. The angle is indexed by angleIdx with indices 0 to 23. AngleIdx with values 0 and 12 represent the vertical line and angleIdx with values 6 and 18 the horizontal line.

In addition, there are three or four distances defined and they are indexed by distanceIdx. Horizontal and vertical angles have only three distances with distanceIdx 1 to 3, and other angles have four distances with distanceIdx 0 to 3. The distanceIdx with value 0 indicates that the partition line goes through the center of the block. The distanceIdx with value 3 indicates that the partition line is the furthest away from the center.

The partition lines are evenly spaced for a given angleIdx, as shown in Figure 1. For square blocks the spacing is the same in either direction. However, when width and height are not equal, the spacing is different in vertical and horizontal directions.

(a) Partition in vertical direction,

angleIdx = 0, distanceIdx = 1, 2, 3.

(b) Partition in diagonal direction,

angleIdx = 3, distanceIdx = 0, 1, 2, 3.

Figure 1. Examples of partition lines in vertical direction and in diagonal direction.

Let a block have width nCbW and height nCbH. Then, the spacing in horizontal direction is nCbW/8 and in vertical direction nCbH/8. Therefore, for a given angleIdx, the splitting line with distanceIdx = 1, 2, and 3 can be obtained by shifting the splitting line with distanceIdx = 0 in horizontal or vertical direction by nCbW/8 * distanceIdx or nCbH/8 * distanceIdx. The shifting direction is based on the width and height of the block and the angleIdx.

If the GEO partitioning is not calculated on the fly, it is possible to compute a weighting mask that is big enough to cover a shift with distanceIdx = 3. Shifting the partition line inside the block is equivalent to shifting the block inside the weighting mask to obtain weights. In GEO, seven weighting masks are used to define the partition directions with angleIdx 0 to 6. The masks for the remaining angles can be derived. An overview of which angle uses which mask is shown in Table 1.

Table 1. Mapping between angles and weighting masks

| Angle | maskIdx |
| --- | --- |
| 0, 12 | 0 |
| 1, 13 | 1 |
| 2, 14 | 2 |
| 3, 15 | 3 |
| 4, 16 | 4 |
| 5, 17 | 5 |
| 6, 18 | 6 |
| 7, 19 | 5 |
| 8, 20 | 4 |
| 9, 21 | 3 |
| 10, 22 | 2 |
| 11, 23 | 1 |

The size of the weighting mask is determined by the largest block that is allowed in GEO. Currently, the largest block is 128x128. The largest shifting in one direction is 128/8 * 3 = 48 (with distanceIdx = 3), and it needs to be shifted in four directions, left, right, up, and down. Thus, the weighting mask needs to have a size of (128 + 48 + 48) x (128 + 48 + 48) = 224×224. Figure 2 depicts an example of the weighting mask with size 224×224, where black represents a weight of zero and white a weight of eight. The block can be shifted in vertical or horizontal directions.

Figure 2. An example of weighting mask with size 224×224 and shifting of the block in vertical or horizontal directions.

The variable shiftHor determines where the shift is in horizontal direction. If shiftHor = 0, the shift is in vertical direction. Otherwise, the shift is in horizontal direction. In the Spec given in JVET-P0884 [1], it is specified as

If one of the following conditons is true, variable shiftHor is set equal to 0: angleIdx % 12 is equal to 6 (1) angleIdx % 12 is not equal to 0 and hwRatio ≥ 1 (2) Otherwise, shiftHor is set equal to 1.

The hwRatio is the ratio between height and width, i.e., hwRatio = nCbH/nCbW.

Based on these conditions, the shift directions for different angles and block shapes are given in Table 2.

Table 2. Shifting direction as described in [1]

| Angle | nCbH ≥ nCbW | nCbH < nCbW |
| --- | --- | --- |
| 0, 12 | 1 | 1 |
| 1, 13 | 0 | 1 |
| 2, 14 | 0 | 1 |
| 3, 15 | 0 | 1 |
| 4, 16 | 0 | 1 |
| 5, 17 | 0 | 1 |
| 6, 18 | 0 | 0 |
| 7, 19 | 0 | 1 |
| 8, 20 | 0 | 1 |
| 9, 21 | 0 | 1 |
| 10, 22 | 0 | 1 |
| 11, 23 | 0 | 1 |

As shown in Table 2, the shift is always in horizontal direction for vertical angles and the shift is always in vertical direction for horizontal angles. For non-horizontal or non-vertical angles, the shift can be in both directions depending on the block size.

In hardware implementation, the memory required for the weighting mask is different for different masks. For vertical angles (angleIdx 0 and 12), the shift is only in the horizontal direction and the required mask size is 224×128 samples. Similarly, for horizontal angles (angleIdx 6 and 18), the shift is only in the vertical direction and the required mask size is 128×224. For the other five masks, the size must be a full 224×224 samples. Thus, the total memory requirement for the seven masks is 224 * 128 * 2 + 224 * 224 * 5 = 308224 samples.

Another issue with the shift direction is that the spacing might be inappropriate for certain angles. For example, for square blocks with angleIdx 1 and 13, the shift is always in vertical direction. The partitions for different distanceIdx are depicted in the left picture of Figure 3. As is shown, the partition locations are close to each other. However, if the shift direction is changed to horizontal, the partition is more evenly placed inside the block.

Figure 3. Partitions for angleIdx = 1 and 13 and square blocks. Left: Shift in vertical direction. Right: Shift in horizontal direction.

# Proposal – Fixed shift direction

This method proposes to fix the shift direction for each angle. The decision of the shifting direction is according to

The variable shiftHor is calculated as follows:

shiftHor = ((angleIdx % 12 >= 3) && (angleIdx % 12 <= 9)) ? 0 : 1					(1)

The values of shiftHor for difference angles are given in Table 3, where 1 indicates shifting in horizontal direction and 0 in vertical direction.

Table 3. Shifting direction for fixed shifting.

| Angle | Direction |
| --- | --- |
| 0, 12 | 1 |
| 1, 13 | 1 |
| 2, 14 | 1 |
| 3, 15 | 0 |
| 4, 16 | 0 |
| 5, 17 | 0 |
| 6, 18 | 0 |
| 7, 19 | 0 |
| 8, 20 | 0 |
| 9, 21 | 0 |
| 10, 22 | 1 |
| 11, 23 | 1 |

The method uses fixed shift directions for all the seven masks. Fixing the shifting direction has the advantage that the weighting mask size can be reduced, since there is no need to allow a shift in both directions. For vertical shifting (Direction=0 in Table 3) a mask of 128×224 samples is needed whereas for horizontal shifting (Direction=1 in Table 3), a mask of 224×128 samples is needed. The total required memory of this method is 224 * 128 * 7 = 200704. This reduces the memory to 200704 / 308224 ≈ 65%, a reduction of 35%.

# Simulation results for fixed shifting (Test 1)

The proposed method was implemented on top of the common base used in CE4 [2], which provides a basic implementation of GEO. The simulations were run using the VTM common test configurations [3]. Note that encoder and decoder run times are inaccurate due to different cluster loads, but it is claimed that there should be no difference in run times compared to the common base.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 Common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.01% | 0.01% | 99% | 102% |
| Class A2 | 0.01% | 0.08% | -0.07% | 99% | 101% |
| Class B | -0.01% | 0.00% | 0.02% | 97% | 100% |
| Class C | -0.07% | 0.06% | -0.05% | 98% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.03% | -0.02% | 98% | 101% |
| Class D | -0.02% | -0.18% | -0.06% | 97% | 100% |
| Class F | 0.01% | 0.03% | 0.04% | 98% | 101% |
|  | Low delay B |  |  |  |  |
|  | Over VTM-7.0 Common base |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.04% | 0.14% | 98% | 101% |
| Class C | -0.04% | -0.13% | -0.05% | 94% | 98% |
| Class E | 0.09% | 0.18% | -0.16% | 97% | 100% |
| Overall | 0.00% | -0.02% | 0.00% | 96% | 99% |
| Class D | -0.06% | 0.11% | 0.39% | 97% | 102% |
| Class F | -0.07% | -0.40% | -0.27% | 97% | 103% |

# Additional simulations

## Combination with CE4-1 (Test 2)

The proposed method is tested on top of CE4-1 [2] [4], giving the following results. One of the weighting masks is completely removed, reducing the memory requirements further. The required memory is 224 * 128 * 6 = 172032, which reduces the memory to 172032 / 308224 ≈ 56%, a reduction by 44%.

|  | Random access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 Common base |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |  |
| Class A1 | -0.01% |  | -0.06% |  | 0.03% | 104% | 99% |  |
| Class A2 | 0.03% |  | 0.11% |  | -0.07% | 103% | 101% |  |
| Class B | 0.01% |  | -0.03% |  | 0.08% | 98% | 102% |  |
| Class C | -0.05% |  | 0.02% |  | -0.09% | 98% | 102% |  |
| Class E |  |  |  |  |  |  |  |  |
| Overall | -0.01% |  | 0.01% |  | 0.00% | 100% | 101% |  |
| Class D | -0.06% |  | -0.09% |  | -0.02% | 97% | 101% |  |
| Class F | 0.04% |  | 0.04% |  | 0.05% | 98% | 100% |  |
|  | Low delay B |  |  |  |  |  |  |  |
|  | Over VTM-7.0 Common base |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT | DecT |  |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0.03% |  | 0.03% |  | 0.05% | 101% | 101% |  |
| Class C | 0.00% |  | -0.05% |  | -0.05% | 101% | 101% |  |
| Class E | 0.05% |  | 0.53% |  | 0.04% | 99% | 102% |  |
| Overall | 0.02% |  | 0.13% |  | 0.01% | 100% | 101% |  |
| Class D | -0.13% |  | 0.03% |  | 0.23% | 98% | 101% |  |
| Class F | -0.04% |  | 0.07% |  | -0.17% | 103% | 103% |  |

## Mode reduction (Test 3)

In another test, we reduced the number of total GEO modes to 54. This was done by removing the angles with absolute tangents ¼ and 4. This change allows to remove two of the seven weighting masks, as all angles using weighting masks 1 and 5 are removed. The required memory is 224 * 128 * 5 = 143360, which reduces the memory to 143360 / 308224 ≈ 47%, a reduction by 53%.

|  | Random Access |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over GEO common base |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |
| Class A1 | 0.02% |  | -0.11% |  | 0.02% | 94% |  | 102% |
| Class A2 | 0.04% |  | 0.07% |  | 0.01% | 94% |  | 99% |
| Class B | 0.00% |  | -0.05% |  | 0.05% | 95% |  | 99% |
| Class C | 0.04% |  | 0.24% |  | 0.15% | 95% |  | 102% |
| Class E |  |  |  |  |  |  |  |  |
| Overall | 0.02% |  | 0.04% |  | 0.06% | 95% |  | 101% |
| Class D | 0.00% |  | -0.10% |  | 0.03% | 95% |  | 102% |
| Class F | 0.00% |  | 0.01% |  | 0.00% | 97% |  | 99% |
|  | Low delay B |  |  |  |  |  |  |  |
|  | Over GEO common base |  |  |  |  |  |  |  |
|  | Y |  | U |  | V | EncT |  | DecT |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0.01% |  | -0.03% |  | 0.20% | 92% |  | 102% |
| Class C | 0.06% |  | 0.05% |  | 0.02% | 90% |  | 101% |
| Class E | 0.16% |  | -0.13% |  | 0.21% | 92% |  | 101% |
| Overall | 0.06% |  | -0.03% |  | 0.14% | 91% |  | 101% |
| Class D | -0.04% |  | -0.69% |  | 0.17% | 93% |  | 99% |
| Class F | -0.05% |  | 0.29% |  | -0.25% | 93% |  | 97% |

Version 2 of this document provides more accurate decoder run times.

Versions 3 and 4 of this document correct a bug in Test 3 and updates the results of Test 3.

# Conclusions

This contribution proposes to specify the shift direction for each angle, such that only one shift direction is allowed for each weighting mask. The changes to the memory requirement for the weighting mask and BD-rate for the different combinations are shown in Table 4. The max size represents the largest block size allows in GEO. The memory reduction stays the same regardless of the largest block size.

Table 4. Summary of BD-rate results and memory savings

| Test | Random Access | Low Delay B | Mask memory (max size 128) | Mask memory (max size 64) | Mask memory reduction |
| --- | --- | --- | --- | --- | --- |
| Common base | 0.00% | 0.00% | 100% (301k) | 100% (⁓75k) | 0% |
| Fixed shifting | -0.02% | 0.00% | 65% (196k) | 65% (49k) | -35% |
| Fixed shifting + CE4-1 | -0.01% | 0.02% | 56% (168k) | 56% (42k) | -44% |
| Fixed shifting + 54 modes | 0.02% | 0.06% | 47% (140k) | 47% (35k) | -53% |

We recommend adopting the proposed method into the next VTM software and VVC working draft.

# References

- H. Gao, S. Esenlik, E. Alshina, A. M. Kotra, B. Wang, M. Blaser, J. Sauer, "Simplified GEO without multiplication and minimum blending mask storage (harmonization of JVET-P0107, JVET-P0264 and JVET-P0304)", Input document to JVET, document number JVET-P0884-v8, Geneva, Oct. 2019.

- C.-C. Chen et al. Description of Core Experiment 4 (CE4): Inter prediction with geometric partitioning. JVET-P2024, Geneva, Switzerland, October 2019

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring. JVET common test conditions and software reference configurations for SDR video. JVET-N1010, Geneva, Switzerland, March 2019.

- H. Gao et al. CE4-1: Geometric Inter Prediction with 64 Modes. JVET-Q0059, Brussels, Belgium, January 2020

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).