| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0392-v2 |
| --- | --- |

| Title: | On the coding of cclm_mode_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Geert Van der Auwera, A. K. Ramasubramonian, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

It is proposed to have separate context for cclm_mode_flag when the co-located luma block is coded with MIP. Results on VTM-7.0, for CTC (dual-tree on) and dual-tree off cases are reported below. In version v2, the source code is attached.

Table 1: CTC results

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | -0.01% | -0.01% | 0.03% | 100% | 101% |
| RA | -0.02% | 0.00% | 0.06% | 99% | 99% |

Table 2: dual-tree off results

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | -0.06% | -0.13% | -0.05% | 100% | 102% |
| RA | -0.03% | -0.01% | 0.11% | 99% | 100% |

# Introduction

The chroma intra prediction mode coding syntax and relevant aspects of the specification of intra chroma prediction mode from VVC Draft 7 [1] are shown below. The syntax element cclm_mode_flag indicates whether the chroma prediction mode is a CCLM mode (INTRA_LT_CCLM, INTRA_L_CCLM, or INTRA_T_CCLM). Currently, a dedicated context is used for coding cclm_mode_flag.

| if( CclmEnabled ) |  |
| --- | --- |
| cclm_mode_flag | ae(v) |
| if( cclm_mode_flag ) |  |
| cclm_mode_idx | ae(v) |
| else |  |
| intra_chroma_pred_mode | ae(v) |

Table 1– Specification of IntraPredModeC[ xCb ][ yCb ] depending on cclm_mode_flag, cclm_mode_idx, intra_chroma_pred_mode and lumaIntraPredMode

| cclm_mode_flag | cclm_mode_idx | intra_chroma_pred_mode | lumaIntraPredMode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 0 | 50 | 18 | 1 | X ( 0 <= X <= 66 ) |
| 0 | - | 0 | 66 | 0 | 0 | 0 | 0 |
| 0 | - | 1 | 50 | 66 | 50 | 50 | 50 |
| 0 | - | 2 | 18 | 18 | 66 | 18 | 18 |
| 0 | - | 3 | 1 | 1 | 1 | 66 | 1 |
| 0 | - | 4 | 0 | 50 | 18 | 1 | X |
| 1 | 0 | - | 81 | 81 | 81 | 81 | 81 |
| 1 | 1 | - | 82 | 82 | 82 | 82 | 82 |
| 1 | 2 | - | 83 | 83 | 83 | 83 | 83 |

# Proposal

It is typically observed that MIP prediction is helpful for blocks having complex (non-directional) pattern, whereas regular intra prediction is efficient for blocks for having directional pattern. When the co-located luma block is MIP-coded, lumaIntraPredMode = 0 (i.e., Planar mode) is used for coding the corresponding chroma blocks. However, chroma blocks often show complex (non-directional) pattern similar to luma, where CCLM modes may be more effective than regular intra prediction modes. So, it is proposed to have a separate context for cclm_mode_flag when the co-located block is MIP-coded. With the inclusion of the new context, the statistics for cclm_mode_flag, intra_chroma_pred_mode and cclm_mode_idx are changed; so, the corresponding contexts are retrained.

The following change (highlighted) is proposed for context derivation on top of JVET-P2001-vE.

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| cclm_mode_flag | intra_mip_flag[xCb+cbWidth/2 ][yCb+cbHeight/2 ]? 1 : 0 | na | na | na | na | na |

# Experimental results

The proposed method is implemented on top of VTM-7.0 and tested with CTC configuration (dual-tree on) [2] and dual-tree off.

Table 3: CTC results

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.10% | 0.15% | 100% | 101% |
| Class A2 | -0.01% | 0.01% | -0.08% | 99% | 98% |
| Class B | 0.00% | 0.04% | -0.03% | 100% | 99% |
| Class C | 0.00% | 0.02% | 0.03% | 99% | 102% |
| Class E | -0.01% | -0.08% | 0.09% | 99% | 100% |
| Overall | -0.01% | -0.01% | 0.03% | 100% | 100% |
| Class D | 0.01% | 0.12% | -0.01% | 99% | 98% |
| Class F | -0.01% | 0.05% | -0.09% | 99% | 101% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access |  |  |  |  |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.20% | 0.11% | 100% | 102% |
| Class A2 | -0.05% | 0.09% | 0.00% | 97% | 98% |
| Class B | -0.03% | 0.03% | 0.10% | 99% | 100% |
| Class C | -0.01% | 0.03% | 0.01% | 100% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | 0.06% | 99% | 99% |
| Class D | -0.01% | -0.13% | -0.17% | 100% | 100% |
| Class F | -0.03% | 0.05% | -0.11% | 98% | 98% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 4: Results (dual tree off )

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | 0.04% | 0.30% | 100% | 100% |
| Class A2 | -0.05% | -0.11% | -0.11% | 101% | 102% |
| Class B | -0.07% | -0.13% | 0.04% | 99% | 98% |
| Class C | -0.04% | -0.09% | -0.06% | 100% | 105% |
| Class E | -0.03% | -0.38% | -0.48% | 100% | 107% |
| Overall | -0.06% | -0.13% | -0.05% | 100% | 102% |
| Class D | -0.01% | 0.15% | 0.04% | 99% | 94% |
| Class F | -0.02% | -0.03% | -0.13% | 99% | 101% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
|  | Random Access |  |  |  |  |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.11% | 0.13% | 99% | 100% |
| Class A2 | -0.05% | -0.07% | -0.07% | 101% | 100% |
| Class B | -0.01% | -0.05% | 0.16% | 100% | 100% |
| Class C | -0.03% | 0.02% | 0.15% | 99% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.01% | 0.11% | 99% | 100% |
| Class D | -0.01% | 0.25% | 0.26% | 106% | 105% |
| Class F | -0.04% | 0.05% | -0.05% | 99% | 99% |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

From the results, it can be observed that the proposed method works better for dual tree off case. Under dual tree conditions, the partitioning of luma and chroma blocks are different. So, the co-located luma and chroma blocks do not generally correspond to the same image area, and thus the assumption of similarity between luma and chroma blocks is weakened. The encoding and decoding runtimes are similar to that of VTM-7.0.

# Conclusion

The proposal adds a separate context for cclm_mode_flag when the co-located block is MIP-coded. The results are comparatively better for dual-tree off case.

# References

[1] Bross et al., “Versatile Video Coding (Draft 7),” Joint Video Experts Team, doc. JVET-P2001, Geneva, CH, Oct. 2019.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).