| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0396 |
| --- | --- |

| Title: | AHG9: On mixing NAL unit types in a coded picture |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes a set of rules for NAL unit types that are allowed to be mixed within a coded picture.

It is asserted that the current draft specification only considers mixing IRAP types and TRAIL_NUT up to RSV_VCL_6 or GDR_NUT which is not sufficient for envisioned applications.

Compared to the current draft specification, this proposal allows TRAIL_NUT and STSA_NUT NAL unit types to be mixed with NAL unit types such as RADL_NUT and RASL_NUT.

It also proposes to allow mixing CRA_NUT and IDR_W_RADL NAL units, as well as RADL_NUT and RASL_NUT NAL units.

# Introduction

During the last JVET meeting a solution was accepted for allowing mixing different NAL unit types within coded pictures to fulfil requirements that come from use-cases related to subpicture-based streaming scenarios as for 360° video streaming. In those scenarios, different subpicture bitstreams are merged within a single bitstream depending on the viewing orientation of the user. Allowing that the IRAP properties of the respective subpicture NAL units in a picture do not align, reduces the average bitrate to be transmitted as well as bitrate spikes.

The current solution in the VVC draft specification consists of a PPS flag (mixed_nalu_types_in_pic_flag) that indicates whether all the VCL NAL units of a picture are of the same NAL unit type or whether they are a mixture of single one type of TRAIL up to RSV_VCL_6 or GDR_NUT with another single one type of IDR_W_LP, IDR_N_LP and CRA.

However, these measures do not suffice for the envisioned application where open GOP structures may be applied introducing leading pictures, which would be mixed with NAL unit types that are not of the same type.

Leading pictures

In the following figures Figure 1 to Figure 2, cases are shown in which each of the two RAP types (IDR_W_RADL and CRA) is present in the merged bitstream, for which associated leading pictures are present in the bitstream. In these figures, it is shown how as a result of a mix between IRAP types with TRAIL_NUTs within a picture, in case of leading pictures being present in the bitstream, other pictures end up having also mixed NAL unit types.

Figure 1: IDR_W_LP: There is also RADL NAL units mixed with TRAIL NAL units

As can be seen in Figure 1, when there are associated leading pictures of type RADL_NUT, both the IRAP NAL unit types as well as RADL_NUT are mixed with TRAIL_NUT NAL units.

Figure 2: CRA: There are RADL and RASL NAL units mixed with TRAIL NAL units (figure shows the RASL case)

As can be seen in Figure 2, when there are associated leading pictures of type RASL_NUT, both the IRAP NAL unit types as well as RASL_NUT are mixed with TRAIL_NUT NAL units.

Similarly, not only TRAIL_NUT might be present in the bitstream, but also STSA_NUTs. Such a case is shown in Figure 3.

Figure 3: Exemplary coding structure where in one tile (top example), a picture uses a STSA NAL unit type to indicate step-wise temporal sublayer access, while the same picture in another tiles (bottom example) uses a RASL NAL unit type.

In such a case, also STSA_NUTs might be mixed with RASL_NUTs or RADL_NUTs. In Table 2-1, all applicable combinations of NAL unit types that can be mixed are listed.

Open vs. closed GOP

Another important case that is not supported in the current draft specification is allowing mixing two different types of IRAP NAL unit type, namely IDR_W_LP and CRA NAL unit types. Allowing such a mixture would be beneficial in the 360° video streaming case where the whole low-resolution video content is transmitted irrespective of whether portions thereof are also transmitted as high-resolution tiles. In such a case, a change in the viewing orientation and a corresponding change of the high-resolution tile selection from one viewport to another viewport is illustrated in the following Figure 4.

Figure 4: Two exemplary viewports with full 360-degree low resolution thumbnail with selected high-resolution tiles.

Since the entire low-resolution video content is always available at the client-side, and no tile-wise stream switching is required for this region, the low-resolution content could be encoded with CRA_NUTs and thus make use of a higher coding efficiency compared to a closed GOP structure using IDRs. Also, a longer IRAP period than necessary for stream switching would suffice for this area leading to coding efficiency benefits.

On the other hand, the high-resolution tiles, might frequently change in fast progression from one viewport to another as indicated in the Figure 4 and therefore, using CRA_NUTs and associated RASL pictures as for the low-resolution video would not allow a smooth switch of the viewport. This is because of the RASL areas within pictures that change position or represent newly added tile bitstreams are not able to be decoded correctly due to incorrect sections in the reference pictures.

Therefore, it would be beneficial to allow CRA_NUT in the region corresponding to the low-resolution content to be mixed also with IDRs in regions corresponding to high-resolution content.

The above scenario is based on mixing CRA_NUTs with IDR_W_RADLs that are both within corresponding closed/open GOP structures and not to mixing CRA_NUTs with IDR_N_LP. The latter combination usually represents different GOP structures, i.e. the decoding order of picture and output order of both (CRAs and IDR_N_LP) is not aligned.

# Proposal

The proposal here is to allow the following NAL unit type combinations in VVC bitstreams (new ones highlighted with respect to current VVC draft specification).

Table 2-1: applicable NAL unit types combinations.

| NAL unit type | to be mixed with |
| --- | --- |
| CRA_NUT | TRAIL_NUT…RSV_VCL_6 + GDR + IDR_W_RADL |
| IDR_N_LP | TRAIL_NUT…RSV_VCL_6 + GDR_NUT |
| IDR_W_RADL | TRAIL_NUT…RSV_VCL_6 + GDR + CRA_NUT |
| TRAIL_NUT | CRA_NUT + IDR_W_RADL + IDR_N_LP + RADL_NUT + RASL_NUT |
| STSA_NUT | CRA_NUT + IDR_W_RADL + IDR_N_LP + RADL_NUT + RASL_NUT |
| RASL_NUT | CRA_NUT + IDR_W_RADL + IDR_N_LP + TRAIL_NUT + RADL_NUT |
| RADL_NUT | CRA_NUT + IDR_W_RADL + IDR_N_LP + TRAIL_NUT + RASL_NUT |

Currently, the VVC draft specification includes two flags related to mixing NAL unit types within a coded picture, namely mixed_nalu_types_in_pic_flag in the PPS and no_mixed_nalu_types_in_pic_constraint_flag in general_constraint_info( ).

Although, the new combinations of non-IRAP types, i.e. TRAIL+RAD/SL or STSA + RAD/SL or RADL + RASL, do not have a particular decoding process associated with them, MANEs and other devices operating on the bitstream may benefit from an indication of how to interpret mixed non-IRAP access units.

Mixed AUs of IRAP NAL unit types, e.g. CRA_NUT + IDR_W_RADL, care has to be taken to ensure identifying the IRAP property and progress accordingly on decoder side when necessary. With the current VVC draft solution the IRAP property of a mixed picture that is not the first picture in a bitstream can be ignored without affecting the decoder result. However, not treating a mixed picture with IRAP types CRA_NUT + IDR_W_RADL that follows a NAL unit of type EOS_NUT or is the first picture in the bitstream as an IRAP would lead to a undesired situation. The proposed change is to treat such pictures with NAL units of types CRA_NUT + IDR_W_RADL as a CRA picture.

# Specification changes

The proposed specification changes to the PPS are shown in the following:

no_mixed_nalu_types_in_pic_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that mixed_nalu_types_in_pic_flagidc shall be equal to 0. no_mixed_nalu_types_in_pic_constraint_flag equal to 0 does not impose such a constraint.

mixed_nalu_types_in_pic_flagidc equal to 0 specifies that each picture referring to the PPS has one or more VCL NAL units and the VCL NAL units of each picture refering to the PPS have the same value of nal_unit_type.

mixed_nalu_types_in_pic_idc equal to 1 specifies that each picture referring to the PPS has more than one VCL NAL unit and that the VCL NAL units shall all have two particular values of nal_unit_type in the range of TRAIL_NUT to RSV_VCL_6, inclusive.

mixed_nalu_types_in_pic_idc equal to 2 specifies that each picture referring to the PPS has more than one VCL NAL unit and one or more of the VCL NAL units shall all have a particular value of nal_unit_type in the range of IDR_W_RADL to CRA_NUT, inclusive, and the other VCL NAL units shall all have a particular value of nal_unit_type in the range of TRAIL_NUT to RSV_VCL_6, inclusive, or equal to GRA_NUT.

mixed_nalu_types_in_pic_idc equal to 3 specifies that each picture referring to the PPS has more than one VCL NAL unit with the value of nal_unit_type IDR_W_RADL and one or more VCL NAL unit with the value of nal_unit_type CRA_NUT .

When no_mixed_nalu_types_in_pic_constraint_flag is equal to 1, the value of mixed_nalu_types_in_pic_idcflag shall be equal to 0.

For each slice with a nal_unit_type value nalUnitTypeA in the range of IDR_W_RADL to CRA_NUT, inclusive, in a picture picA that also contains one or more slices with another value of nal_unit_type (i.e., the value of mixed_nalu_types_in_pic_idcflag for the picture picA is equal to 1), the following applies:

–	The slice shall belong to a subpicture subpicA for which the value of the corresponding subpic_treated_as_pic_flag[ i ] is equal to 1.

–	The slice shall not belong to a subpicture of picA containing VCL NAL units with nal_unit_type not equal to nalUnitTypeA.

–	For all the following PUs in the CLVS in decoding output order, neither RefPicList[ 0 ] nor RefPicList[ 1 ] of a slice in subpicA shall include any picture preceding picA in decoding order in an active entry.

The proposed specification changes to the nal_unit_type are shown in the following:

For VCL NAL units of any particular picture, the following applies:

–	If mixed_nalu_types_in_pic_flagidc is equal to 0, the value of nal_unit_type shall be the same for all coded slice NAL units of a picture. A picture or a PU is referred to as having the same NAL unit type as the coded slice NAL units of the picture or PU.

–	Otherwise( mixed_nalu_types_in_pic_flagidc equal to 1 or 2), the following applies.

–	If one of the VCL NAL units in the picture has value of nal_unit_type equal to TRAIL_NUT, a picture or a PU is referred to as being a trailing picture or a trailing PU.

–	Otherwise (no TRAIL_NUT VCL NAL units in the picture), a picture or PU is referred to as being a leading picture or a leading PU.

–	If one of the VCL NAL units in the picture has value of nal_unit_type equal to RADL_NUT, a picture or a PU is referred to as being a RADL picture or a RADL PU.

–	Otherwise (none of the VCL NAL units in the picture has value of nal_unit_type equal to RADL_NUT), a picture or a PU is referred to as being a RASL picture or a RASL PU.

–	Otherwise (mixed_nalu_types_in_pic_flagidc equal to 3), a picture is referred to as being a CRA picture or.a CRA PU.

Note that when NAL unit types of RADL_NUT or RASL_NUT are mixed with STSA_NUTs, there is no information whether the mixed AU has the characteristics of STSA AUs.

The proposed specification changes to the general constraint information are shown in the following:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_progressive_source_flag | u(1) |
| … |  |
| leading_stsa_pictures_flag | u(1) |
| … |  |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| } |  |

leading_stsa_pictures_flag equal to 1 specifies that inter prediction reference for RADL and RASL pictures are constraint as specified below. leading_stsa_pictures_flag equal to 0 does not impose such a constraint.

When leading_stsa_pictures_flag equal to 1, the following applies:

- When the current picture is a RASL or RADL picture, there shall be no active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that has TemporalId equal to that of the current picture.

- When the current picture is a picture that follows, in decoding order, a RASL or RADL picture and preceds the associated IRAP picture that has TemporalId equal to that of the current picture, there shall be no picture that has TemporalId equal to that of the current picture included as an active entry in RefPicList[ 0 ] or RefPicList[ 1 ] that precedes the RASL or RADL picture in decoding order.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)