| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0398 |
| --- | --- |

| Title: | AHG9: Sub-layer wise dependency in multi-layer |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

The layer dependency signalling in VVC cannot express coding structures with independent sub-layers in higher layer, albeit such coding structures can be beneficial to lower level requirements. Exemplary operation points are given for various codec levels and use cases.

The contribution proposes to extend the dependency signalling with a sub-layer specific indication that allows to identify the independent sub-layers. Modifications to OLS and extraction process are proposed to benefit from the lower codec level requirements by dropping the lower layer sub-layer pictures that are not used for reference and not output.

Revision 1 of this document updates the proposal to include the syntax similar as in HEVC.

# Introduction

Layer dependency signalling in the current VVC draft specification is similar to HEVC. The dependency of one layer on another layer is indicated in the VPS by the syntax elements vps_independent_layer_flag[ i ] and vps_direct_ref_layer_flag[ i ][ j ] that indicate whether a layer is independent or, when it is not independent, which are its reference layers, respectively. In addition, the reference picture list indicates in a granular way whether a given slice is predicted by using inter-layer prediction or not.

One could envision that in a scalable bitstream not all picture in the enhancement layer depend on their corresponding lower layers, as shown in the following Figure 1, in which one OLS (decodable subset marked grey) would entail all coded pictures in the bitstream except for the L0 – Tid1:

Figure 1: Example of a bitstream with two layers with the highest sub-layer in layer 1 being independent of layer 0

It is asserted that such an encoding requires less samples to be decoded involved for decoding a target TID at enhancement layers and therefore, lower decoder level requirements. Some examples are shown in the following tables that show the highest framerate that can be decoded with a given level:

SNR scalability

Table 1:Max framerate

| Size | # indep sub-layers | Level = 4.0 | Level=4.1 | Level=5.0 | Level=5.1 |
| --- | --- | --- | --- | --- | --- |
| 1080x1920 | 0 | 16 | 32 | 64 | 128 |
|  | 1 | 21.3 | 42.6 | 85.3 | 170.6 |
|  | 2 | 24 | 48 | 96 | 192 |
| 1920x2560 (16VGA) | 0 | N/A | N/A | 27.7 | 54.4 |
|  | 1 | N/A | N/A | 36.2 | 72.5 |
|  | 2 | N/A | N/A | 40.8 | 81.6 |

For instance, in the table above, it is shown that by having some sub-layers independent for layer 1 framerates of 24 Hz or 30 Hz can be achieved.

Spatial Scalability 1.5x

Table 1:Max framerate

| Size | # indep sub-layers | Level = 4.0 | Level=4.1 | Level=5.0 | Level=5.1 |
| --- | --- | --- | --- | --- | --- |
| 1080x1920 | 0 | 22.1 | 44.3 | 88.6 | 177.2 |
|  | 1 | 26.1 | 52.3 | 104.7 | 209.4 |
|  | 2 | 27.8 | 55.7 | 111.4 | 222.9 |
| 1920x2560 (16VGA) | 0 | N/A | N/A | 37.6 | 75.3 |
|  | 1 | N/A | N/A | 44.5 | 89 |
|  | 2 | N/A | N/A | 47.3 | 94.7 |

For instance, in the table above, it is shown that by having some sub-layers independent for layer 1 framerates of 24 Hz or 50 Hz can be achieved.

Spatial Scalability 2x

| Size | # indep sub-layers | Level = 4.0 | Level=4.1 | Level=5.0 | Level=5.1 |
| --- | --- | --- | --- | --- | --- |
| 1080x1920 | 0 | 25.6 | 51.2 | 102.4 | 204.8 |
|  | 1 | 28.4 | 56.8 | 113.7 | 227.5 |
|  | 2 | 29.5 | 59 | 118.1 | 236.3 |
| 1920x2560 (16VGA) | 0 | N/A | N/A | 43.5 | 87 |
|  | 1 | N/A | N/A | 48.3 | 96.7 |
|  | 2 | N/A | N/A | 50.2 | 100.4 |

For instance, in the table above, it is shown that by having some sub-layers independent for layer 1 framerates of Hz can be achieved.

However, it is also asserted that typically by disallowing inter-layer prediction at higher sub-layers the coding efficiency gains of layered coding versus simulcast or gains on CDN storage for DASH environments are reduced. This is a trade-off that could be controlled in context of the requirements for the application or service.

# Proposal

The proposal is to introduce high-level signalling on SPS level that allows to express sub-layer specific inter-layer dependency signalling as highlighted in the following.

## Signalling

#### Video parameter set

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| vps_max_layers_minus1 | u(6) |
| vps_max_sublayers_minus1 | u(3) |
| if( vps_max_layers_minus1 > 0 && vps_max_sublayers_minus1 > 0 ) |  |
| vps_all_layers_same_num_sublayers_flag | u(1) |
| if( vps_max_layers_minus1 > 0 ) |  |
| vps_all_independent_layers_flag | u(1) |
| for( i = 0; i <= vps_max_layers_minus1; i++ ) { |  |
| vps_layer_id[ i ] | u(6) |
| if( i > 0 && !vps_all_independent_layers_flag ) { |  |
| vps_independent_layer_flag[ i ] | u(1) |
| if( !vps_independent_layer_flag[ i ] ) { |  |
| for( j = 0; j < i; j++ ) |  |
| vps_direct_ref_layer_flag[ i ][ j ] | u(1) |
| max_tid_ref_present_flag[ i ] | u(1) |
| if( max_tid_ref_present_flag[ i ] ) |  |
| max_tid_il_ref_pics_plus1[ i ] | u(3) |
| } |  |
| } |  |
| } |  |
| […] |  |

max_tid_ref_present_flag[ i ] equal to 1 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ] is present. max_tid_ref_present_flag[ i ] equal to 0 specifies that the syntax element max_tid_il_ref_pics_plus1[ i ] is not present.

max_tid_il_ref_pics_plus1[ i ] equal to 0 specifies that non-IRAP pictures in any direct or indirect reference layer are not used as source pictures for inter-layer prediction for pictures with nuh_layer_id equal to vps_layer_id[ i ]. max_tid_il_ref_pics_plus1[ i ] greater than 0 specifies that pictures in any direct or indirect reference layer and with TemporalId greater than max_tid_il_ref_pics_plus1[ i ] − 1 are not used as source pictures for inter-layer prediction for pictures with nuh_layer_id equal to vps_layer_id[ i ]. When not present, the value of max_tid_il_ref_pics_plus1[ i ] is inferred to be equal to 7.

## PTL implications and Extraction Process

Two options have been considered with respect to PTL implications. The first one is to take into account the max_tid_il_ref_pics_plus1[ ] for OLS and therefore remove the unnecessary sub-layers in the non-output layers during the extraction process. The second option is to add an additional PTL for the case that the unnecessary sub-layers in the non-output layers are removed.

The first option is considered to be beneficial and therefore is proposed in the following, including changes to the semantics of ols_output_layer_flag[ i ][ j ] as well as the sub-bitstream extraction process in Annex C.6.

ols_output_layer_flag[ i ][ j ] equal to 1 specifies that the layer with nuh_layer_id equal to vps_layer_id[ j ] is an output layer of the i-th OLS when ols_mode_idc is equal to 2. ols_output_layer_flag[ i ][ j ] equal to 0 specifies that the layer with nuh_layer_id equal to vps_layer_id[ j ] is not an output layer of the i-th OLS when ols_mode_idc is equal to 2.

The variable NumOutputLayersInOls[ i ], specifying the number of output layers in the i-th OLS, NumSubLayersInLayerInOLS[ i ][ j ], specifying the number of sub-layers in the j-th layer in the i-th OLS, and the variable OutputLayerIdInOls[ i ][ j ], specifying the nuh_layer_id value of the j-th output layer in the i-th OLS, are derived as follows:

NumOutputLayersInOls[ 0 ] = 1
OutputLayerIdInOls[ 0 ][ 0 ] = vps_layer_id[ 0 ]
NumSubLayersInLayerInOLS[ 0 ][ 0 ] = vps_max_sub_layers_minus1 + 1
for( i = 1; i < TotalNumOlss; i++ ) {
	if( each_layer_is_an_ols_flag  | |  ols_mode_idc  = =  0 ) {
		NumOutputLayersInOls[ i ] = 1
		OutputLayerIdInOls[ i ][ 0 ] = vps_layer_id[ i ]
		for( j = 0; j < i && (ols_mode_idc  = =  0); j++ )
			NumSubLayersInLayerInOLS[ i ][ j ] = max_tid_il_ref_pics_plus1[ i ]
		NumSubLayersInLayerInOLS[ i ][ i ] = vps_max_sub_layers_minus1 + 1
	} else if( ols_mode_idc  = =  1 ) {
		NumOutputLayersInOls[ i ] = i + 1
		for( j = 0; j < NumOutputLayersInOls[ i ]; j++ ) {
			OutputLayerIdInOls[ i ][ j ] = vps_layer_id[ j ]
			NumSubLayersInLayerInOLS[ i ][ j ] = vps_max_sub_layers_minus1 + 1
		}
	} else if( ols_mode_idc  = =  2 ) {
		for( j = 0; j  <=  vps_max_layers_minus1; j++ ) {
			layerIncludedInOlsFlag[ i ][ j ] = 0
			NumSubLayersInLayerInOLS[ i ][ j ] = 0
		}
		for( k = 0, j = 0; k  <=  vps_max_layers_minus1; k++ ) 	(40)
			if( ols_output_layer_flag[ i ][ k ] ) {
				layerIncludedInOlsFlag[ i ][ k ] = 1
				OutputLayerIdx[ i ][ j ] = k
				OutputLayerIdInOls[ i ][ j++ ] = vps_layer_id[ k ]
				NumSubLayersInLayerInOLS[ i ][ j ] = vps_max_sub_layers_minus1 + 1
			}
		NumOutputLayersInOls[ i ] = j
		for( j = 0; j < NumOutputLayersInOls[ i ]; j++ ) {
			idx = OutputLayerIdx[ i ][ j ]
			for( k = 0; k < NumRefLayers[ idx ]; k++ ) {
				layerIncludedInOlsFlag[ i ][ RefLayerIdx[ idx ][ k ] ] = 1
				if( NumSubLayersInLayerInOLS[ i ][ RefLayerIdx[ idx ][ k ] ] < 				max_tid_il_ref_pics_plus1[ OutputLayerIdInOls[ i ][ j ] ] )
					NumSubLayersInLayerInOLS[ i ][ RefLayerIdx[ idx ][ k ] ] = 
					max_tid_il_ref_pics_plus1[ OutputLayerIdInOls[ i ][ j ] ]
			}
		}
	}
}

The sub-bitstream extraction process in Annex C.6 is modified as follows:

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, and a target highest TemporalId value tIdTarget.

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that is the output of the process specified in this clause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and tIdTarget equal to any value in the range of 0 to 6, inclusive, as inputs, and that satisfies the following conditions shall be a conforming bitstream:

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

The output sub-bitstream OutBitstream is derived as follows:

–	The bitstream outBitstream is set to be identical to the bitstream inBitstream.

–	Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

–	Remove from outBitstream all NAL units with nal_unit_type not equal to any of VPS_NUT, DPS_NUT, and EOB_NUT and with nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ].

–	Remove from outBitstream all NAL units for which all of the following conditions are true:

–	nal_unit_type is not equal to any of IDR_W_RADL, IDR_N_LP and CRA_NUT.

–	nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] – 1 inclusive.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ j ].

–	Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has nesting_ols_flag equal to 1 and there is no value of i in the range of 0 to nesting_num_olss_minus1, inclusive, such that NestingOlsIdx[ i ] is equal to targetOlsIdx.

–	When targetOlsIdx is greater than 0, remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 0 (buffering period), 1 (picture timing), or 130 (decoding unit information).

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)