| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0399 |
| --- | --- |

| Title: | AHG8: On conformance window and scaling window |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yago Sanchez Robert Skupin Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

In the current VVC draft specification, the scaling ratio derivation for reference picture resampling and scalability support was changed from using the conformance window to using a scaling window that might be different to the conformance window.

In many cases both values are the same and therefore seems to be redundant information. This contribution proposes to add inference values of scaling window to be equal to the conformance window when the syntax elements are not present.

# Introduction

During the previous JVET meeting the scaling ratio required for reference picture resampling and scalability support was changed from using the conformance window to an explicit signalling that is decoupled, namely the scaling window.

With that change, some shortcomings of using the conformance window are solved. The newly added scaling window supports in fact some basic functionalities for ROI.

However, in most practical scenarios, where no ROI is used but scaling is applied to the whole reference picture, a target scaling ratio is selected based on the croppedOut picture. Therefore, in many scenarios, the conformance window and scaling window will typically be the equal.

In addition, it is asserted that the constraint for scaling_window_flag “When ref_pic_resampling_enabled_flag is equal to 0, the value of scaling_window_flag shall be equal to 0” is not desirable as the scaling window functionality is also used for inter-layer prediction in case of scalability even when coded picture sizes within each individual layers are constant.

# Proposal

The proposal here is to change the inference value of the scaling offsets that when not present in the bitstream and reference picture resampling is enabled, the value is inferred to be equal to the corresponding conformance window offset values.

scaling_window_flag equal to 1 specifies that the scaling window offset parameters are present in the PPS. scaling_window_flag equal to 0 specifies that the scaling window offset parameters are not present in the PPS. When ref_pic_resampling_enabled_flag is equal to 0, the value of scaling_window_flag shall be equal to 0.

scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset specify the offsets, in units of luma samples, that are applied to the picture size for scaling ratio calculation. When scaling_window_flag is equal to 0, the values of scaling_win_left_offset, scaling_win_right_offset, scaling_win_top_offset, and scaling_win_bottom_offset are inferred to be equal to the values of conf_win_left_offset, conf_win_right_offset, conf_win_top_offset, and conf_win_bottom_offset, respectively.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)