| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0401 |
| --- | --- |

| Title: | AHG9/AHG12: On bitstream merging |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

Merging of multiple input bitstreams (representing subpictures) into a composite joint output bitstream is valid operation choice for some applications where a single decoding instance is required, e.g. 360-degree video streaming to constrained devices.

This document proposes an SEI message to express that bitstreams originate from a coordinated encoding setup and can be merged without low-level rewriting or even adjusting APS, PH, SH. In addition, information towards the level of the merged bitstream based on the subpicture level information SEI message is derived.

# Introduction

Merging of multiple input bitstreams (representing subpictures) into a composite joint output bitstream is valid operation choice for some applications where a single decoding instance is required, e.g. 360-degree video streaming to constrained devices.

However, while the scope of HEVC and AVC specifications is only a single bitstream and same is envisioned for the VVC draft specification, information guiding systems in carrying out a merging operation can be carried in each subpicture bitstream individually. JVET-O492 and JVET-P0459 proposed signalling variants which were intended to help in content preparation or at client side during playout to identify suitable merging method of varying complexity.

As stated during those meetings, it became clear that merging operations will likely require highly-coordinated encoder setups and a large variety of merging methods may not be of interest. Therefore, the contribution proposes two options adjusted to that insight.

In the first proposal option 1, the proposal is reduced compared to the earlier proposals in that only a single merging method (rewriting DPS, VPS, SPS, PPS) is in scope, thereby excluding merging methods that would be able to address header issues such as virtual boundaries mismatches or slice_subpic_id collisions.

The second proposal option 2 includes the above merging method that requires rewriting DPS, VPS, SPS and PPS and, in addition, a second method that can address PH and slice header issues as described above.

Furthermore, the proposal is refined with respect to level information, which is carried out based on the subpicture level information SEI message as adopted at the last JVET meeting which allows to derive the level contribution of single subpicture as well as a set of subpictures originating from a single bitstream. This proposal includes a derivation of the level contribution a set of subpictures originating from a different bitstream, each carrying a subpicture level information SEI message with a common reference level.

# Proposal

Option 1 – Single Merging Method (DPS, VPS, SPS, PPS rewriting):

Mergeability information SEI message syntax

| mergeability_information( payloadSize ) { | Descriptor |
| --- | --- |
| num_mergeability_ids_minus1 | u(8) |
| for( i = 0; i <= num_mergeability_ids_minus; i ++) |  |
| mergeability_id[ i ] | u(16) |
| } |  |

Mergeability information SEI message semantics

The mergeability information SEI message carries information that allows identification of mergeable (subpicture) bitstreams.

When a mergeability information SEI message is present for any picture of a CLVS, a mergeability information SEI message shall be present for the first picture of the CLVS. The mergeability information SEI message persists for the current layer in decoding order from the current picture until the end of the CLVS. All mergeability information SEI messages that apply to the same CLVS shall have the same content.

A mergeability information SEI message shall not be present in an access unit unless there is subpicture level information SEI message present in the access unit. A subpicture level information SEI message present in the same access unit as a mergeability information SEI message is referred to as the associated subpicture level information  SEI message of the mergeability information SEI message.

num_mergeability_ids_minus1 plus 1 specifies the number of merge idenfiers in the SEI message. The value of num_mergeability_ids_minus1 shall be in the range of 0 to 255, inclusive.

mergeability_id[ i ] contains the i-th identifying number that may be used to identify bitstreams with matching value of mergeability_id[ i ] which can be merged with the current bitstream into an conformant joint bitstream as described below. The value of mergeability_id[ i ] shall be in the range of 0 to 214 − 1, inclusive. If some external means, not specified in this Specification, is available to set the value of MergeabilityIdSpace, it is a bitstream conformance that bitstreams carrying the same value of mergeability_id[ i ] and MergeabilityIdSpace can be merged as described below. Values of mergeability_id[ i ] from 214 to 216 − 1, inclusive, are reserved for future use by ITU-T | ISO/IEC and bitstreams shall not contain such values. Decoders encountering a value of mergeability_id[ i ] in the range of 214 to 216 − 1, inclusive, shall ignore it.

Merging one bitstream with another bitstream with matching values of the associated mergeability_id[ i ] and MergeabilityIdSpace into a single joint bitstream does not require changes of referred APS, picture headers, slice headers or slice payloads in the bitstream.

NOTE 1 – Merging of a set of numBs bitstreams inBitstreams that carry the same value of mergeability_id[ i ] and MergeabilityIdSpace into a single bitstream outBitstream, requires rewriting of the values of syntax elements in the DPS, VPS, SPS, and PPS of the bitstreams in inBitstreams before a copy of the required parameter set are put into outBitstream. The affected syntax elements are the profile_tier_level() syntax structre as well as syntax elements related to picture size, tiles, slices and subpictures. The changes depend on the spatial layout of the bitstreams of inBitstream within the coded picture of outBitstream which is defined by some external means, not specified in this Specification.

When a set of numBs bitstreams inBitstreams that carry the same value of mergeability_id[ i ] and MergeabilityIdSpace are merged into a single bitstream outBitstream, the level information of outBitstream is derived as follows.

There shall be at least one value of i, denoted as ik, for the k-th bitstream in inBitstreams with k = 0..numBs for which the value of ref_level_idc[ ik ] shall be equal in all bitstreams in inBitstreams.

For the bitstream set inBitstreams, the level information of outBitstream is derived based on the variables SubPicSetAccLevelFraction[ ik ], SubPicSetCpbSizeVcl[ ik ], SubPicSetCpbSizeNal[ ik ], SubPicSetBitRateVcl[ ik ], SubPicSetBitrateNal[ ik ], SubPicSetNumTiles[ ik ] derived for each bitstream in inBitstreams for the subpicture set that consists of all subpictures in of the bitstreams according to D.3.8 and rearranged into the lists inBsAccLevelFraction[ k ], inBsCpbSizeVcl[ k ], inBsCpbSizeNal[ k ], inBsBitRateVcl[ k ], inBsBitRateNal[ k ], inBsNumTiles[ k ].

[Ed.: BitRate derivation is currently missing from the subpicture level information SEI message but proposed in JVET-Q0395]

The variable OutBsAccLevelFraction for the total level fraction with respect to the reference level ref_level_idc[ ik ], and the variables OutBsCpbSizeVcl, OutBsCpbSizeNal, OutBsBitRateVcl, OutBsBitRateNal, and OutBsNumTiles of outBitstream, are derived as follows:

OutBsAccLevelFraction = 0
OutBsCpbSizeVcl = 0
OutBsCpbSizeNal = 0
OutBsBitRateVcl = 0
OutBsBitRateNal = 0
OutBsNumTiles  = 0
for (k = 0; k < numBs; k ++) {
	OutBsAccLevelFraction  +=  inBsAccLevelFraction[ k ]	(D.X)
	OutBsCpbSizeVcl  +=  inBsCpbSizeVcl[ k ]
	OutBsCpbSizeNal  +=  inBsCpbSizeNal[ k ]
	OutBsBitRateVcl  +=  inBsBitRateVcl[ k ]
	OutBsBitRateNal  +=  inBsBitRateNal[ k ]
	OutBsNumTiles  +=  inBsNumTileCols[ k ] * inBsNumTileRow[ k ]
}

It is a requirement of bitstream conformance that the following applies for outBitstream:

- The sum of OutBsAccLevelFraction is less or equal to 100

- OutBsNumTiles is less than or equal to MaxTileCols * MaxTileRows,

- OutBsCpbSizeVcl and OutBsCpbSizeNal are less than or equal to MaxCPB,

- OutBsBitRateVcl and OutBsBitRateNal are less than or equal to MaxBR,

where MaxTileCols, MaxTileRows, MaxCPB and MaxBR are specified for ref_level_idc[ ik ] in Table A.1.

The variable picHeightY and picWidthY is set equal to the height and width in number of samples of the maximum luma picture sample array of the coded pictures in outBitstream, and the variable picSizeY is set equal to picHeightY * picWidthY, and the variable numTileCols and numTileRows are set equal to the number of tile coloumns and tile rows in the coded picture of outBitstream.

The bitstream outBitstream conforming to a profile with general_tier_flag equal to 0 and a level equal to ref_level_idc[ ik ] shall obey the following constraints for each bitstream conformance test as specified in Annex C:

–	Ceil( 256 * picSizeY ÷ OutBsAccLevelFraction ) shall be less than or equal to MaxLumaPs, where MaxLumaPs is specified in Table A.1.

–	The value of Ceil( 256 * ( picWidthY ) ÷ OutBsAccLevelFraction ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of Ceil( 256 * ( picHeightY ) ÷ OutBsAccLevelFraction ) shall be less than or equal to Sqrt( MaxLumaPs * 8 ).

–	The value of numTileCols shall be less than or equal to MaxTileCols and of numTileRows shall be less than or equal to MaxTileRows, where MaxTileCols and MaxTileRows are specified in Table A.1.

- For the VCL HRD parameters, OutBsCpbSizeVcl shall be less than or equal to CpbVclFactor * MaxCPB, where CpbVclFactor is specified in Table A.3 and MaxCPB is specified in Table A.1 in units of CpbVclFactor bits.

- For the NAL HRD parameters, OutBsCpbSizeNal shall be less than or equal to CpbNalFactor * MaxCPB, where CpbNalFactor is specified in Table A.3, and MaxCPB is specified in Table A.1 in units of CpbNalFactor bits.

- For the VCL HRD parameters, OutBsBitRateVcl shall be less than or equal to CpbVclFactor * MaxBR, where CpbVclFactor is specified in Table A.3 and MaxBR is specified in Table A.1 in units of CpbVclFactor bits.

- For the NAL HRD parameters, OutBsBitRateNal shall be less than or equal to CpbNalFactor * MaxBR, where CpbNalFactor is specified in Table A.3, and MaxBR is specified in Table A.1 in units of CpbNalFactor bits.MaxCPB

NOTE – When a bitstream set is merged, the resulting bitstream has a CpbSize (either indicated in the SPS  or inferred) that is greater than or equal to OutBsCpbSizeVcl  and OutBsCpbSizeNal and a BitRate (either indicated in the SPS  or inferred) that is greater than or equal to OutBsBitRateVcl  and OutBsBitRateNal.

Proposal Option 2:– Two Merging Methods:

(DPS, VPS, SPS, PPS rewriting vs. DPS, VPS, SPS, PPS, PH, SH rewriting)

Only changes with respect to proposal option 1 are shown and highlighted:

Mergeability information SEI message syntax

| mergeability_information( payloadSize ) { | Descriptor |
| --- | --- |
| num_mergeability_ids_minus1 | u(8) |
| for( i = 0; i <= num_mergeability_ids_minus; i ++) { |  |
| mergeability_id[ i ] | u(15) |
| mergeability_type[ i ] | u(1) |
| } |  |
| } |  |

Mergeability information SEI message semantics

mergeability_id[ i ] contains the i-th identifying number that may be used to identify bitstreams with matching value of mergeability_id[ i ] which can be merged with the current bitstream into an conformant joint bitstream as described below. The value of mergeability_id[ i ] shall be in the range of 0 to 214 − 1, inclusive. If some external means, not specified in this Specification, is available to set the value of MergeabilityIdSpace, it is a bitstream conformance that more than one bitstream carrying the same value of mergeability_id[ i ] and MergeabilityIdSpace can be merged according to mergeability_type[ i ] as described below. Values of mergeability_id[ i ] from 214 to 215 − 1, inclusive, are reserved for future use by ITU-T | ISO/IEC and bitstreams shall not contain such values. Decoders encountering a value of mergeability_id[ i ] in the range of 214 to 215 − 1, inclusive, shall ignore it.

Merging one bitstream with another bitstream with matching values of the associated mergeability_id[ i ] and MergeabilityIdSpace into a single joint bitstream does not require changes of referred APS, picture headers, slice headers or slice payloads in the bitstream.

mergeability_type[ i ] equal to 0 specifies that merging of a bitstream with another bitstream with matching values of the associated mergeability_id[ i ] and MergeabiltyIdSpace into a single joint bitstream does not require changes of referred APS, picture headers, slice headers or slice payloads in the bitstream. mergeability_type[ i ] equal to 1 specifies that merging of a bitstream with another bitstream with matching values of the associated mergeability_id[ i ] and MergeabilityIdSpace into a single joint bitstream does not require changes of slice payloads in the bitstream.

NOTE 1 – Merging of a set of numBs bitstreams inBitstreams that carry the same value of mergeability_id[ ] and MergeabilityIdSpace, and a mergeabillity_type[ ] equal to into a single bitstream outBitstream, requires rewriting of the values of syntax elements in the DPS, VPS, SPS, and PPS of the bitstreams in inBitstreams before a copy of the required parameter set are put into outBitstream. The affected syntax elements are the profile_tier_level() syntax structre as well as syntax elements related to picture size, tiles, slices and subpictures. The changes depend on the spatial layout of the bitstreams of inBitstream within the coded picture of outBitstream which is defined by some external means, not specified in this Specification.

NOTE 1 – Merging of a set of numBs bitstreams inBitstreams that carry the same value of mergeability_id[ i ] and MergeabilityIdSpace and and a mergeabillity_type[ ] equal to 0, into a single bitstream outBitstream, requires rewriting of the values of syntax elements in the DPS, VPS, SPS, and PPS of the bitstreams in inBitstreams before a copy of the required parameter set are put into outBitstream. The affected syntax elements are the profile_tier_level() syntax structre as well as syntax elements related to picture size, tiles, slices and subpictures. The changes depend on the spatial layout of the bitstreams of inBitstream within the coded picture of outBitstream which is defined by some external means, not specified in this Specification. When mergeabillity_type[ ] equal to1 , rewriting of APS, PH and SH may be necessary in addition.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)