| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0402-v2 |
| --- | --- |

| Title: | AHG12: On subpicture and scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes to add signalling that indicates that subpicture boundaries across layers of an OLS are aligned in a way that all subpicture boundaries of a reference layer are aligned with collocated subpicture boundaries in the referencing higher layer and inter-layer prediction references obey the boundaries of subpictures with subpic_treated_as_pic_flag[ i ] equal to 1 in inter-layer prediction.

A flag is added to the OLS syntax in the VPS and changes are proposed to the relevant parts of Section 8.5.

Revision 1 of this document adds proposed specification text changes (Option A and Option B) according to the comments received as reported in the HLS BoG report in JVET-Q0625.

The proposed modification are as follows:

Option A: add a conformance constraint to the semantics of subpic_treated_as_pic_flag[] to allows SNR scalability with independent subpictures when subpictures are aligned.

Option B: Prohibit the use of subpictures with in subpic_treated_as_pic_flag[ i ] equal to 1 in dependent layers.

# Introduction

There are different use-cases that make use of subpictures in layered bitstreams. Some include encoder parallelization, other RoI scalability or 360° streaming scenarios where only the higher resolution layer is divided into subpictures, etc.

Three examples of potential subpicture configurations are shown in the following that are considered to be relevant.

Figure 1: Scalable subpicture based bitstream with aligned subpictures

In Figure 1, an example is shown where subpicture in both layer of an OLS are completely aligned in the way that each subpicture in the reference layer is collocated with a subpicture in the referencing higher

layer.

Figure 2: Scalable subpicture based bitstream with partially aligned subpictures

In Figure 2, an example is shown where subpicture boundaries of the lower layer are also collocated with subpicture boundaries in the referencing layer, but the referencing higher layer contains additional subpictures.

However, there are cases where subpicture boundaries do not have collocated subpicture boundaries in the referencing higher layers as shown in the following.

Figure 3: Scalable bitstream with subpictures only in lower layers

Figure 3 shows an example where the base layer is provided with subpicture based RoI but the enhancement layer does not have subpictures. Since the enhancement layer does not have any subpicture and therefore subpictures are not aligned, it does not make sense to take into account the subpicture boundaries for inter-layer prediction in this example.

In SHVC, alignment of tiles was signalled as a constraint for applications to use. However, as discussed above and proposed in the following, non-alignment would have an impact on the decoding process, i.e. not clipping sample positions depending whether the reference picture is an inter-layer reference or not.

# Proposal

Option A: Allow SNR scalability with independent subpictures

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0.

When subpic_treated_as_pic_flag[ i ] is equal to 1, it is a requirement of bitstream conformance that all of the following conditions shall be true for each output layer and its reference layers in an OLS that contains the output layer with the i-th subpicture:

- all pictures have the same value of pic_width_in_luma_samples and pic_height_in_luma_samples in the PPS.

- all subpictures of pictures have the same values of sps_num_subpics_minus1, subpic_ctu_top_left_x[ j ], subpic_ctu_top_left_y[ j ], subpic_width_minus1[ j ], subpic_height_minus1[ j ] and loop_filter_across_subpic_enabled_flag[ j ] of the j-th subpicture for j in the range of 0 to sps_num_subpics_minus1 in the SPS.

- all pictures in an access unit have the same value of SubpicIdList[ j ] of the j-th subpicture for j in the range of 0 to sps_num_subpics_minus1 in the SPS.

Option B: Prohibit the use of subpictures with in subpic_treated_as_pic_flag[ i ] equal to 1 in dependent layers.

vps_independent_layer_flag[ i ] equal to 1 specifies that the layer with index i does not use inter-layer prediction. vps_independent_layer_flag[ i ] equal to 0 specifies that the layer with index i may use inter-layer prediction and the syntax elements vps_direct_ref_layer_flag[ i ][ j ] for j in the range of 0 to i − 1, inclusive, are present in VPS. When not present, the value of vps_independent_layer_flag[ i ] is inferred to be equal to 1.

When vps_independent_layer_flag[ i ] equal to 0, it is a requirement of bitstream conformance that the value of subpic_treated_as_pic_flag[ j ] shall be equal to 0 for each j-th subpicture in the SPS of the layer with nuh_layer_id equal to nuh_layer_id[ i ].

Option C (Initial proposal):

Two aspects are proposed in the following:

- Signalling of alignment across layers

- Subpicture boundary handling for inter-layer motion compensation

The first part of this proposal in this document is to add a flag to the SPS that indicates that subpictures are fully aligned (one-to-one mapping) across layers in an OLS.

If so, subpictures are taken into account when handling inter-layer motion compensation, i.e. they are treated as indicated by the syntax element subpic_treated_as_pic_flag also for inter-layer prediction. It is also proposed that, in this case, the subpicture IDs of the different layers shall be the same for collocated subpictures as identifying collocated subpictures is easier.

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| if( vps_max_layers_minus1 > 0 ) { |  |
| if( vps_all_independent_layers_flag ) |  |
| each_layer_is_an_ols_flag | u(1) |
| if( !each_layer_is_an_ols_flag ) { |  |
| if( !vps_all_independent_layers_flag ) |  |
| ols_mode_idc | u(2) |
| if( ols_mode_idc = = 2 ) { |  |
| num_output_layer_sets_minus1 | u(8) |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) { |  |
| ols_subpics_across_layers_aligned_flag[ i ] | u(1) |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |
| ols_output_layer_flag[ i ][ j ] | u(1) |
| } |  |
| } |  |
| } |  |
| } |  |

ols_subpics_across_layers_aligned_flag[ i ] equal to 1 specifies that the subpicutres of layers in the i-th OLS are aligned in spatial position across layers in a way that all subpicture boundaries of a reference layer are aligned with collocated subpicture boundaries in the referencing higher layer and inter-layer prediction obeys the boundaries of subpictures with subpic_treated_as_pic_flag[ i ] equal to 1 in inter-layer prediction. ols_subpics_across_layers_aligned_flag[ i ] equal to 0 specifies that the subpicutres of layers in the i-th OLS may or may not be aligned in spatial position across layers and inter-layer prediction does not obey the boundaries of subpictures with subpic_treated_as_pic_flag[ i ] equal to 1 in inter-layer prediction.

The proposed changes to the relevant section in 8.5.X are shown highlighted.

8.5.3.2.2	Luma sample bilinear interpolation process

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1 and interLayerPredictionFlag is equal to 0 or ols_subpics_across_layers_aligned_flag[ TargetOlsIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i )	(642)

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i )	(643)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0 or interLayerPredictionFlag is equal to 1 and ols_subpics_across_layers_aligned_flag[ TargetOlsIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(644)
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(645)

8.5.5.3	Derivation process for subblock-based temporal merging candidates

- For xSbIdx = 0..numSbX − 1 and ySbIdx = 0 .. numSbY − 1, the motion vectors mvLXSbCol[ xSbIdx ][ ySbIdx ] and prediction list utilization flags predFlagLXSbCol[ xSbIdx ][ ySbIdx ] are derived as follows:

- The luma location ( xSb, ySb ) specifying the below-right center sample of the current coding subblock relative to the top-left luma sample of the current picture is derived as follows:

xSb  =  xCb + xSbIdx * sbWidth + sbWidth / 2	(733)

ySb  =  yCb + ySbIdx * sbHeight + sbHeight / 2	(734)

- The location ( xColSb, yColSb ) of the collocated subblock inside ColPic is derived as follows.

    - The following applies:

yColSb = Clip3( yCtb,
	Min( pic_height_in_luma_samples − 1, yCtb + ( 1  <<  CtbLog2SizeY ) − 1 ),	(735)
	ySb + tempMv[1] )

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1 and interLayerPredictionFlag is equal to 0 or ols_subpics_across_layers_aligned_flag[ TargetOlsIdx ] is equal to 1, the following applies:

xColSb = Clip3( xCtb, 
	Min( SubPicRightBoundaryPos, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ),	(736)
	xSb + tempMv[0] )

  - Otherwise ( subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0 or interLayerPredictionFlag is equal to 1 and ols_subpics_across_layers_aligned_flag[ TargetOlsIdx ] is equal to 0), the following applies:

xColSb = Clip3( xCtb,
	Min( pic_width_in_luma_samples − 1, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ),	(737)

8.5.5.4	Derivation process for subblock-based temporal merging base motion data

The location ( xColCb, yColCb ) of the collocated block inside ColPic is derived as follows.

  - The following applies:

yColCb = Clip3( yCtb,
	Min( pic_height_in_luma_samples − 1, yCtb + ( 1  <<  CtbLog2SizeY ) − 1 ),	(742)
	yColCtrCb + tempMv[1] )

  - If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1 and interLayerPredictionFlag is equal to 0 or ols_subpics_across_layers_aligned_flag[ TargetOlsIdx ] is equal to 1, the following applies:

xColCb = Clip3( xCtb,
	Min( SubPicRightBoundaryPos, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ),	(743)
	xColCtrCb + tempMv[0] )

  - Otherwise ( subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0 or interLayerPredictionFlag is equal to 1 and ols_subpics_across_layers_aligned_flag[ TargetOlsIdx ] is equal to 0, the following applies:

xColCb = Clip3( xCtb,
	Min( pic_width_in_luma_samples − 1, xCtb + ( 1  <<  CtbLog2SizeY ) + 3 ),	(744)
	xColCtrCb + tempMv[0] )

8.5.6.3.3	Luma integer sample fetching process

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1 and interLayerPredictionFlag is equal to 0 or ols_subpics_across_layers_aligned_flag[ TargetOlsIdx ] is equal to 1, the following applies:

xInt = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL )	(966)

yInt = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL )	(967)

–	Otherwise, the following applies:

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?	(968)
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL ) : xIntL )

yInt = Clip3( 0, picH − 1, yIntL )		(969)

8.5.6.3.4	Chroma sample interpolation process

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1 and interLayerPredictionFlag is equal to 0 or ols_subpics_across_layers_aligned_flag[ TargetOlsIdx ] is equal to 1, the following applies:

xInti = Clip3( SubPicLeftBoundaryPos / SubWidthC, SubPicRightBoundaryPos / SubWidthC, xIntC + i − 1 )	(971)

yInti = Clip3( SubPicTopBoundaryPos / SubHeightC, SubPicBotBoundaryPos / SubHeightC, yIntC + i − 1 )	(972)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0 or interLayerPredictionFlag is equal to 1 and ols_subpics_across_layers_aligned_flag[ TargetOlsIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	(973)
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(974)

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)