| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0403 |
| --- | --- |

| Title: | AHG12: On subpicture specific MV wraparound |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

At the previous JVET meeting, JVET-P0127 proposed MV wraparound at subpicture boundaries but during discussion participants disliked the complexity burden of tracking subpicture boundaries.

This contribution proposes to allow MV wraparound when subpictures span the complete horizontal picture plane. For the purpose, a flag is added to the SPS syntax for the appropriate subpictures and the relevant derivations of reference samples in clause 8 are adjusted.

# Introduction

At the previous JVET meeting, JVET-P0127 proposed MV wraparound at subpicture boundaries. During the discussion, it was mentioned that the complexity burden associated with allowing this functionality was undesirable. Compared to the current VVC draft specification with MV wraparound procedure, it would be required to additionally track the subpicture boundaries.

However, it is also asserted that there is a value on allowing picture boundary MV wraparound on a per-subpicture granularity which would not incur a requirement of tracking additional boundaries within the picture. This excludes MV wraparound for references from within a subpicture that does not span the complete horizontal dimension of the picture. It is asserted that there is no significant complexity added to the decoding process in the proposed variant.

For instance, example use-cases related to 360-degree video that could benefit from such a per-subpicture switchable MV wraparound are shown in the following.

Figure 1: ERP with downscaled packed top and bottom polar regions.

Figure 2: Full low-resolution ERP (bottom) plus high-resolution viewport section (top).

The figures show 360-degree video divided into subpictures of which some are marked with red borders to indicate that MV wraparound is not beneficial for these subpictures, while the regions without the red border may benefit from MV wraparound.

With the current design, applications which rely on subpictures would have either disable MV wraparound completely or enable MV wraparound and encode certain subpictures with encoder-side constraints, all of which incur coding efficiency penalties.

The proposed design would infer disabled MV wraparound for the red regions in Figure 1 and use explicit signalling for the red region in Figure 2.

# Proposal

This contribution proposes to change MV wraparound to be switchable per-subpictures that span across the whole picture width. A syntax element sps_subpic_ref_wraparound_enabled_flag[ i ] is added to the SPS syntax.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |
| sps_ref_wraparound_enabled_flag | u(1) |
| if( sps_ref_wraparound_enabled_flag ) |  |
| sps_ref_wraparound_offset_minus1 | ue(v) |
| if( subpics_present_flag ) { |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| if( subpic_width_minus1[ i ] == Ceil( pic_width_max_in_luma_samples ÷ CtbSizeY ) – 1) |  |
| sps_subpic_ref_wraparound_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … | u(1) |
| } |  |

sps_subpic_ref_wraparound_enabled_flag[ i ] equal to 1 specifies that horizontal wrap-around motion compensation is applied in inter prediction when decoding the i-th subpicture.  sps_subpic_ref_wraparound_enabled_flag[ i ] equal to 0 specifies that horizontal wrap-around motion compensation is not applied in inter prediction when decoding the i-th subpicture.

When sps_ref_wraparound_enabled_flag is set to 1, there shall be at least one value of i in the range of 0 to sps_num_subpics_minus1 with sps_subpic_ref_wraparound_enabled_flag[ i ] equal to 1. When not present, the value of sps_subpic_ref_wraparound_enabled_flag[ i ] is inferred to be equal to 0.

The proposal further entails the following occurances of MV clipping are changed as follows.

##### 8.5.3.2.2	Luma sample bilinear interpolation process

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..1:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i )	(X)

- If sps_subpic_ref_wraparound_enabled_flag[ i ] is equal to 1, the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + ) :	(X) 
		xIntL + i )

- Otherwise (sps_subpic_ref_wraparound_enabled_flag[ i ] is equal to 0), the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i )	(X)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, ( xIntL + i ) ) :	(X)
		xIntL + i )

yInti = Clip3( 0, picH − 1, yIntL + i )		(X)

##### 8.5.6.3.2	Luma sample interpolation filtering process

The luma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..7:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

yInti = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL + i − 3 )	(X)

- If sps_subpic_ref_wraparound_enabled_flag[ i ] is equal to 1, the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(X) 
		xIntL + i − 3 )

- Otherwise (sps_subpic_ref_wraparound_enabled_flag[ i ] is equal to 0), the following applies:

xInti = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL + i − 3 )	(X)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 3 ) :	(X) 
		xIntL + i − 3 )

yInti = Clip3( 0, picH − 1, yIntL + i − 3 )		(X)

##### 8.5.6.3.3	Luma integer sample fetching process

The luma locations in full-sample units ( xInt, yInt ) are derived as follows:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

yInt = Clip3( SubPicTopBoundaryPos, SubPicBotBoundaryPos, yIntL )	(X)

- If sps_subpic_ref_wraparound_enabled_flag[ i ] is equal to 1, the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL  ) :	(X) 
		xIntL )

- Otherwise (sps_subpic_ref_wraparound_enabled_flag[ i ] is equal to 0), the following applies:

xInt = Clip3( SubPicLeftBoundaryPos, SubPicRightBoundaryPos, xIntL )	(X)

–	Otherwise, the following applies:

xInt = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?	(X)
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL ) : xIntL )

yInt = Clip3( 0, picH − 1, yIntL )		(X)

The predicted luma sample value predSampleLXL is derived as follows:

predSampleLXL = refPicLXL[ xInt ][ yInt ] << shift3		(X)

##### 8.5.6.3.4	Chroma sample interpolation process

The chroma locations in full-sample units ( xInti, yInti ) are derived as follows for i = 0..3:

–	If subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 1, the following applies:

yInti = Clip3( SubPicTopBoundaryPos / SubHeightC, SubPicBotBoundaryPos / SubHeightC, yIntL + i )	(X)

- If sps_subpic_ref_wraparound_enabled_flag[ i ] is equal to 1, the following applies:

xInti = Clip3( 0, picW − 1, sps_ref_wraparound_enabled_flag ?
		ClipH( ( sps_ref_wraparound_offset_minus1 + 1 ) * MinCbSizeY, picW, xIntL + i − 1 ) :	(X) 
		xIntL + i − 1 )

- Otherwise (sps_subpic_ref_wraparound_enabled_flag[ i ] is equal to 0), the following applies:

xInti = Clip3( SubPicLeftBoundaryPos / SubWidthC, SubPicRightBoundaryPos / SubWidthC, xIntL + i )	(X)

–	Otherwise (subpic_treated_as_pic_flag[ SubPicIdx ] is equal to 0), the following applies:

xInti = Clip3( 0, picWC − 1, sps_ref_wraparound_enabled_flag ? ClipH( xOffset, picWC, xIntC + i − 1 ) :	(X)
		xIntC + i − 1 )

yInti = Clip3( 0, picHC − 1, yIntC + i − 1 )		(X)

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)