| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0405 |
| --- | --- |

| Title: | AHG12: On subpicture and OLS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document proposes to support subpicture-based RoI scalability in which higher layers use only a subset of subpictures of the reference layers. Such a reduced subpicture reference (RSR) mode brings along benefits in terms of bitrate and decoder level requirements.

It is proposed to an indication about RSR mode to OLS and PTL signalling.

# Introduction

Subpictures in layered bitstreams can be used for RoI scalability where only the lower layer is divided into subpictures according to the RoI. In such subpicture-based RoI scalability scenarios, higher layers improve upon the RoI subpicture in terms of fidelity or resolution.

The following Figure 1 shows an example of subpicture-based RoI scalability where the spatial enhancement layer uses only the center RoI subpicture of the base layer for reference.

Figure 1: Example for subpicture-based multi-layer RoI scalability.

In such a scenario, the current VVC draft specification does not offer means to identify the limited inter-layer prediction configuration. The bitstream according to the OLS would have to contain all lower layer subpictures which would put a burden on the decoder sample throughput. Furthermore, the VCL NAL units of lower layer subpictures that are not referenced by the higher layer would increase the bitrate of the bitstream.

This document asserts that it is beneficial to have an indication in the bitstream that it is possible to decode the higher layer using only part of the lower layer that is used as reference in a so-called reduced subpicture reference (RSR) mode. Adding such an RSR indication to the OLS signalling would make it accessible for extraction purposes and respective PTL signalling.

The proposed solution requires less decoder resources in terms of sample throughput to decode the RSR mode OLS.

In order to achieve a lower bitrate of the resulting RSR OLS bitstream, compared to keeping the unreferenced subpicture VCL NAL units of the lower layer in the bitstream, it would be required to facilitate the RSR OLS mode in the OLS sub-bitstream extraction as well.

# Proposal

Therefore, this document proposes to

- add an indication to the VPS OLS signalling about the RSR mode

- add PTL signalling for the RSR mode of an OLS

The first part of the proposal is to add three syntax elements to the VPS to indicate that an OLS supports an RSR mode as follows.

It is important to notice that an proper identification of subpictures in a multi-layer bitstream through the subpicture ID requires a bitstream constraint that the subpicture IDs are unique across all layers in the bitstream. Alternatively, it is possible to identify the subpictures required for RPR through an array of layer-specific lists of subpicture IDs.

#### 7.3.2.2	Video parameter set syntax

| video_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| vps_video_parameter_set_id | u(4) |
| […] |  |
| if( vps_max_layers_minus1 > 0 ) { |  |
| if( vps_all_independent_layers_flag ) |  |
| each_layer_is_an_ols_flag | u(1) |
| if( !each_layer_is_an_ols_flag ) { |  |
| if( !vps_all_independent_layers_flag ) |  |
| ols_mode_idc | u(2) |
| if( ols_mode_idc = = 2 ) { |  |
| num_output_layer_sets_minus1 | u(8) |
| for( i = 1; i <= num_output_layer_sets_minus1; i ++) |  |
| for( j = 0; j <= vps_max_layers_minus1; j++ ) |  |
| ols_output_layer_flag[ i ][ j ] | u(1) |
| reduced_subpic_reference_present_flag[ i ] | u(1) |
| if( reduced_subpic_reference_present_flag[ i ] ) |  |
| num_subpic_ids[ i ] | u(8) |
| for( j = 0; j <= num_subpic_ids; j++ ) |  |
| ref_subpic_id[ i ][ j ] | u(v) |
| } |  |
| } |  |
| […] |  |
| } |  |

reduced_subpic_reference_present_flag[ i ] equal to 1 specifies that the output layers of the i-th OLS can be decoded by decoding the VCL NAL units assocaited to the subpictures of the respective reference layers as indicated by the syntax elements num_subpic_ids[ i ] and ref_subpic_id[ i ][ j ]. reduced_subpic_reference_present_flag[ i ] equal to 0 specifies that all VCL NAL units of the indicated reference layers of the output layers of the i-th OLS are required for decoding.

When reduced_subpic_reference_present_flag[ i ] equal to 1 for any index i out of 0 to num_output_layer_sets_minus1 and a subpicutre of layer layerA has the subpicture ID subpicIdA, there exist no subpicture with subpicture ID subpicIdB in any other layer layerB with the value of subpicIdB equal to value of subpicIdA.

num_subpic_ids_minus1[ i ] specifies the number of subpictures required for decoding the output layers of the i-th OLS using reduced subpicture references.

ref_subpic_id[ i ][ j ] specifies the subpictre ID of the j-th subpictures required for decoding the output layers of the i-th OLS using reduced subpicture references.

The second part of the proposal is to add two syntax elements to the VPS to indicate the level conformance of the RSR mode of an OLS as follows.

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| vps_video_parameter_set_id | u(4) |  |
| […] |  |  |
| vps_num_ptls | u(8) |
| for( i = 0; i < vps_num_ptls; i++ ) { |  |  |
| if( i > 0 ) |  |  |
| pt_present_flag[ i ] | u(1) |  |
| if( vps_max_sublayers_minus1 > 0 && !vps_all_layers_same_num_sublayers_flag ) |  |  |
| ptl_max_temporal_id[ i ] | u(3) |  |
| } |  |  |
| while( !byte_aligned( ) ) |  |  |
| vps_ptl_byte_alignment_zero_bit /* equal to 0 */ | u(1) |  |
| for( i = 0; i < vps_num_ptls; i++ ) |  |  |
| profile_tier_level( pt_present_flag[ i ], ptl_max_temporal_id[ i ] ) |  |
| for( i = 0; i < TotalNumOlss; i++ ) |  |  |
| if( NumLayersInOls[ i ] > 1 && vps_num_ptls > 1 ) { |  |  |
| ols_ptl_idx[ i ] | u(8) |  |
| subpic_rsr_ptl_present_flag[ i ] | u(1) |  |
| if( subpic_rsr_ptl_present_flag[ i ] ) |  |  |
| subpic_rsr_ols_ptl_idx[ i ] | u(8) |  |
| } |  |  |
| […] |  |  |
| } |  |  |

subpic_rsr_ptl_present_flag[ i ] equal to 1 specifies that the syntax element subpic_rsr_ols_ptl_idx[ i ] is present for the i-th OLS.

subpic_rsr_ols_ptl_idx[ i ] specifies the index, to the list of profile_tier_level( ) syntax structures in the VPS, of the profile_tier_level( ) syntax structure that applies to the reduced subpicture reference mode of the i-th OLS. When present, the value of subpic_rsr_ols_ptl_idx[ i ] shall be in the range of 0 to vps_num_ptls − 1, inclusive.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)