| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q408-v1 |
| --- | --- |

| Title: | On the combination of JCCR and TS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Bappaditya Ray, Geert Van der Auwera, Marta Karczewicz 5775 Morehouse Drive San Diego, CA 92121, USA | Email: | bray@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

Current VTM software (VTM-7.0) does not perform encoder search of JCCR in combination with ChromaTS, as only DCT2 combination is checked for JCCR, although VVC WD allows this combination. This contribution proposes two ways to fix this: 1. enabling encoder search of JCCR in combination with ChromaTS (non-normative) 2. Disabling chromaTS and JCCR combination (normative). CTC results on YUV420 and YUV444 content, for both proposals, over VTM-7.0, are as follows.

Table1.a: CTC results (YUV420) – Proposal 1

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | 0.00% | -0.04% | -0.03% | 99% | 98% |
| RA | 0.00% | -0.05% | 0.02% | 99% | 98% |
| LD | -0.02% | -0.07% | -0.14% | 98% | 97% |

Table1.b: CTC results (YUV444) – Proposal 1

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | -0.11% | -0.17% | -0.19% | 100% | 101% |
| RA | -0.06% | -0.21% | -0.20% | 100% | 99% |
| LD | -0.13% | -0.27% | -0.20% | 99% | 99% |

Table2.a: CTC results (YUV420) – Proposal 2

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | 0.00% | 0.01% | 0.00% | 100% | 100% |
| RA | 0.00% | -0.04% | 0.03% | 100% | 101% |
| LD |  |  |  |  |  |

Table2.b: CTC results (YUV444) – Proposal 2

| Config. | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| AI | -0.01% | -0.01% | 0.00% | 100% | 100% |
| RA | -0.01% | 0.00% | 0.01% | 101% | 99% |
| LD | -0.07% | -0.18% | -0.04% | 100% | 101% |

# Introduction

Due to the inclusion of Chroma Transform Skip (ChromaTS) last meeting, chroma components currently have two options for MTS: DCT2 (mts_idx = 0) and ChromaTS (mts_idx=1). The current VVC Draft allows joint coding of chroma residuals (JCCR) both for DCT2 and Chroma TS [1]. However, in VTM-7.0, the encoder search for JCCR utilizes only DCT2 but not ChromaTS. The following snippet of VTM-7.0 code in IntraSearch.cpp indicates it. Similar can be seen in InterSearch.cpp also.

if ( cs.sps->getJointCbCrEnabledFlag() && (TU::getCbf(tmpTU, COMPONENT_Cb) || TU::getCbf(tmpTU, COMPONENT_Cr)))

{

jointCbfMasksToTest = m_pcTrQuant->selectICTCandidates(currTU, orgResiCb, orgResiCr);

}

for( int cbfMask : jointCbfMasksToTest )

{

Distortion distTmp = 0;

currTU.jointCbCr               = (uint8_t)cbfMask;

currTU.compAlpha[COMPONENT_Cb] = 0;

currTU.compAlpha[COMPONENT_Cr] = 0;

#if JVET_P0058_CHROMA_TS

// encoder bugfix: initialize mtsIdx for chroma under JointCbCrMode.

currTU.mtsIdx[COMPONENT_Cb] = currTU.mtsIdx[COMPONENT_Cr]  = MTS_DCT2_DCT2;

#endif

m_CABACEstimator->getCtx() = ctxStartTU;

resiCb.copyFrom( orgResiCb[cbfMask] );

resiCr.copyFrom( orgResiCr[cbfMask] );

xIntraCodingTUBlock( currTU, COMPONENT_Cb, false, distTmp, 0 );

double costTmp = std::numeric_limits<double>::max();

if( distTmp < std::numeric_limits<Distortion>::max() )

{

uint64_t bits  = xGetIntraFracBitsQTChroma( currTU, COMPONENT_Cb );

costTmp = m_pcRdCost->calcRdCost( bits, distTmp );

}

if( costTmp < bestCostCbCr )

{

bestCostCbCr  = costTmp;

bestDistCbCr  = distTmp;

bestJointCbCr = currTU.jointCbCr;

// store data

if( cbfMask != jointCbfMasksToTest.back() )

{

#if KEEP_PRED_AND_RESI_SIGNALS

saveCS.getOrgResiBuf(cbArea).copyFrom(cs.getOrgResiBuf(cbArea));

saveCS.getOrgResiBuf(crArea).copyFrom(cs.getOrgResiBuf(crArea));

#endif

saveCS.getPredBuf   (cbArea).copyFrom(cs.getPredBuf   (cbArea));

saveCS.getPredBuf   (crArea).copyFrom(cs.getPredBuf   (crArea));

if( keepResi )

{

saveCS.getResiBuf (cbArea).copyFrom(cs.getResiBuf   (cbArea));

saveCS.getResiBuf (crArea).copyFrom(cs.getResiBuf   (crArea));

}

saveCS.getRecoBuf   (cbArea).copyFrom(cs.getRecoBuf   (cbArea));

saveCS.getRecoBuf   (crArea).copyFrom(cs.getRecoBuf   (crArea));

tmpTU.copyComponentFrom(currTU, COMPONENT_Cb);

tmpTU.copyComponentFrom(currTU, COMPONENT_Cr);

ctxBest = m_CABACEstimator->getCtx();

}

else

{

lastIsBest = true;

}

}

}

# Proposal 1

In VTM-7.0, encoder search for JCCR is performed after the encoder search of independent cb and cr residual coding. JCCR search is invoked depending on the best independent coding of cb and cr component, i.e., when either cb or cr component has a nonzero CBF, JCCR search is enabled.

Now, a straightforward way to enable JCCR search also for ChromaTS would be to do another RD search with ChromaTS (in addition to DCT2). However, it may increase the encoding runtime.

In this proposal, we have kept single RD search (same as VTM-7.0) for JCCR. However, the decision whether to perform the search with DCT2 or ChromaTS is taken from the MTS index of the best independent coding of cb and cr component, as described in Table 3.

Table 3: MTS selection for JCCR search

| bestTU[Cb] | bestTU[Cr] | MTS selection for JCCR |
| --- | --- | --- |
| DCT2 | - | DCT2 |
| DCT2 | DCT2 | DCT2 |
| - | DCT2 | DCT2 |
| TS | - | TS |
| - | TS | TS |
| TS | TS | TS |
| TS | DCT2 | - |
| DCT2 | TS | - |
| - | - | - |

As shown in Table 3, JCCR search is avoided when the MTS index of best independent coding of cb and cr component is different, i.e., when one is using DCT2 and other is using ChromaTS. The motivation here is that JCCR assumes the residual of cb and cr component is similar (atleast structure-wise, one can be a scaled version of another although). Different MTS index (DCT2 or ChormaTS) suggests that the residual characteristics of cb and cr are different, and thus most probably JCCR will not be used. For the rest combinations, for JCCR, DCT2 is checked when either of the component use DCT2 for best independent coding, and similarly ChromaTS is checked when either of the component use ChromaTS for best independent coding. This same strategy is used both in IntraSearch.cpp and InterSearch.cpp, respectively for the encoder search of Intra and Inter TU.

# Proposal 2

In this proposal, the combination of chromaTS and JCCR is disallowed. Thus, when JCCR is enabled for a TU (for cb and cr component), transform_skip flag for chroma is not parsed, and inferred to be 0.

| if( tu_cbf_cb[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| --- | --- |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag && !tu_joint_cbcr_residual_flag[ xC ][ yC ] ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cbf_cr[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cbf_cb[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag && !tu_joint_cbcr_residual_flag[ xC ][ yC ] ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |

# Experimental results

The proposed methods are implemented on top of VTM-7.0 and tested with YUV420 and YUV444 content [2]. For YUV444 content-based test, IBC, PLT, BDPCM (both for luma and chroma) are enabled. For YUV420 content-based test, for ClassF and ClassTGM, IBC, PLT, and luma BDPCM is enabled. For both YUV420 and YUV444, dual tree is enabled for I Slices. The QP’s tested are (22, 27, 32, 37).

Proposal 1:

Table 4.a: CTC results (YUV420) – Proposal 1

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.05% | 0.05% | 100% | 103% |
| Class A2 | 0.00% | -0.05% | -0.06% | 98% | 97% |
| Class B | 0.00% | -0.03% | -0.02% | 100% | 100% |
| Class C | 0.00% | -0.06% | -0.10% | 98% | 95% |
| Class E | 0.00% | -0.04% | -0.04% | 98% | 94% |
| Overall | 0.00% | -0.04% | -0.03% | 99% | 98% |
| Class D | -0.02% | -0.11% | -0.08% | 100% | 101% |
| Class F | 0.04% | -0.24% | -0.30% | 100% | 101% |
| TGM | -0.08% | -0.35% | -0.38% | 103% | 82% |
|  | Random Access |  |  |  |  |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.02% | -0.05% | -0.01% | 101% | 101% |
| Class A2 | -0.01% | -0.05% | 0.02% | 98% | 100% |
| Class B | 0.02% | 0.02% | 0.06% | 100% | 99% |
| Class C | -0.01% | -0.12% | -0.01% | 98% | 93% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.05% | 0.02% | 99% | 98% |
| Class D | 0.01% | -0.24% | -0.25% | 101% | 101% |
| Class F | -0.05% | -0.23% | -0.32% | 99% | 99% |
| TGM | -0.05% | -0.40% | -0.32% | 95% | 88% |
|  | Low delay B |  |  |  |  |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.21% | -0.26% | 99% | 97% |
| Class C | -0.01% | -0.60% | -0.22% | 99% | 98% |
| Class E | -0.05% | 0.87% | 0.17% | 97% | 96% |
| Overall | -0.02% | -0.07% | -0.14% | 98% | 97% |
| Class D | 0.04% | 0.04% | -0.46% | 101% | 104% |
| Class F | 0.03% | -0.81% | 0.23% | 102% | 104% |
| TGM | 0.01% | -0.39% | -0.25% | 102% | 103% |

Table 4.b: Results (YUV444) – Proposal 1

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.18% | -0.16% | -0.21% | 100% | 104% |
| TGM 720p | -0.04% | -0.02% | -0.22% | 100% | 101% |
| Animation | -0.18% | -0.35% | -0.35% | 99% | 100% |
| Mixed content | -0.11% | -0.23% | -0.06% | 100% | 100% |
| Camera-Captured content | 0.00% | -0.13% | -0.05% | 101% | 101% |
| Overall | -0.11% | -0.17% | -0.19% | 100% | 101% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-7.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.15% | -0.24% | -0.25% | 99% | 99% |
| TGM 720p | -0.05% | -0.06% | -0.25% | 100% | 101% |
| Animation | -0.05% | -0.45% | -0.38% | 99% | 96% |
| Mixed content | 0.02% | -0.22% | 0.03% | 101% | 100% |
| Camera-Captured content | -0.01% | -0.03% | -0.05% | 100% | 99% |
| Overall | -0.06% | -0.21% | -0.20% | 100% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM-7.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.18% | -0.36% | -0.25% | 99% | 99% |
| TGM 720p | -0.16% | -0.48% | -0.15% | 100% | 99% |
| Animation | -0.10% | -0.20% | -0.27% | 99% | 99% |
| Mixed content | -0.17% | -0.05% | -0.05% | 100% | 98% |
| Camera-Captured content | 0.05% | -0.07% | -0.29% | 98% | 99% |
| Overall | -0.13% | -0.27% | -0.20% | 99% | 99% |

For YUV420 content, almost no performance benefit is observed (except for LDB config., and for ClassTGM sequences), as ChromaTS is particularly useful for screen content and at lower QPs. For YUV444 content, moderate performance benefit can be observed. The encoding and decoding runtime of the proposed method is the same as VTM-7.0.

Proposal 2:

Table 5.a: CTC results (YUV420) – Proposal 2

|  | All Intra |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | 0.02% | 100% | 100% |
| Class A2 | -0.01% | -0.02% | -0.02% | 100% | 97% |
| Class B | 0.00% | 0.03% | -0.01% | 100% | 100% |
| Class C | 0.01% | 0.01% | -0.01% | 100% | 102% |
| Class E | 0.00% | 0.01% | 0.03% | 100% | 100% |
| Overall | 0.00% | 0.01% | 0.00% | 100% | 100% |
| Class D | 0.01% | -0.01% | -0.03% | 94% | 96% |
| Class F | -0.01% | 0.01% | -0.01% | 99% | 100% |
| TGM | -0.03% | -0.03% | -0.04% | 99% | 99% |
|  | Random Access |  |  |  |  |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.15% | 0.00% | 100% | 100% |
| Class A2 | -0.01% | -0.04% | 0.10% | 101% | 101% |
| Class B | 0.00% | 0.03% | 0.04% | 100% | 100% |
| Class C | 0.03% | -0.04% | -0.04% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.04% | 0.03% | 100% | 101% |
| Class D | 0.00% | -0.22% | -0.16% | 101% | 101% |
| Class F | -0.02% | 0.02% | -0.02% | 100% | 100% |
| TGM | -0.05% | -0.08% | -0.04% | 101% | 100% |
|  | Low delay B |  |  |  |  |
|  | Over VTM 7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.00% | -0.24% | -0.18% | 94% | 89% |
| Class E | 0.02% | 0.35% | -0.15% | 95% | 95% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.02% | 0.13% | -0.26% | 102% | 100% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

Table 5.b: Results (YUV444) – Proposal 2

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.02% | -0.02% | -0.01% | 99% | 101% |
| TGM 720p | -0.01% | -0.03% | -0.01% | 100% | 99% |
| Animation | -0.02% | 0.03% | 0.02% | 99% | 100% |
| Mixed content | 0.01% | 0.02% | 0.02% | 100% | 100% |
| Camera-Captured content | 0.01% | -0.07% | -0.06% | 101% | 102% |
| Overall | -0.01% | -0.01% | 0.00% | 100% | 100% |
|  |  |  | Random access Main10 |  |  |
|  | Over VTM-7.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | 0.00% | 0.01% | 0.00% | 100% | 99% |
| TGM 720p | -0.02% | 0.03% | 0.05% | 101% | 101% |
| Animation | -0.03% | -0.03% | -0.06% | 100% | 97% |
| Mixed content | 0.01% | 0.00% | 0.06% | 101% | 100% |
| Camera-Captured content | 0.01% | -0.04% | -0.04% | 101% | 100% |
| Overall | -0.01% | 0.00% | 0.01% | 101% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  | Over VTM-7.0_PLT1_IBC1_DT1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| TGM 1080p | -0.04% | -0.09% | 0.01% | 98% | 100% |
| TGM 720p | -0.14% | -0.30% | 0.04% | 101% | 100% |
| Animation | -0.07% | -0.12% | -0.15% | 100% | 102% |
| Mixed content | -0.03% | -0.29% | -0.03% | 101% | 100% |
| Camera-Captured content | -0.02% | -0.05% | -0.16% | 101% | 109% |
| Overall | -0.07% | -0.18% | -0.04% | 100% | 101% |

It can be observed by comparing Table 4.b and 5.b that the proposal 1 provides better performance for YUV444 content, i.e., the combination of TS and JCCR shows some benefits.

# Conclusion

This proposal investigates the misalignment between VVC WD and VTM-7.0 for TS and JCCR combination. Proposal 1 enables encoder search of JCCR in combination with ChromaTS, while for proposal 2, this combination is disabled normatively. For YUV420 content, no performance benefit is observed especially for CTC sequences, while for YUV444 content, proposal 1 performs better than proposal 2.

# References

[1] Bross et al., “Versatile Video Coding (Draft 7),” Joint Video Experts Team, doc. JVET-P2001, Geneva, CH, Oct. 2019.

[2] F. Bossen, J. Boyce, X. Li, V. Seregin, and K. Sühring, “JVET common test conditions and software reference configurations”, Document of Joint Video Experts Team, JVET-N1010, Geneva, CH, Mar. 2019.

# Patent rights declaration

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).