| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0409_r1 |
| --- | --- |

| Title: | AHG12: On APS id for bitstream merging for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nael Ouedraogo Eric Nassor Franck Denoual Frederic Maze | Email: | nael.ouedraogo@crf.canon.fr eric.nassor@crf.canon.fr franck.denoual @crf.canon.fr frederic.maze@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution is a continuation of JVET-N0191 that was noted as “further study” at JVET#14 meeting. It proposes to add an optional extension identifier (aps_extension_id) in the APS and to associate a subpicture extension identifier (subpicture_extension_id) with subpictures and their slices. It is reported that the new identifiers simplify handling of APS identifier when combining or merging subpictures originated from different bitstreams. In particular, it avoids rewriting the APS identifier specified in the slice headers in the merged bitstream. It is asserted that the merging at Systems level would be simplified since it would mainly consist in modifying Parameter Set NAL units (i.e. no VCL NAL units would have to be modified).

First revision of this document contains the following modifications:

- Removal of the signaling of subpicture extension identifiers at Picture Header (PH) level following the decision to adopt the removal of subpicture ID mapping from PH.

- Add a constraint for the number of referenced ALF APSs per PU to the same number as in current VVC specification

Second revision of this document adds a second proposal (see section 3) that relies on unused bits of the APS ALF identifier to specify the extension identifier in the APS, letting the APS syntax unchanged;

# Introduction

During the creation of a new video stream with regions taken from different video streams (for OMAF viewport dependent streaming for instance), same APS identifiers are likely to be used in the video streams. Indeed, the APS identifier is a value between 0 and 31 and it may change at each frame. The figure below illustrates the generation of a new bitstream from two input bitstreams.

Figure 1: Slices and bitstream merging

In this example, we want to merge the slice of the subpicture 3 from first video and the slices of the subpictures 1, 2 and 4 from second video in a new video (see Figure 1). It is possible for the slice of the subpicture  3 to reference a first APS with id 0 in first video and for slice of the subpicture 4 to reference a second different APS with same id 0 in the second video. In this case, in order to create the merged video, we will need to change the id of one of the APS and to change also the header of one of the slice (or/and the picture header) to change the reference during the rewriting process.

The first operation (changing the APS id) requires reading the non-VCL NAL units in the video streams, decoding the NAL unit headers and rewriting the APS units. This operation is not too complex because the non-VCL NAL units are relatively small and the headers are mostly encoded with fixed length values. But we also need to read all the VCL NAL units to modify the APS id inside the slice headers. This second operation is complex because it requires reading all the video bitstream, decoding all the slice headers which contain many variable length fields and rewriting the complete bitstream.

# Proposal : Option 1

A better structure for the APS would allow an efficient merge of several videos allowing easy resolution of APS id conflicts without requiring modification of the VCL NAL units (and in particular the slice headers).

## Signaling of the extension identifiers for the subpictures

### Sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| […] |  |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| sps_log2_ctu_size_minus5 | u(2) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| sps_subpic_aps_extension_id_present_flag | u(1) |
| if( sps_subpic_aps_extension_id_present_flag ) { |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_signalled_aps_extension_id_flag[ i ] | u(1) |
| if(sps_subpic_signalled_aps_extension_id_flag[ i ]) |  |
| sps_subpic_aps_extension_id[ i ] | u(7) |
| } |  |
| } |  |
| […] |  |

sps_subpic_aps_extension_id_present_flag equal to 1 specifies that APS extension ID is present in the SPS. sps_subpic_aps_extension_id_present_flag equal to 0 specifies the absence of the APS extension ID in the SPS.

sps_subpic_signalled_aps_extension_id_flag[ i ] equal to 1 specifies the presence of sps_subpic_signalled_aps_extension_id[ i ] for the i-th subpicture in the SPS. sps_subpic_signalled_aps_extension_id_flag[ i ] equal to 0 specifies the absence of the sps_subpic_signalled_aps_extension_id[ i ] in the SPS for the i-th subpicture.

sps_subpic_aps_extension_id[ i ]specifies the extension ID of the i-th subpicture for APS, when present. When not present, the sps_subpic_aps_extension_id[ i ] is inferred equal to 0, for each i in the range of 0 to sps_num_subpics_minus1inclusive.

### Picture Parameter Set syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| […] |  |
| pps_subpic_id_signalling_present_flag | u(1) |
| pps_subpic_aps_extension_id_present_flag | u(1) |
| if(pps_subpic_id_signalling_present_flag ¦ ¦ pps_subpic_aps_extension_id_present_flag) | u(1) |
| pps_num_subpics_minus1 | ue(v) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| if(pps_subpic_aps_extension_id_present_flag ) { |  |
| for( i = 0; i <= pps_num_subpics_minus1; i++ ) |  |
| pps_subpic_signalled_aps_extension_id_flag[ i ] | u(1) |
| if(pps_subpic_signalled_aps_extension_id_flag[ i ]) |  |
| pps_subpic_aps_extension_id[ i ] | u(7) |
| } |  |
| } |  |

pps_subpic_aps_extension_id_present_flag equal to 1 specifies that APS extension ID is present in the PPS. pps_subpic_aps_extension_id_present_flag equal to 0 specifies the absence of the APS extension ID in the PPS. When sps_subpic_aps_extension_id_present_flag is 1, pps_subpic_aps_extension_id_present_flag shall be equal to 0.

pps_subpic_signalled_aps_extension_id_flag[ i ] equal to 1 specifies the presence of pps_subpic_aps_extension_id[ i ] for the i-th subpicture in the PPS. pps_subpic_signalled_aps_extension_id_flag[ i ] equal to 0 specifies the absence of the pps_subpic_aps_extension_id[ i ] in the PPS for the i-th subpicture.

pps_subpic_aps_extension_id[ i ] specifies the extension ID of the i-th subpicture for APS, when present. When not present, the pps_subpic_aps_extension_id[ i ] is inferred equal to 0, for each i in the range of 0 to pps_num_subpics_minus1inclusive.

The list SubpicAPSExtensionIdList[ i ] is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	SubpicAPSExtensionIdList [ i ] =
		sps_subpic_aps_extension_id_present_flag ? sps_subpic_aps_extension_id[ i ] :
		( pps_subpic_aps_extension_id_present_flag? pps_subpic_aps_extension_id[ i ] : 0 )

### Adaptation parameter set syntax

In order to solve the problem of conflicting APS id when merging videos, we propose to add a new ID (the aps_extension_id called APS extension id) inside the APS.

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| aps_ id_extension_flag | u(1) |
| if(aps_id_extension_flag) { |  |
| aps_extension_id | u(7) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

aps_id_extension_flag equal to 1 specifies the presence of aps_extension_id in the APS. aps_extension_flag equal to 0 specifies the absence of the aps_extension_id.

aps_extension_id when present, provides extended identifier for the APS to use as reference by other syntax elements. The value of aps_extension_id shall be in the range of 0 to 127. When not present, the value of aps_extension_id is inferred to be equal 0.

## Referring to APS signalling

### General slice header semantics

No modification of the slice header syntax is contemplated. The presence of the APS identifier when combined with the APS extension identifier obtained from the PPS allows determining the right APS in all configurations.

In current VVC specification, the APS id shall be the same for all slices of a picture for LMCS and Scaling lists. As a result, the extension identifier is used only for ALF parameters.

The semantics of some syntax elements of the slice header are updated as follows:

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slice refers to.

The i-th ALF APS that the slice refers to is the APS NAL unit having adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] and having the aps_extension_id equal to  SubpicAPSExtensionIdList[ SubPicIdx ].

The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and, adaptation_parameter_set_id equal to slice_alf_aps_id_luma[ i ] and the aps_extension_id equal to  SubpicAPSExtensionIdList[ SubPicIdx ] shall be less than or equal to the TemporalId of the coded slice NAL unit.

When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_luma[ i ] is not present, the value of slice_alf_aps_id_luma[ i ] is inferred to be equal to the value of pic_alf_aps_id_luma[i]

slice_alf_chroma_idc (unchanged)

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slice refers to.

The APS that the slice refers to is the APS NAL unit having adaptation_parameter_set_id equal to slice_alf_aps_id_chroma and having the aps_extension_id equal to  SubpicAPSExtensionIdList[ SubPicIdx ].

The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and, adaptation_parameter_set_id equal to slice_alf_aps_id_chroma and the aps_extension_id equal to  SubpicAPSExtensionIdList[ SubPicIdx ] shall be less than or equal to the TemporalId of the coded slice NAL unit.

When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_chroma is not present, the value of slice_alf_aps_id_chroma is inferred to be equal to the value of pic_alf_aps_id_chroma.

### Picture Header semantics

pic_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slices associated with the PH refers to.

The i-th ALF APS that the slice associated with the PH refers to is the APS NAL unit having adaptation_parameter_set_id equal to pic_alf_aps_id_luma[ i ] and having the aps_extension_id equal to  SubpicAPSExtensionIdList[ SubPicIdx ].

The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS, adaptation_parameter_set_id equal to pic_alf_aps_id_luma[ i ] and having the aps_extension_id equal to SubpicAPSExtensionIdList[ SubPicIdx ] shall be equal to 1.

pic_alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. pic_alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. pic_alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. pic_alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components. When pic_alf_chroma_idc is not present, it is inferred to be equal to 0.

pic_alf_aps_id_chroma specifies the adaptation_parameter_set_id of the ALF APS that the chroma component of the slices associated with the PH refers to.

The ALF APS that the slice associated with the PH refers to is the APS NAL unit having adaptation_parameter_set_id equal to pic_alf_aps_id_chroma and having the aps_extension_id equal to  SubpicAPSExtensionIdList[ SubPicIdx ].

The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS, adaptation_parameter_set_id equal to pic_alf_aps_id_chroma and having the aps_extension_id equal to  SubpicAPSExtensionIdList[ SubPicIdx ] shall  be equal to 1.

## Constraining the total number of ALF APSs per PU

The total number of ALF APSs referred to by all slices of a PU is constrained to the same number as in current VVC specification with the following modifications in APS semantics:

All APS NAL units with a particular value of adaptation_parameter_set_id, and a particular value of aps_params_type and a particular value of aps_extension _id within a PU, regardless of whether they are prefix or suffix APS NAL units, shall have the same content.

The following modifications in the semantics of slice_alf_aps_id_luma[ i ] and  slice_alf_aps_id_chroma ensure this constraint:

The total number of ALF APSs with a particular value of adaptation_parameter_set_id and a particular value of aps_extension _id that all slices of the coded picture refers to shall be less or equal than 8.

## Example

Figure 3: Illustration of aps extension to solve APS_id conflicts in bitstream extraction and merging

The figure above is an application example of the proposal. As in previous example, two bitstreams with respective slice of subpictures 3 and 4 to be merged, each referring to an APS with id equal 0.

In the resulting bitstream after rewriting, both slice NAL units of subpictures 3 and 4 are unmodified and continue to refer to the associated APS using the APS identifier with a value of 0. What changed is that now, the APS originated from the first bitstream specifies both an APS identifier with a value 0 and an APS extension identifier with a value of 0 corresponding to the specified subpicture extension Id for its SubPicture in the SPS or PPs. The APS from the second bitstream comprises an APS identifier with a value 0 and an APS extension identifier with a value 1, corresponding to the specified subpicture extension Id for its SubPicture in the SPS or PPS.

At decoding, the decoder is therefore able to decode the slice of the subpicture 3 with the correct identification of the associated APS based on the APS identifier stored in the slice of the subpicture 3 and the APS extension identifier associated to the subpicture 3. The same is true for the decoding of the slice of the subpicture 4.

It is to be noted that this mechanism works even if the APS identifier changes from one picture to another for the slices in subsequent picture. The APS extension identifier will stay the same, still allowing the identification of the correct APS.

This proposal fixes the APS identifier collisions while keeping the slice structure intact, thus simplifying the merge process of slices from different original bitstreams.

# Proposal: Option 2

This second proposal uses the same syntax elements as in Option 1 for the SPS and PPS and lets the APS syntax unchanged. Additional elements compared to Option 1 are highlighted in green in the following sections. Parts highlighted in yellow are proposed modifications to current VVC specification.

A new flag (sps_subpic_aps_extension_id_enabled_flag) in the SPS indicates if the extension IDs are signalled (in the SPS or the PPS) or not signalled.

The extension identifier of the APS is inferred from the 2 most significant bits of adaptation_parameter_set_id. The APS identifier corresponds to the 3 least significant bits of the  adaptation_parameter_set_id.

## Signaling of the extension identifiers for the subpictures

### Sequence parameter set syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_decoding_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| […] |  |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| sps_log2_ctu_size_minus5 | u(2) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpics_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpics_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| sps_subpic_aps_extension_id_enabled_flag | u(1) |
| if (sps_subpic_aps_extension_id_enabled_flag) { |  |
| sps_subpic_aps_extension_id_present_flag | u(1) |
| if( sps_subpic_aps_extension_id_present_flag ) { |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_signalled_aps_extension_id_flag[ i ] | u(1) |
| if(sps_subpic_signalled_aps_extension_id_flag[ i ]) |  |
| sps_subpic_aps_extension_id[ i ] | u(2) |
| } |  |
| } |  |
| } |  |
| […] |  |

sps_subpic_aps_extension_id_enabled_flag equal to 1 specifies that APS extension ID is present either in the SPS or in the PPS that refers to the SPS. sps_subpic_aps_extension_id_enabled _flag equal to 0 specifies the absence of the APS extension ID in the SPS and in the PPS that refers to the SPS.

sps_subpic_aps_extension_id_present_flag equal to 1 specifies that APS extension ID is present in the SPS. sps_subpic_aps_extension_id_present_flag equal to 0 specifies the absence of the APS extension ID in the SPS.

sps_subpic_signalled_aps_extension_id_flag[ i ] equal to 1 specifies the presence of sps_subpic_signalled_aps_extension_id[ i ] for the i-th subpicture in the SPS. sps_subpic_signalled_aps_extension_id_flag[ i ] equal to 0 specifies the absence of the sps_subpic_signalled_aps_extension_id[ i ] in the SPS for the i-th subpicture.

sps_subpic_aps_extension_id[ i ]specifies the extension ID of the i-th subpicture for APS, when present. When not present, the sps_subpic_aps_extension_id[ i ] is inferred equal to 0, for each i in the range of 0 to sps_num_subpics_minus1inclusive.

### Picture Parameter Set syntax

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| pps_pic_parameter_set_id | ue(v) |
| pps_seq_parameter_set_id | u(4) |
| […] |  |
| pps_subpic_id_signalling_present_flag | u(1) |
| pps_subpic_aps_extension_id_present_flag | u(1) |
| if(pps_subpic_id_signalling_present_flag ¦ ¦ pps_subpic_aps_extension_id_present_flag) | u(1) |
| pps_num_subpics_minus1 | ue(v) |
| if( pps_subpics_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| if(pps_subpic_aps_extension_id_present_flag ) { |  |
| for( i = 0; i <= pps_num_subpics_minus1; i++ ) |  |
| pps_subpic_signalled_aps_extension_id_flag[ i ] | u(1) |
| if(pps_subpic_signalled_aps_extension_id_flag[ i ]) |  |
| pps_subpic_aps_extension_id[ i ] | u(2) |
| } |  |
| } |  |

pps_subpic_aps_extension_id_present_flag equal to 1 specifies that APS extension ID is present in the PPS. pps_subpic_aps_extension_id_present_flag equal to 0 specifies the absence of the APS extension ID in the PPS. When sps_subpic_aps_extension_id_present_flag is 1, pps_subpic_aps_extension_id_present_flag shall be equal to 0. It is a requirement of the bitstream that pps_subpic_aps_extension_id_present_flag equal 0 when  sps_subpic_aps_extension_id_enabled_flag equal to 0.

pps_subpic_signalled_aps_extension_id_flag[ i ] equal to 1 specifies the presence of pps_subpic_aps_extension_id[ i ] for the i-th subpicture in the PPS. pps_subpic_signalled_aps_extension_id_flag[ i ] equal to 0 specifies the absence of the pps_subpic_aps_extension_id[ i ] in the PPS for the i-th subpicture.

pps_subpic_aps_extension_id[ i ] specifies the extension ID of the i-th subpicture for APS, when present. When not present, the pps_subpic_aps_extension_id[ i ] is inferred equal to 0, for each i in the range of 0 to pps_num_subpics_minus1inclusive.

The list SubpicAPSExtensionIdList[ i ] is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	SubpicAPSExtensionIdList [ i ] =
		sps_subpic_aps_extension_id_present_flag ? sps_subpic_aps_extension_id[ i ] :
		( pps_subpic_aps_extension_id_present_flag? pps_subpic_aps_extension_id[ i ] : 0 )

### Adaptation parameter set syntax

In order to solve the problem of conflicting APS id when merging videos, we propose to reuse unused bits in the adaptation_parameter_set_id syntax element of ALF APS to infer the value of the extension ID.

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| if( aps_params_type = = ALF_APS ) |  |
| alf_data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

The variable apsId specifying the APS Id of the ALF APS and the variable apsExtId specifying the extension id of the ALF APS are derived as follows:

- if sps_subpic_aps_extension_id_enabled_flag equal 0, apsId is equal to adaptation_parameter_set_id and apsExtId is equal to 0.

- otherwise, apsId  and apsExtId are derived as follows

apsId = adaptation_parameter_set_id & 0x07
apsExtId = adaptation_parameter_set_id >> 3

## Referring to APS signalling

### General slice header semantics

No modification of the slice header syntax is contemplated. The presence of the APS identifier when combined with the APS extension identifier obtained from the PPS allows determining the right APS in all configurations.

In current VVC specification, the APS id shall be the same for all slices of a picture for LMCS and Scaling lists. As a result, the extension identifier is used only for ALF parameters.

The semantics of some syntax elements of the slice header are updated as follows:

slice_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_idapsId of the i-th ALF APS that the luma component of the slice refers to.

The i-th ALF APS that the slice refers to, is the APS NAL unit having apsId equal to slice_alf_aps_id_luma[ i ] and having the apsExtId equal to  SubpicAPSExtensionIdList[ SubPicIdx ].

The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and, adaptation_parameter_set_id apsId equal to slice_alf_aps_id_luma[ i ] and the apsExtId equal to  SubpicAPSExtensionIdList[ SubPicIdx ] shall be less than or equal to the TemporalId of the coded slice NAL unit.

When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_luma[ i ] is not present, the value of slice_alf_aps_id_luma[ i ] is inferred to be equal to the value of pic_alf_aps_id_luma[i]

slice_alf_chroma_idc (unchanged)

slice_alf_aps_id_chroma specifies the adaptation_parameter_set_idapsId of the ALF APS that the chroma component of the slice refers to.

The APS that the slice refers to is the APS NAL unit having adaptation_parameter_set_idapsId equal to slice_alf_aps_id_chroma and having the apsExtId equal to  SubpicAPSExtensionIdList[ SubPicIdx ].

The TemporalId of the APS NAL unit having aps_params_type equal to ALF_APS and, adaptation_parameter_set_idapsId equal to slice_alf_aps_id_chroma and apsExtId equal to  SubpicAPSExtensionIdList[ SubPicIdx ] shall be less than or equal to the TemporalId of the coded slice NAL unit.

When slice_alf_enabled_flag is equal to 1 and slice_alf_aps_id_chroma is not present, the value of slice_alf_aps_id_chroma is inferred to be equal to the value of pic_alf_aps_id_chroma.

### Picture Header semantics

pic_alf_aps_id_luma[ i ] specifies the adaptation_parameter_set_idapsId of the i-th ALF APS that the luma component of the slices associated with the PH refers to.

The i-th ALF APS that the slice associated with the PH refers to is the APS NAL unit having adaptation_parameter_set_idapsId equal to pic_alf_aps_id_luma[ i ] and having the apsExtId equal to  SubpicAPSExtensionIdList[ SubPicIdx ].

The value of alf_luma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS, adaptation_parameter_set_idapsId equal to pic_alf_aps_id_luma[ i ] and having the apsExtId equal to SubpicAPSExtensionIdList[ SubPicIdx ] shall be equal to 1.

pic_alf_chroma_idc equal to 0 specifies that the adaptive loop filter is not applied to Cb and Cr colour components. pic_alf_chroma_idc equal to 1 indicates that the adaptive loop filter is applied to the Cb colour component. pic_alf_chroma_idc equal to 2 indicates that the adaptive loop filter is applied to the Cr colour component. pic_alf_chroma_idc equal to 3 indicates that the adaptive loop filter is applied to Cb and Cr colour components. When pic_alf_chroma_idc is not present, it is inferred to be equal to 0.

pic_alf_aps_id_chroma specifies the apsId of the ALF APS that the chroma component of the slices associated with the PH refers to.

The ALF APS that the slice associated with the PH refers to is the APS NAL unit having adaptation_parameter_set_idapsId equal to pic_alf_aps_id_chroma and having the apsExtId equal to  SubpicAPSExtensionIdList[ SubPicIdx ].

The value of alf_chroma_filter_signal_flag of the APS NAL unit having aps_params_type equal to ALF_APS, adaptation_parameter_set_idapsId equal to pic_alf_aps_id_chroma and having the apsExtId equal to  SubpicAPSExtensionIdList[ SubPicIdx ] shall  be equal to 1.

## Constraining the total number of ALF APSs per PU

The total number of ALF APSs referred to by all slices of a PU is constrained to the same number as in current VVC specification with the following modifications in APS semantics:

The following modifications in the semantics of slice_alf_aps_id_luma[ i ] and  slice_alf_aps_id_chroma ensure this constraint:

The total number of ALF APSs with a particular value of adaptation_parameter_set_id that all slices of the coded picture refer to shall be less or equal than 8.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).