| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0410 |
| --- | --- |

| Title: | AHG12: Bitstream merging with variable initial Qp |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nael Ouedraogo Eric Nassor Gerald Kergourlay Frederic Maze | Email: | nael.ouedraogo@crf.canon.fr eric.nassor@crf.canon.fr gerald.kergourlay@crf.canon.fr frederic.maze@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution proposes new syntax elements in the PPS that optionally associate a QP offsets value to each subpicture in a picture.

It is reported that these new syntax elements permit to fix QP initialization issues when combining subpictures from bitstreams using different initial QP values.

It is also asserted that bitstreams combination is simplified and complexity reduced by avoiding the rewriting of the headers of the combined slices.

# Introduction

In OMAF, viewport dependent streaming may consist for example in assembling slices from different coded video bitstreams of different qualities to emphasize a specific region with higher quality. This leads to combining slices from bitstreams encoded with different initial QP values. Merging subpictures with different initial QP values actually creates an issue as illustrated in the figure below.

For example, the first video has a high quality and its initial QP is a low value for example 20 (init_qp_minus26 = −6 in PPS1). The second video has a low quality with high value for its initial QP for example 40 (init_qp_minus26 = 14 in PPS2).

When we merge slices of subpicture 3 from the first video (high quality) and slices of subpictures 1, 2 and 4 from the second video (low quality) within a new third video, it is not possible to keep the same slice_qp_delta values in all merged slices. Indeed, they are defined by reference to different PPS with different initial QP values (init_qp_minus26). It is thus necessary to generate a new PPS with a new initial QP value (for example init_qp_minus26 = −6) and to modify the slice_qp_delta values of slices of several subpictures of the video. For example, slices of the subpicture 4 are modified to set slice_qp_delta equal to 20. The first operation of creating a new PPS is not too complex because the non-VCL unit PPS are relatively small and not frequent and the headers are encoded with mostly fixed length values. The second operation of changing slice headers is very complex and time consuming because it requires reading all the bitstreams, decoding all the slice headers which contains many variable length fields and rewriting the complete bitstream. Moreover, we can remark in this example that the value slice_qp_delta for some slices will have a value far different from 0 and thus will use a large number of bits in its variable length encoding. The same issue is also present for the slice values slice_cb_qp_offset, slice_cr_qp_offset and slice_joint_cbcr_qp_offset and for the cb_qp_offset_list, cr_qp_offset_list and joint_cbcr_qp_offset_list

It is thus desirable to have a better encoding for the slice QP to allow an efficient merge of subpictures from different videos allowing an easy resolution of QP conflicts without requiring modification of the VCL units (and in particular the slice headers).

# Proposal

In order to solve the problem described above when merging subpictures from different videos, we propose to associate a QP offset to each subpicture inside the PPS that will be added to the initial QP and offsets values (both for luma and chromas) and to the slice QP delta values.

The syntax of the PPS is modified as follows:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_subpic_id_signalling_present_flag | u(1) |
| pps_subpic_qp_offset_present_flag | u(1) |
| if( pps_subpic_id_signalling_present_flag ¦ ¦ pps_subpic_qp_offset_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| } |  |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpic_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |
| […] |  |
| init_qp_minus26 | se(v) |
| log2_transform_skip_max_size_minus2 | ue(v) |
| cu_qp_delta_enabled_flag | u(1) |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| if (pps_subpic_qp_offset_present_flag) { |  |
| for( i = 0; i <= pps_num_subpics_minus1; i++ ) |  |
| pps_subpic_qp_offset[ i ] | se(v) |
| pps_subpic_cb_qp_offset[ i ] | se(v) |
| pps_subpic_cr_qp_offset[ i ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_subpic_joint_cbcr_qp_offset_value[ i ] | se(v) |
| } |  |
| […] |  |

The semantics of the new syntax element is the following:

pps_subpic_qp_offset_present_flag equal to 1 specifies the presence of pps_subpic_qp_offset[ i ], pps_subpic_cb_qp_offset[ i ], pps_subpic_cr_qp_offset[ i ] and pps_subpic_joint_cbcr_qp_offset_value[ i ]  in the PPS. pps_subpic_qp_offset_present_flag equal to 0 specifies the absence of the pps_subpic_qp_offset[ i ], pps_subpic_cb_qp_offset[ i ], pps_subpic_cr_qp_offset[ i ] and pps_subpic_joint_cbcr_qp_offset_value[ i ].

pps_subpic_qp_offset[ i ] when present, specifies the offset value that applies to the initial value of SliceQpY for the i-th subpicture. The value of 26 + init_qp_minus26 + pps_subpic_qp_offset[ i ] shall be in the range of −QpBdOffsetY to +63, inclusive. When not present, the value of pps_subpic_qp_offset[ i ] is inferred to be equal 0 for each value of i in range of 0 to pps_num_subpic_minus1, inclusive.

pps_subpic_cb_qp_offset[ i ] and pps_subpic_cr_qp_offset[ i ] specify  differences to be added to the values of pps_cb_qp_offset and pps_cr_qp_offset when determining the value of the quantization parameters Qp′Cb and Qp′Cr respectively for the i-th subpicture. The values of pps_subpic_cb_qp_offset[ i ] and pps_subpic_cr_qp_offset[ i ] shall be in the range of −12 to +12, inclusive. When not present, the values of pps_subpic_cb_qp_offset[ i ] and pps_subpic_cr_qp_offset[ i ] are inferred to be equal to 0. The values of pps_cb_qp_offset + pps_subpic_cb_qp_offset[ i ] and pps_cr_qp_offset + pps_subpic_cr_qp_offset[ i ] shall be in the range of −12 to +12, inclusive.

pps_subpic_joint_cbcr_qp_offset_value[ i ] specifies the difference to be added to the value of pps_joint_cbcr_qp_offset when determining the value of the quantization parameter Qp′CbCr for the i–th subpicture. The value of pps_subpic_joint_cbcr_qp_offset_value[ i ] shall be in the range of −12 to +12, inclusive. When not present, the value of pps_subpic_joint_cbcr_qp_offset_value[ i ]  is inferred to be equal to 0. The value of pps_joint_cbcr_qp_offset_value + pps_subpic_joint_cbcr_qp_offset_value[ i ] shall be in the range of −12 to +12, inclusive.

The syntax elements of the slice header remain identical but the semantics of slice_qp_delta, slice_cb_qp_offset, slice_cr_qp_offset  and slice_joint_cbcr_qp_offset are the following:

slice_qp_delta specifies the initial value of QpY to be used for the coding blocks in the slice until modified by the value of CuQpDeltaVal in the coding unit layer. The initial value of the QpY quantization parameter for the slice, SliceQpY , is derived as follows:

SliceQpY = 26 + init_qp_minus26 + pps_subpic_qp_offset[ SubPicIdx ] + slice_qp_delta	(7-111)

The value of SliceQpY shall be in the range of −QpBdOffsetY to +63, inclusive.

slice_cb_qp_offset specifies a difference to be added to the value of pps_cb_qp_offset when determining the value of the Qp′Cb quantization parameter. The value of slice_cb_qp_offset shall be in the range of −12 to +12, inclusive. When slice_cb_qp_offset is not present, it is inferred to be equal to 0. The value of pps_cb_qp_offset + pps_subpic_cb_qp_offset[ SubPicIdx ] + slice_cb_qp_offset shall be in the range of −12 to +12, inclusive.

slice_cr_qp_offset specifies a difference to be added to the value of pps_cr_qp_offset when determining the value of the Qp′Cr quantization parameter. The value of slice_cr_qp_offset shall be in the range of −12 to +12, inclusive. When slice_cr_qp_offset is not present, it is inferred to be equal to 0. The value of pps_cr_qp_offset + pps_subpic_cr_qp_offset[ SubPicIdx ] + slice_cr_qp_offset shall be in the range of −12 to +12, inclusive.

slice_joint_cbcr_qp_offset specifies a difference to be added to the value of pps_joint_cbcr_qp_offset_value when determining the value of the Qp′CbCr. The value of slice_joint_cbcr_qp_offset shall be in the range of −12 to +12, inclusive. When slice_joint_cbcr_qp_offset is not present, it is inferred to be equal to 0. The value of pps_joint_cbcr_qp_offset_value + pps_subpic_joint_cbcr_qp_offset_value[ SubPicIdx ] + slice_joint_cbcr_qp_offset shall be in the range of −12 to +12, inclusive.

## Additional Offsets for cb_qp_offset_list and cr_qp_offset_list

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pps_num_qp_offset_lists_minus1 | ue(v) |
| for( i = 0; i <= pps_num_qp_offset_lists_minus1; i++ ) { |  |
| chroma_qp_offset_list_len_minus1[ i ] | ue(v) |
| for( j = 0; j <= chroma_qp_offset_list_len_minus1[ i ]; j++ ) { |  |
| cb_qp_offset_list[ i ] [ j ] | se(v) |
| cr_qp_offset_list[ i ] [ j ] | se(v) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| joint_cbcr_qp_offset_list[ i ] [ j ] | se(v) |
| } |  |
| } |  |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| subpic_chroma_qp_offset_list_index[ i ] | u(v) |
| } |  |

The introduced new syntax elements have the following semantic:

pps_num_qp_offset_lists_minus1 plus 1 specifies the number of chroma_qp_offset_list_len_minus1[ i ] syntax elements that are present in the PPS RBSP syntax structure. The value of pps_num_qp_offset_lists_minus1 shall be in the range of 0 to 5, inclusive.

chroma_qp_offset_list_len_minus1[ i ] plus 1 specifies the number of cb_qp_offset_list[ i ], cr_qp_offset_list[ i ] and joint_cbcr_qp_offset_list[ i ] syntax elements that are present in the PPS RBSP syntax structure. The value of the sum of the chroma_qp_offset_list_len_minus1[i] for i in the range 0 to pps_num_qp_offset_lists_minus1 shall be in the range of 0 to 5, inclusive.

cb_qp_offset_list[ i ] [ j ], cr_qp_offset_list[ i ] [ j ] and joint_cbcr_qp_offset_list[ i ] [ j ] specify offsets used in the derivation of Qp′Cb, Qp′Cr, and Qp′CbCr, respectively. The values of cb_qp_offset_list[ i ] [ j ], cr_qp_offset_list[ i ] [ j ], and joint_cbcr_qp_offset_list[ i ] [ j ] shall be in the range of −12 to +12, inclusive. When not present, cb_qp_offset_list[ i ] [ j ], cr_qp_offset_list[ i ] [ j ] and joint_cbcr_qp_offset_list[ i ] are inferred to be equal to 0.

subpic_chroma_qp_offset_list_index[ i ] specifies the index of the chroma qp offset tables used by the i-th subpicture. subpic_chroma_qp_offset_list_index[ i ] shall be in the range 0 to pps_num_qp_offset_lists_minus1 inclusive.

The syntax elements of the transform unit containing a CU remain identical but the semantics of cu_chroma_qp_offset_idx is the following:

cu_chroma_qp_offset_idx, when present, specifies the index into the cb_qp_offset_list[ ][ ], cr_qp_offset_list[ ][ ], and joint_cbcr_qp_offset_list[ ][ ] that is used to determine the value of CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr. When present, the value of cu_chroma_qp_offset_idx shall be in the range of 0 to chroma_qp_offset_list_len_minus1[ subpic_chroma_qp_offset_list_index[ SubPicIdx ] ] inclusive. When not present, the value of cu_chroma_qp_offset_idx is inferred to be equal to 0.

When cu_chroma_qp_offset_flag is present, the following applies:

- The variable IsCuChromaQpOffsetCoded is set equal to 1.

- The variables CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are derived as follows:

- If cu_chroma_qp_offset_flag is equal to 1, the following applies:

CuQpOffsetCb = cb_qp_offset_list[ subpic_chroma_qp_offset_list_index[ SubPicIdx ] ] [ cu_chroma_qp_offset_idx ]	(186)

CuQpOffsetCr = cr_qp_offset_list[ subpic_chroma_qp_offset_list_index[ SubPicIdx ] ] [ cu_chroma_qp_offset_idx ]	(187)

CuQpOffsetCbCr = joint_cbcr_qp_offset_list[ subpic_chroma_qp_offset_list_index[ SubPicIdx ] ] [ cu_chroma_qp_offset_idx ]	(188)

- Otherwise (cu_chroma_qp_offset_flag is equal to 0), CuQpOffsetCb, CuQpOffsetCr, and CuQpOffsetCbCr are all set equal to 0.

# Example

The figure above shows how this proposal solves the issues presented in previous example. The bitstream resulting from the combination of the slices from the two bitstreams with PPS1 and PPS2 contains a new PPS (PPS3) with an initial QP value equal to 20. This new PPS provides also QP offsets for the slices that correct the initial QP of the slices so that it corresponds to the initial QP of the originated stream. As a result, the QpY is correct without having to rewrite the slice_qp_delta of the slice headers.

The proposal allows solving the issue of slices QP value without modification of the slice NAL unit. Only the PPS is rewritten which reduces the complexity of bitstream merging and also simplifies some applications like for example viewport dependent streaming.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).