| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0411 |
| --- | --- |

| Title: | AHG12: On Subpictures merging for VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nael Ouedraogo Franck Denoual Frédéric Mazé | Email: | nael.ouedraogo@crf.canon.fr franck.denoual@crf.canon.fr frederic.maze@crf.canon.fr |
| Source: | Canon Research Centre France |  |  |

_____________________________

# Abstract

This contribution addresses the signaling of subpictures suitable for merging operation. Indeed, VVC provides several possible configurations for specific use cases of subpictures. For simple merging operation it is asserted that defining a flag that constrains a subpicture for simple merging operations is desirable.

In particular it is proposed to constrain the subpicture signaling for merging operations as follows:

- Constrain picture width and height to be a multiple of CTB size. This constraint fixes a technical issue, discussed at JVET#16 meeting in JVET-P0351 (further study was encouraged), encountered with VVC subpictures merging coming from pictures with sizes that are not multiple of CTB size.

- Constrain subpicture boundaries to be treated as picture boundaries

- Constrain loop filter to be disabled at subpicture boundaries

- Constrain subpicture ID signaling to be defined at SPS or PPS but not at PH level. This is to prevent any picture-based changes of the subpicture ID mapping which is author asserted as complex to monitor

- Provide subpicture conformance window SEI message for cropping operation. This was also discussed during JVET#16.

# Introduction

During the creation of a new video stream with subpictures taken from different video streams, rightmost or bottom subpictures may be located elsewhere in a merged bitstream as represented in the Figure 1. In this example, the rightmost subpictures 2 and 4 of the HQ bitstream are placed at leftmost position in the merged stream.

Figure 1: Subpicture and bitstream merging

When the picture width (respectively the height) of the bitstream (for example the HQ bitstream) is not a multiple of CTB width (resp. height), the last column (resp. row) of CTB are automatically split and skipped to avoid coding the coding blocks outside of the picture. In VVC, this splitting inference relies on the comparison of the coding block location and size with the actual width and height of the picture (pic_width_in_luma_samples and pic_height_in_luma_samples).

As a result, when the slices initially located at a rightmost or bottom subpicture are moved elsewhere, their coding block locations change and the splitting inference process may fail to determine the skipped coded blocks.

As a result, the picture size of the video sequence may limit the merging operations that can be done without breaking the decoding of subpictures.

# Proposal

## Principle

We propose to indicate in VVC bitstreams with a new flag whether the subpictures it contains are suitable for merging operations or not. The VVC specification currently specifies or allows subpicture signaling ins different parameter sets. For example the layout is defined in the SPS but the subpicture IDs may be defined in SPS, PPS or even Picture Header. This requires parsing and checking multiple flags or parameters in the HLS to determine whether a subpicture can easily be merged in another sequence. Having a simple flag indicating the mergeability may make life easier for applications using subpicture merging. As one example, when such flag is set, we force the size of subpictures to be multiple of CTB, to be fully independent for bitstream merging and also to put the following constraints:

- Constrain picture width and height to be a multiple of CTB size in order to fix a technical issue exposed in the Introduction (merging VVC subpicture coming from pictures with size that is not multiple of CTB size).

- Constrain subpicture boundaries to be treated as picture boundaries

- Constrain loop filter to be disabled at subpicture boundaries

- Constrain subpicture ID signaling at SPS or PPS level but not at PH level. This is to prevent any picture-based changes of the subpicture ID mapping which is author asserted as complex to monitor

## Update of SPS syntax

The syntax of the SPS is changed to contain one additional flag (subpic_mergeable_flag) indicating whether the subpictures of the bitstream are independently decodable or not. When this flag is set, the subpic_treated_as_pic_flag and loop_filter_across_subpic_enabled_flag are assumed to be set and are not present in the SPS.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| […] |  |
| pic_width_max_in_luma_samples | ue(v) |
| pic_height_max_in_luma_samples | ue(v) |
| sps_log2_ctu_size_minus5 | u(2) |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| subpic_mergeable_flag | u(1) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| if (!subpic_mergeable_flag) { |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } | u(8) |
| } |  |
| } |  |
| […] |  |

## Update of SPS semantics

pic_width_max_in_luma_samples specifies the maximum width, in units of luma samples, of each decoded picture referring to the SPS. pic_width_max_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ). When subpic_mergeable_flag equal to 1, pic_width_max_in_luma_samples shall be an integer multiple of CtbSizeY.

pic_height_max_in_luma_samples specifies the maximum height, in units of luma samples, of each decoded picture referring to the SPS. pic_height_max_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ). When subpic_mergeable_flag equal to 1, pic_height_max_in_luma_samples shall be an integer multiple of CtbSizeY.

subpic_mergeable_flag equal to 1 indicates all subpictures of the picture are constrained for merging operations. subpic_mergeable_flag equal to 0 indicates that subpictures may or may not be constrained.

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0subpic_mergeable_flag.

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1!subpic_mergeable_flag.

## Update of PPS semantics

pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_width_max_in_luma_samples. When subpic_mergeable_flag equal to 1, pic_width_in_luma_samples shall be an integer multiple of CtbSizeY.

pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to pic_height_max_in_luma_samples. When subpic_mergeable_flag equal to 1, pic_height_in_luma_samples shall be an integer multiple of CtbSizeY.

## Update of PH semantics

The presence of the subpicture identifiers in the Picture Header may make the merging operation more complex. Indeed, when the signaling of identifiers is declared in the Picture Header, it overrides the mapping of the subpicture identifiers declared in the Parameter Sets NAL units.  Since a different Picture Header is sent for each picture, this allows to change the mapping at each Picture. As a result, a typical merging operation needs to check, for every picture, whether the Picture Header modifies the subpicture identifiers mapping. To avoid this complex checking operation, when the subpictures are constrained for merging operations, the mapping of the identifiers in the Picture Header is disabled.

ph_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the PH. ph_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the PH. When subpic_mergeable_flag is equal to 1, ph_subpic_id_signalling_present_flag shall be equal to 0.

## SEI message for subpicture-based conformance windows

The proposed SEI message allows decoders to handle in-picture areas that require post-decoding crop operation. This may result for example of a bitstream extraction and merging operations where some subpictures originally at the right or bottom border of a picture contained padding data. This SEI indicates decoders the picture areas containing padding or even unreliable or useless data that the content creator considers should be removed on the image to be displayed.

| subpicture_conformance_window (payloadSize) { | Descriptor |
| --- | --- |
| subpic_conf_win_cancel_flag | u(1) |
| if (!subpic_conf_win_cancel_flag) { |  |
| subpic_conf_win_num_subpics_minus1 | ue(v) |
| for( i = 0; i <= subpic_conf_win_num_subpics_minus1; i++ ) { |  |
| subpic_conf_win_left_offset[ i ] | ue(v) |
| subpic_conf_win_right_offset[ i ] | ue(v) |
| subpic_conf_win_top_offset[ i ] | ue(v) |
| subpic_conf_win_bottom_offset[ i ] | ue(v) |
| } |  |
| } |  |
| } |  |

subpic_conf_win_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous subpicture conformance window SEI message in output order that applies to the current layer. subpic_conf_win_cancel_flag equal to 0 indicates that subpicture conformance window information follows.

subpic_conf_win_num_subpics_minus1 plus 1 specifies the number of subpicture conformance windows present in the SEI message.

It is a requirement that the value of subpic_conf_win_num_subpics_minus1 shall be equal to sps_num_subpics_minus1.

subpic_conf_win_left_offset[ i ], subpic_conf_win_right_offset[ i ], subpic_conf_win_top_offset[ i ], and subpic_conf_win_bottom_offset[ i ] specify the samples of the i-th subpicture of the picture in the CLVS that are output from the decoding process, in terms of a rectangular region specified in picture coordinates for output relatively to the origin of the i-th subpicture as described in the SPS NAL unit.

The subpicture conformance cropping window of the i-th subpicture contains the luma samples with horizontal picture coordinates from SubPictureLuma_X[ i ] + SubWidthC * conf_win_left_offset to SubPictureLuma_X[ i ] + SubPictureLuma_Width[ i ] − ( SubWidthC * subpic_conf_win_right_offset [ i ] + 1 ) and vertical picture coordinates from SubPictureLuma_Y[ i ] + SubHeightC * subpic_conf_win_top_offset[ i ] to SubPictureLuma_Y[ i ] + SubPictureLuma_Height[ i ] − ( SubHeightC * subpic_conf_win_bottom_offset [ i ] + 1 ), inclusive. Wherein the SubPictureLuma_X[ i ] and SubPictureLuma_Y[ i ] specify the horizontal and vertical picture coordinates of the first pixel in the i-th subpicture, and SubPictureLuma_Width[ i ] and  SubPictureLuma_Height[ i ] specify the width and the height in luma sample of the i-th subpicture described in the SPS. These variables are computed as follows:

SubPictureLuma_X[ i ]  = subpic_ctu_top_left_x[ i ] * CtbSizeY

SubPictureLuma_Y[ i ] = subpic_ctu_top_left_y[ i ] * CtbSizeY

SubPictureLuma_Width[ i ] = (subpic_width_minus1[ i ] +1 ) * CtbSizeY

SubPictureLuma_Height[ i ] = (subpic_height_minus1[ i ] +1 ) * CtbSizeY

# Conclusion

A new signaling for mergeable subpicture is proposed with an associated set of coding constraints. The proposed signaling makes subpicture-based bitstream merging easier.do.

# Patent rights declaration(s)

Canon Research Centre France may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).