| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0412-v1 |
| --- | --- |

| Title: | AHG9/AHG12: A syntax and semantics fix for subpicture ID mapping |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Ru-Ling Liao, Yan Ye, Jiancong Luo Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In VVC draft 7, the subpictue ID may be signaled in either SPS, PPS or PH, or not signaled in the bitstream at all. According to current syntax, when sps_subpic_id_present_flag is equal to 1, the subpicture ID may not be signaled in SPS, which conflicts with semantics of sps_subpic_id_present_flag. Moreover, in the case of sps_subpic_id_present_flag equal to 1 but subpicture ID not being signalled in SPS, PPS or PH, the subpicture ID list derived in decoder side becomes undefined, according to the current subpicture ID list derivation rules. Thus, two methods are proposed in this contribution to solve these two problems.

In method 1, when sps_subpic_id_present_flag is equal to 1 and subpicture ID is not signaled in SPS or PPS, it is mandatory for subpicture ID to be signaled in PH in order to avoid the case of having undefined subpicture ID list. And the semantics of sps_subpic_id_present_flag is changed to specify the signaling of subpicture ID in the SPS, PPS or PH.

In method 2, an inferred value is assigned to pps_subpic_id[i], so that the inferred ID will be used in the case when sps_subpic_id_present_flag is equal to 1 but subpicture ID is not signaled in SPS, PPS or PH. And the semantics of sps_subpic_id_present_flag is changed to specify the possibility of signaling of subpicture ID in the SPS, PPS or PH

Problem statement

Subpicture was firstly adopted to VVC draft 6 in Gothenburg meeting[1], and later was improved in Geneva meeting. In VVC draft 7[2], the subpicture ID may be explicitly signaled by the encoder in either SPS, PPS or PH, or implicitly derived at the decoder side.

Table 1 SPS syntax of subpicture identifier

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_subpic_id_present_flag | u(1) |
| if( sps_subpic_id_present_flag ) { |  |
| sps_subpic_id_signalling_present_flag | u(1) |
| if( sps_subpic_id_signalling_present_flag ) { |  |
| sps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| sps_subpic_id[ i ] | u(v) |
| } |  |
| } |  |

Table 2 PPS syntax of subpicture identifier

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_subpic_id_signalling_present_flag | u(1) |
| if( pps_subpic_id_signalling_present_flag ) { |  |
| pps_num_subpics_minus1 | ue(v) |
| pps_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= pps_num_subpic_minus1; i++ ) |  |
| pps_subpic_id[ i ] | u(v) |
| } |  |

Table 3 PH syntax of subpicture identifier

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |

sps_subpic_id_present_flag equal to 1 specifies that subpicture ID mapping is present in the SPS. sps_subpic_id_present_flag equal to 0 specifies that subpicture ID mapping is not present in the SPS.

sps_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the SPS. sps_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the SPS. When not present, the value of sps_subpic_id_signalling_present_flag is inferred to be equal to 0.

sps_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sps_subpic_id[ i ]. The value of sps_ subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

sps_subpic_id[ i ] specifies that subpicture ID of the i-th subpicture. The length of the sps_subpic_id[ i ] syntax element is sps_subpic_id_len_minus1 + 1 bits. When not present, and when sps_subpic_id_present_flag equal to 0, the value of sps_subpic_id[ i ] is inferred to be equal to i, for each i in the range of 0 to sps_num_subpics_minus1, inclusive

pps_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the PPS. pps_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the PPS. When sps_subpic_id_present_flag is 0 or sps_subpic_id_signalling_present_flag is equal to 1, pps_subpic_id_signalling_present_flag shall be equal to 0.

pps_num_subpics_minus1 plus 1 specifies the number of subpictures in the coded pictures referring to the PPS.

It is a requirement of bitstream conformance that the value of pps_num_subpic_minus1 shall be equal to  sps_num_subpics_minus1.

pps_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element pps_subpic_id[ i ]. The value of pps_ subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of pps_subpic_id_len_minus1 shall be the same for all PPSs that are referred to by coded pictures in a CLVS.

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits.

ph_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the PH. ph_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the PH.

ph_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element ph_subpic_id[ i ]. The value of pic_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of ph_subpic_id_len_minus1 shall be the same for all PHs that are referred to by coded pictures in a CLVS.

ph_subpic_id[ i ] specifies that subpicture ID of the i-th subpicture. The length of the ph_subpic_id[ i ] syntax element is ph_subpic_id_len_minus1 + 1 bits.

The list SubpicIdList[ i ] is derived as follows:

for( i = 0; i  <=  sps_num_subpics_minus1; i++ )
	SubpicIdList[ i ] = sps_subpic_id_present_flag ?	(1)
		( sps_subpic_id_signalling_present_flag ? sps_subpic_id[ i ] :
		( ph_subpic_id_signalling_present_flag ? ph_subpic_id[ i ] : pps_subpic_id[ i ] ) ) : i

First, sps_subpic_id_present_flag is signaled in SPS, if sps_subpic_id_present_flag is equal to 1, the subpicture may be explicitly signaled in SPS, PPS or PH; otherwise subpicture ID is not explicitly signaled but implicitly derived as subpicture index as formula (1). The subpicture ID syntax in SPS, PPS and PH are shown as Table 1, Table 2 and Table 3, respectively. If subpicture ID is signaled in SPS, it will be directly used for the whole sequence, and PPS and PH cannot signal the subpicture ID anymore in this sequence; otherwise, PPS and PH may signal subpicture ID. If both PPS and PH signal the subpicture ID, the ID signaled in PH overrides the ID signaled in PPS.

However, there are two problem in the subpicture ID related syntax and semantics as follows.

- The semantics of sps_subpic_id_present_flag is that it specifies subpicture ID mapping is present in the SPS if sps_subpic_id_signalling_present_flag is 1. However, in VVC draft 7, when sps_subpic_id_signalling_present_flag is 0, the subpicture ID mapping is not present in the SPS even sps_subpic_id_present_flag is 1. Moreover, pps_subpic_id_signalling_present_flag and ph_subpic_id_signalling_present_flag are currently controlled by sps_subpic_id_present_flag, so sps_subpic_id_present_flag does not only control the subpicture ID signaling in SPS, but also control subpicture ID signaling in PPS or PH. So the current semantics of sps_subpic_id_present_flag is not correct.

- There is one case that sps_subpic_id_present_flag is true, but none of SPS, PPS and PH signals the subpicture ID. In this case, according formula (1), the undefined value of pps_subpic_id[i] will be assigned to SubpicIDList[i], which leads to the subpicture ID list derived in decoder side undefined.

Thus, in this contribution, two methods are proposed to solve the problems.

Proposed method

Method 1

First, semantics of sps_subpic_id_present_flag is change to specify subpicture ID mapping is present either in SPS, PPS or PH when the value of this flag is equal to 1. And then, to follow this semantics when sps_subpic_id_present_flag is 1 but subpicture ID is not signaled in SPS or PPS, it is proposed to force the subpicture ID to be signaled in PH. Thus, when sps_subpic_id_present_flag is 1, it is guaranteed subpicture ID is present in at least one of SPS, PPS or PH. The case that derived subpicture ID list is undefined which is mentioned in section 1 is also eliminated.

The syntax changes are as Table 4 and the semantics changes are given below the table. In this method, the subpicture derivation rule is not changed. (same as formula 1)

Table 4 syntax for method 1

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_present_flag ) { |  |
| if(pps_subpic_id_signalling_present_flag) |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpic_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |

sps_subpic_id_present_flag equal to 1 specifies that subpicture ID mapping is present in the SPS, PPS or PH. sps_subpic_id_present_flag equal to 0 specifies that subpicture ID mapping is not present in the SPS, PPS and PH.

ph_subpic_id_signalling_present_flag equal to 1 specifies that subpicture ID mapping is signalled in the PH. ph_subpic_id_signalling_present_flag equal to 0 specifies that subpicture ID mapping is not signalled in the PH. When not present, if sps_subpic_id_present_flag is equal to 1 and sps_subpic_id_signalling_present_flag is equal to 0, the value of ph_subpic_id_signalling_present_flag is inferred to be 1.

Method 2

According to formula (1), the pps_subpic_id[i] comes last in the subpicture ID checking order among sps_subpic_id[i], pps_subpic_id[i] and ph_subpic_id[i] in subpicture ID list derivation. When none of SPS, PPS and PH signals the subpicture ID, the value of pps_subpic_id[i] is used. Thus, in method 2, an inference rule is assigned to pps_subpic_id[i] to solve the subpicture ID list undefined issue mentioned in section 1. And furthermore, the semantics of sps_subpic_id_present_flag is changed to specify subpicture ID mapping may be present in SPS, PPS or PH when the value of this flag is equal to 1 to match the case.

The spec. change of method is as follows:

sps_subpic_id_present_flag equal to 1 specifies that subpicture ID mapping is may be present in the SPS, PPS or PH. sps_subpic_id_present_flag equal to 0 specifies that subpicture ID mapping is not present in the SPS, PPS and PH.

pps_subpic_id[ i ] specifies the subpicture ID of the i-th subpicture. The length of the pps_subpic_id[ i ] syntax element is pps_subpic_id_len_minus1 + 1 bits. When not present, the value of pps_subpic_id[ i ] is infered to be i.

Conclusion

This contribution proposes to fix the syntax and semantics problem of subpicture ID mapping. Two methods are proposed in this contribution. It is suggested to adopt one of them to the next version of VVC draft.

Reference

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019.

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, October 2019.

G. J. Sullivan, J.-R. Ohm, “Meeting report of the 16th JVET Meeting”, JVET-P2000, October 2019

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).