| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0413-v1 |
| --- | --- |

| Title: | AHG9/AHG12: On subpicture partitioning signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Ru-Ling Liao, Yan Ye, Jiancong Luo Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba group |  |  |

_____________________________

- Abstract

In VVC draft 7, the partitioning information of each picture is always signaled in SPS. However, when the picture only contains one subpicture, the subpicture position and size can be easily derived from the picture size. And moreover, the width and height of the last subpicture can also be easily derived from its top-left position and the width and height of the entire picture. So always signalling all the subpictures’ partitioning information has some redundancy. This contribution proposes to remove such subpicture signaling redundancy by following two changes:

- Modification 1: skip subpicture partitioning information signaling and subpic_treated_as_pic_flag and loop_filter_across_subpic_enabled_flag signaling when sps_num_subpics_minus1 is equal to 0;

- Modification 2: skip the width and height signaling of the last subpicture.

Introduction

Subpicture was firstly adopted to VVC draft 6 in Gothenburg meeting[1], and later improved in Geneva meeting. In the current VVC draft 7[2], the partitioning of subpictures is signaled in SPS as Table 1. If subpictures are present in the current sequence, the number of subpictures is signaled first, and then for each subpicture, the top-left position and width and height are signaled.

Table 1 SPS subpicture syntax in VVC draft 7

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| … |  |

As discussed in the last meeting [3]: 1) extraction and merging functionality requires the ability to have subpics_present_flag equal to 1 even when sps_num_subpics_minus1 is equal to 0; 2) sub-pictures cannot be overlapped with each other, and all the sub-pictures must cover the whole picture.

According to the above two conclusions, the current subpicture partitioning signaling method has some redundancy:

- Since all the sub-pictures must cover the whole picture, when there is only one subpicture, it should be equal to the entire picture. So there is no need to signal the top-left position and width and height of subpicture when there is only one subpicture (sps_num_subpics_minus1=0). Moreover, when sps_num_subpics_minus1 is equal to 0 but subpics_present_flag is equal to 1, the bitstream should be a sub-bitstream from a sub-bitstream extraction process or a bitstream to be merged with other bistreams. Thus, subpic_treated_as_pic_flag should be 1 and loop_filter_across_subpic_enabled_flag should be 0. So there is also no need to signal these two flags.

- In the multiple subpictures case, since all the sub-pictures must cover the whole picture, the width and height of the last subpicture can be easily derived by its left-top position and the width and height of the picture. So there is no need to signal the width and height of the last subpicture.

Proposed method

To remove the redundancy described in section 1, two modifications are proposed.

- Modification 1: skip subpicture partitioning information signaling and subpic_treated_as_pic_flag and loop_filter_across_subpic_enabled_flag signaling when sps_num_subpics_minus1 is equal to 0.

If sps_num_subpics_minus1 is equal to 0, the subpicture should be same as the picture. So there is no need to signal the top-left position and width and height of the subpicture since they can be derived from picture level.

Additionally, when sps_num_subpics_minus1 is equal to 0 but subpics_present_flag is equal to 1. The bitstream should be a sub-bitstream from a sub-bitstream extraction process or a bitstream to be merged with other bitstreams (otherwise, encoder may set subpics_present_flag to 0). Thus, subpic_treated_as_pic_flag should be 1 and loop_filter_across_subpic_enabled_flag should be 0. So there is also no need to signal these two flags.

With modification 1, the syntax is shown and the semantics is given below the Table 2

Table 2 syntax for modification 1

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| if(sps_num_subpics_minus1>0){ |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples÷CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples ÷1 CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples ÷1 CtbSizeY ) ) bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to Ceil( pic_width_max_in_luma_samples ÷1CtbSizeY ) − 1.

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples ÷1 CtbSizeY ) ) bits. When not present, the value of subpic_height_minus1[ i ] is inferred to be equal to Ceil( pic_height_max_in_luma_samples ÷1 CtbSizeY ) − 1.

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, if subpics_present_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1; otherwise the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0.

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, if subpics_present_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0, the value of loop_filter_across_subpic_enabled_flag [ i ] is inferred to be equal to 0; otherwise the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1.

- Modification 2: skip the width and height signaling for the last subpicture.

Since all the sub-pictures must cover the whole picture, the width and height of the last subpicture can be easily derived by its top-left position and the width and height of the picture. So there is no need to signal the width and height of last subpicture.

But please note that with modification 2, a restriction is implicitly imposed on encoder that when signaling the partioning information of each subpicture, the last subpicture should be the one on the bottom-right of the picture.

With modification 2, the syntax is shown in and the semantics are given below

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if(i!=sps_num_subpics_minus1){ |  |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| } |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples ÷ CtbSizeY ) ) bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to Ceil( (pic_width_max_in_luma_samples) ÷  CtbSizeY ) -1- (i== sps_num_subpics_minus1 ? subpic_ctu_top_left_x[sps_num_subpics_minus1]:0)

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples ÷ CtbSizeY ) ) bits. When not present, the value of subpic_height_minus1[ sps_num_subpics_minus1 ] is inferred to be equal to (Ceil( pic_height_max_in_luma_samples ) ÷ CtbSizeY )-1- (i== sps_num_subpics_minus1 ? subpic_ctu_top_left_y[i] :0).

- Modification 1 + Modification 2

By combining the modification 1 modification 2, the proposed the syntax changes and semantics changes are as follows.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| subpics_present_flag | u(1) |
| if( subpics_present_flag ) { |  |
| sps_num_subpics_minus1 | u(8) |
| if(sps_num_subpics_minus1!=0){ |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) { |  |
| subpic_ctu_top_left_x[ i ] | u(v) |
| subpic_ctu_top_left_y[ i ] | u(v) |
| if(i!=sps_num_subpics_minus1){ |  |
| subpic_width_minus1[ i ] | u(v) |
| subpic_height_minus1[ i ] | u(v) |
| } |  |
| subpic_treated_as_pic_flag[ i ] | u(1) |
| loop_filter_across_subpic_enabled_flag[ i ] | u(1) |
| } |  |
| } |  |
| } |  |

subpic_ctu_top_left_x[ i ] specifies horizontal position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples÷CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_x[ i ] is inferred to be equal to 0.

subpic_ctu_top_left_y[ i ] specifies vertical position of top left CTU of i-th subpicture in unit of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples ÷ CtbSizeY ) ) bits. When not present, the value of subpic_ctu_top_left_y[ i ] is inferred to be equal to 0.

subpic_width_minus1[ i ] plus 1 specifies the width of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_width_max_in_luma_samples ÷ CtbSizeY ) ) bits. When not present, the value of subpic_width_minus1[ i ] is inferred to be equal to Ceil( (pic_width_max_in_luma_samples) ÷  CtbSizeY ) -1- (i== sps_num_subpics_minus1 ? subpic_ctu_top_left_x[sps_num_subpics_minus1]:0)

subpic_height_minus1[ i ] plus 1 specifies the height of the i-th subpicture in units of CtbSizeY. The length of the syntax element is Ceil( Log2( pic_height_max_in_luma_samples ÷ CtbSizeY ) ) bits. When not present, the value of subpic_height_minus1[ sps_num_subpics_minus1 ] is inferred to be equal to (Ceil( pic_height_max_in_luma_samples ) ÷ CtbSizeY )-1- (i== sps_num_subpics_minus1 ? subpic_ctu_top_left_y[i] :0).

subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, if subpics_present_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0, the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1; otherwise the value of subpic_treated_as_pic_flag[ i ] is inferred to be equal to 0.

loop_filter_across_subpic_enabled_flag[ i ] equal to 1 specifies that in-loop filtering operations may be performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. loop_filter_across_subpic_enabled_flag[ i ] equal to 0 specifies that in-loop filtering operations are not performed across the boundaries of the i-th subpicture in each coded picture in the CLVS. When not present, if subpics_present_flag is equal to 1 and sps_num_subpics_minus1 is equal to 0, the value of loop_filter_across_subpic_enabled_flag [ i ] is inferred to be equal to 0; otherwise the value of loop_filter_across_subpic_enabled_pic_flag[ i ] is inferred to be equal to 1.

Conclusion

This contribution proposes to remove the redundancies in the subpicture partitioning signaling in VVC draft 7. It is suggested to adopt this contribution to the next version of VVC draft.

Reference

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 6)”, JVET-O2001, July 2019.

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, October 2019.

G. J. Sullivan, J.-R. Ohm, “Meeting report of the 16th JVET Meeting”, JVET-P2000, October 2019

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

In VVC draft 7, it is “/”. It is proposed to fix as “÷” in ticket #771. The fixed version is used as the base of this contribution.