| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0419-v1 |
| --- | --- |

| Title: | AHG 9: On picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li, Byeongdoo Choi, Xiaozhong Xu, Xiang Li, Stephan Wenger, Shan Liu | Email: | {aurali, bdchoi, xiangzhongxu, xlxiangli, swenger, shanl }@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

In this contribution, three aspects are suggested for the picture header syntax design.

- Picture header related syntax element is also included in slice layer NAL unit to have error resilience and error recovery advantages.

- The value of slice_type in slice_header() could be constrained by or inferred from syntax elements signaled in PH.

- With the adoption of PH, constant_slice_header_params_enabled_flag indicates the presence of constantly used syntax elements in slice header is redundant.

# Problem Statement

In the last meeting, PH is adopted to signal syntax elements with constant values for slices such that lighter slice layer NAL unit could be achieved. However, when PH NAL unit is corrupted, there is no other way to recover those syntax elements in PH.

The value of slice_type specifies the coding type of slice as 0 for B slice, 1 for P slice and 2 for I slice. When syntax elements [1] specifying the combination of slice types associated with the PH, the value of slice_type could be constrained  or inferred from them.

With the adoption of PH, constant_slice_header_params_enabled_flag indicates the presence of constantly used syntax elements in slice header is redundant.

# Proposed Aspect 1

Aspect 1 proposes a more error resilient design for PH. PH related syntax elements are included in the slice layer NAL unit, and ph_present_flag is used to indicate the presence of PH related syntax in slice layer NAL unit. Repetition of PH related syntax signaling has advantage of error resilience and error recovery. When PH NAL unit is corrupted, slice layer NAL units are able to recover from the error with presence of PH in slice layer RBSP NAL units.

Syntax and semantics are described as below.

Table 1 –Proposed slice layer RBSP syntax

| slice_layer_rbsp( ) { | Descriptor |
| --- | --- |
| ph_present_flag | u(1) |
| if (ph_present_flag) |  |
| picture_header_rbsp( ) |  |
| slice_header( ) |  |
| slice_data( ) |  |
| rbsp_slice_trailing_bits( ) |  |
| } |  |

ph_present_flag specifies the presence of PH related syntax in slice layer RBSP. When ph_present_flag equal to 1, PH related syntax is present. When ph_present_flag equal to 0, PH related syntax is not present in slice layer RBSP.

# Proposed Aspect 2

Aspect 2 proposes to use syntax elements [1] presented in PH to constrain or infer the value of slice_type of slice header which associated with the PH.

## Solution 1:

Following syntax elements are proposed in PH [1].

pic_single_coding_type_flag specifies whether all the slices in the picture associated with the PH use only I slice type or only P/B slice types. If pic_single_coding_type_flag is equal to 1, all the slices associated with the PH shall choose slice_type all equal to I_slice, or all equal to inter slice types (P_slice or B_slice). Otherwise (pic_single_coding_type_flag is equal to 0), all slice types may be used in the slices of the picture associated with the PH. When not present, the value of pic_single_coding_type_flag is inferred to be equal to 0.

pic_intra_picture_pred_only_flag equal to 1 specifies that slice_type equal to I shall be true for all slices in the picture associated with the PH. pic_intra_picture_pred_only_flag equal to 0 specifies that slice_type equal to P or B shall be true for all the slices associated with the PH. When not present, the value of pic_intra_picture_pred_only_flag is inferred to be equal to 0.

Table 2 –Proposed constraint for slice_type

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( !pic_intra_picture_pred_only_flag) |  |
| slice_type | ue(v) |
| … |  |
| } |  |

When pic_intra_picture_pred_only_flag is equal to 1 specifying all slices associated with the PH are I slices, slice type is infer to be equal to 2. When pic_single_coding_type_flag is equal to 0, slice_type has a range of 0 to 2, inclusive. When pic_single_coding_type_flag is equal to 1 and pic_intra_picture_pred_only_flag is equal to 0, slice_type has a range of 0 to 1, inclusive.

## Solution 2:

Following syntax elements are proposed in PH [1].

pic_inter_slice_only_flag equal to 1 specifies that a slice with slice_type equal to I cannot exist in the picture associated with the PH. pic_intra_slice_only_flag is equal to 0 specifies that a slice with slice_type equal to I may exist in the picture associated with the PH. When not present, the value of pic_inter_slice_only_flag is inferred to be equal to 0.

pic_intra_slice_only_flag equal to 1 specifies that a slice with slice_type equal to P or B cannot exist in the picture associated with the PH. pic_intra_slice_only_flag is equal to 0 specifies that a slice with slice_type equal to P or B may exist in the picture associated with the PH. When not present, the value of pic_intra_slice_only_flag is inferred to be equal to 0.

Table 3 –Proposed constraint for slice_type

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if ( ! pic_intra_slice_only_flag) |  |
| slice_type | ue(v) |
| … |  |
| } |  |

When pic_intra_slice_only_flag is equal to 1 specifying all slices associated with the PH are I slices, slice type is infer to be equal to 2. When pic_inter_slice_only_flag is equal to 1, slice_type has a range of 0 to 1, inclusive. When both pic_inter_slice_only_flag and pic_intra_slice_only_flag are equal to 0, slice_type has a range of 0 to 2, inclusive.

# Proposed Aspect 3

Aspect 3 proposes to remove syntax elements related to constant_slice_header_params_enabled_flag. The constant slice header parameters presented in PPS is adopted without consideration of PH design. These two both aim to reduce the bits used by slices headers. Therefore, to have cleaner design, it is proposed to remove redundant constant slice header parameters presented in PPS.

Table 4 –Proposed to change in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| pps_dep_quant_enabled_idc | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] | u(2) |
| pps_mvd_l1_zero_idc | u(2) |
| pps_collocated_from_l0_idc | u(2) |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | ue(v) |
| } |  |
| … |  |
| } |  |

Table 5 –Proposed changes in PH

| if( pic_rpl_present_flag ) { |  |
| --- | --- |
| for( i = 0; i < 2; i++ ) { |  |
| if( num_ref_pic_lists_in_sps[ i ] > 0 && !pps_ref_pic_list_sps_idc[ i ] && ( i = = 0 ¦ ¦ ( i = = 1 && rpl1_idx_present_flag ) ) ) |  |
| pic_rpl_sps_flag[ i ] | u(1) |
| … |  |
| } |  |
| } |  |
| … |  |
| if ( !pps_dep_quant_enabled_flag ) |  |
| pic_dep_quant_enabled_flag | u(1) |
| … |  |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| … |  |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| … |  |
| } |  |

Table 6 –Proposed changes in slice header

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( slice_type != I ) { |  |
| if( cabac_init_present_flag ) |  |
| cabac_init_flag | u(1) |
| if( pic_temporal_mvp_enabled_flag ) { |  |
| if( slice_type = = B && !pps_collocated_from_l0_idc ) |  |
| collocated_from_l0_flag | u(1) |
| if( ( collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( !collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| collocated_ref_idx | ue(v) |
| } |  |
| if( ( pps_weighted_pred_flag && slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && slice_type = = B ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| … |  |
| } |  |

# Conclusion

In this contribution, three aspects are suggested for the picture header syntax design.

- Picture header related syntax element is also included in slice layer NAL unit to have error resilience and error recovery advantages.

- The value of slice_type in slice_header() could be constraint by or inferred from syntax elements signaled in PH.

- With the adoption of PH, constant_slice_header_params_enabled_flag indicates the presence of constantly used syntax elements in slice header is redundant.

It is recommended to adopt this proposal into next version of VVC.

# References

- X. Xu, B. Choi, L. Li, X. Li, S. Wenger, S. Liu, “AHG 9: Picture header syntax cleanups”, JVET-Qxxxx, Brussels, January 2020.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).