| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0420-v1 |
| --- | --- |

| Title: | AHG12: Signaling of chroma presence in PPS and APS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li, Xiang Li, Cheung Auyeung, Byeongdoo Choi, Stephan, Wenger, Shan Liu | Tel: Email: | {aurali, xlxiangli, cauyeung, bdchoi, swenger, shanl }@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes chroma coding tool syntax element present flag in both PPS and APS to specify whether chroma component presents to simplify signaling while avoid parsing dependency between parameter sets.

- pps_chroma_coding_tool_syntax_elements_present_flag: specifies whether syntax elements for chroma coding tools are present or not in the PPS

- aps_chroma_coding_tool_syntax_elements_present_flag: specifies whether syntax elements for chroma coding tools are present or not in the APS

# Introduction

Compared to HEVC, VVC contains more chroma only related tools such as joint CbCr and its QP offset, user defined chroma QP mapping table, ALF chroma filter and so on. For avoiding dependency between SPS and PPS, those chroma only related syntax elements are decoded but set to equal to 0 by bitstream conformance. With a vast of chroma only related tools, it is proposed to signal chroma present flag in both PPS and APS to simplify the signaling.

# Proposed Spec Text

Changes compared to the latest VVC Draft 7 [1] are highlighted.

Table 1. proposed pps_chroma_coding_tool_syntax_elements_present_flag in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_chroma_coding_tool_syntax_elements_present_flag | u(1) |
| if( pps_chroma_coding_tool_syntax_elements_present_flag ) { |  |
| pps_cb_qp_offset | se(v) |
| pps_cr_qp_offset | se(v) |
| pps_joint_cbcr_qp_offset_present_flag | u(1) |
| if( pps_joint_cbcr_qp_offset_present_flag ) |  |
| pps_joint_cbcr_qp_offset_value | se(v) |
| pps_slice_chroma_qp_offsets_present_flag | u(1) |
| pps_cu_chroma_qp_offset_list_enabled_flag | u(1) |
| } |  |
| … |  |
| } |  |

pps_chroma_coding_tool_syntax_elements_present_flag specifies whether syntax elements for chroma coding tools are present or not in the PPS. When pps_chroma_coding_tool_syntax_elements_present_flag equal to 1, chroma coding tools related syntax elements are presents in PPS. pps_chroma_coding_tool_syntax_elements_present_flag equal to 0 specifies that chroma coding tools related syntax elements are not present in PPS. It is a requirement of bitstream conformance that pps_chroma_coding_tool_syntax_elements_present_flag shall be equal to 0 when ChromaArrayType is equal to 0.

pps_cb_qp_offset and pps_cr_qp_offset specify the offsets to the luma quantization parameter Qp′Y used for deriving Qp′Cb and Qp′Cr, respectively. The values of pps_cb_qp_offset and pps_cr_qp_offset shall be in the range of −12 to +12, inclusive. When ChromaArrayType is equal to 0, pps_cb_qp_offset and pps_cr_qp_offset are not used in the decoding process and decoders shall ignore their value. When pps_cb_qp_offset and pps_cr_qp_offset are not present, both  pps_cb_qp_offset and pps_cr_qp_offset are inferred to be equal to 0.

pps_joint_cbcr_qp_offset_present_flag equal to 1 specifies that pps_joint_cbcr_qp_offset_value and joint_cbcr_qp_offset_list[ i ] are present in the PPS RBSP syntax structure. pps_joint_cbcr_qp_offset_present_flag equal to 0 specifies that pps_joint_cbcr_qp_offset_value and joint_cbcr_qp_offset_list[ i ] are not present in the PPS RBSP syntax structure. When ChromaArrayType is equal to 0 or sps_joint_cbcr_enabled_flag is equal to 0, the value of pps_joint_cbcr_qp_offset_present_flag shall be equal to 0. When pps_joint_cbcr_qp_offset_present_flag is not present, it is inferred to be be equal to 0.

pps_slice_chroma_qp_offsets_present_flag equal to 1 indicates that the slice_cb_qp_offset and slice_cr_qp_offset syntax elements are present in the associated slice headers. pps_slice_chroma_qp_offsets_present_flag equal to 0 indicates that these syntax elements are not present in the associated slice headers. When ChromaArrayType is equal to 0, pps_slice_chroma_qp_offsets_present_flag shall be equal to 0. When pps_slice_chroma_qp_offsets_present_flag is not present, it is inferred to be be equal to 0.

pps_cu_chroma_qp_offset_list_enabled_flag equal to 1 specifies that the pic_cu_chroma_qp_offset_subdiv_intra_slice and pic_cu_chroma_qp_offset_subdiv_inter_slice syntax elements are present in PHs referring to the PPS and that cu_chroma_qp_offset_flag may be present in the transform unit syntax and the palette coding syntax. pps_cu_chroma_qp_offset_list_enabled_flag equal to 0 specifies that the pic_cu_chroma_qp_offset_subdiv_intra_slice and pic_cu_chroma_qp_offset_subdiv_inter_slice syntax elements are not present in picture headers referring to the PPS and that the cu_chroma_qp_offset_flag is not present in the transform unit syntax and the palette coding syntax. When ChromaArrayType is equal to 0, it is a requirement of bitstream conformance that the value of pps_cu_chroma_qp_offset_list_enabled_flag  shall be equal to 0. When pps_cu_chroma_qp_offset_list_enabled_flag is not present, it is inferred to be be equal to 0.

Table 2. proposed aps_chroma_coding_tool_syntax_elements_present_flag in APS

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| aps_chroma_coding_tool_syntax_elements_present_flag | u(1) |
| if( aps_params_type = = ALF_APS ) |  |
| Alf data( ) |  |
| else if( aps_params_type = = LMCS_APS ) |  |
| lmcs_data( ) |  |
| else if( aps_params_type = = SCALING_APS ) |  |
| scaling_list_data( ) |  |
| aps_extension_flag | u(1) |
| if( aps_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| aps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

| alf_data( ) { | Descriptor |
| --- | --- |
| alf_luma_filter_signal_flag | u(1) |
| if( aps_chroma_coding_tool_syntax_elements_present_flag) |  |
| alf_chroma_filter_signal_flag | u(1) |
| … |  |
| } |  |

aps_chroma_coding_tool_syntax_elements_present_flag specifies whether syntax elements for chroma coding tools are present or not in the APS. When aps_chroma_coding_tool_syntax_elements_present_flag equal to 1, chroma coding tools related syntax elements are presents in APS. aps_chroma_coding_tool_syntax_elements_present_flag equal to 0 specifies that chroma coding tools related syntax elements are not present in APS. It is a requirement of bitstream conformance that pps_chroma_coding_tool_syntax_elements_present_flag shall be equal to 0 when ChromaArrayType is equal to 0.

# Conclusion

For a vast of chroma only related tools in VVC, it is proposed to signal chroma coding tool syntax element present flag in both PPS and APS to simplify the signaling while avoiding parsing dependency between parameter sets. It is recommended to adopt this proposal into next version of VVC.

# Reference

- B. Bross, J. Chen, S. Liu, Y.K. Wang, “Versatile Video Coding (Draft 7)” Document of Joint Video Experts Team (JVET), JVET-P2001-vE, Geneva, Switzerland, 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).