| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0421-v2 |
| --- | --- |

| Title: | AHG15: Clean up for signaling quantization matrix |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li, Hongbin Zhang, Xiang Li, Shan Liu | Tel: Email: | {aurali, hobingzhang, xlxiangli, shanl }@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes a cleanup for signaling quantization matrix. When scaling list ID is equal to 0 , 2 and 8, which are the start IDs of 2x2, 4x4 and 8x8 QM, respectively, scaling list copy mode flag is not signaled.

# Introduction

Table 1 – Specification of the scaling matrix identifier variable id according to predMode, cIdx, nTbW, and nTbH

| predMode = MODE_INTRA | cIdx = 0 (Y) |  | 2 | 8 | 14 | 20 | 26 |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | cIdx = 1 (Cb) |  | 3 | 9 | 15 | 21 | 21 |
|  | cIdx = 2 (Cr) |  | 4 | 10 | 16 | 22 | 22 |
| predMode = MODE_INTER | cIdx = 0 (Y) |  | 5 | 11 | 17 | 23 | 27 |
|  | cIdx = 1 (Cb) | 0 | 6 | 12 | 18 | 24 | 24 |
|  | cIdx = 2 (Cr) | 1 | 7 | 13 | 19 | 25 | 25 |
| max( nTbW, nTbH ) |  | 2 | 4 | 8 | 16 | 32 | 64 |
| Decoded QM coefficients |  | 2x2 | 4x4 | 8x8 | 8x8 + DC |  |  |

In the last Geneva meeting, one-dimension quantization matrix signaling is adopted. The variable id and its corresponding quantization matrix are described in Table 1. The id for quantization matrix can have three categories by size:

- 2x2 based QM (id is 0..1)

- 4x4 based QM (id is 2..7)

- 8x8 based QM (id is 8..27)

Inter QM prediction is allowed within the same category, such as QM ID equals to 4 can be copied or predicted from QM id equals 2. However, for those IDs which are the first QM in their categories, it is unable to copy from others. Therefore, copy mode flag is unnecessary.

It is proposed to not signal scaling_list_copy_mode_flag when the QM ID is 0, 2 and 8.

# Proposed Spec Text

Changes compared to the latest VVC Draft 7 [1] are highlighted.

Table 2. proposed syntax change in scaling_list_data ()

| scaling_list_data( ) { | Descriptor |
| --- | --- |
| scaling_matrix_for_lfnst_disabled_flag | u(1) |
| for( id = 0; id < 28; id ++ ) { |  |
| matrixSize = (id < 2 ) ? 2 : ( ( id < 8 ) ? 4 : 8 ) |  |
| if (id != 0 && id != 2 && id != 8) |  |
| scaling_list_copy_mode_flag[ id ] | u(1) |
| if( !scaling_list_copy_mode_flag[ id ] ) |  |
| scaling_list_pred_mode_flag[ id ] | u(1) |
| if( ( scaling_list_copy_mode_flag[ id ] ¦ ¦ scaling_list_pred_mode_flag[ id ] ) && id != 0 && id != 2 && id != 8 ) |  |
| scaling_list_pred_id_delta[ id ] | ue(v) |
| if( (!scaling_list_copy_mode_flag[ id ]) { |  |
| nextCoef = 0 |  |
| if( id > 13 ) { |  |
| scaling_list_dc_coef[ id − 14 ] | se(v) |
| nextCoef += scaling_list_dc_coef[ id − 14 ] |  |
| } |  |
| for( i = 0; i < matrixSize * matrixSize; i++ ) { |  |
| x = DiagScanOrder[ 3 ][ 3 ][ i ][ 0 ] |  |
| y = DiagScanOrder[ 3 ][ 3 ][ i ][ 1 ] |  |
| if( !( id > 25 && x >= 4 && y >= 4 ) ) { |  |
| scaling_list_delta_coef[ id ][ i ] | se(v) |
| nextCoef += scaling_list_delta_coef[ id ][ i ] |  |
| } |  |
| ScalingList[ id ][ i ] = nextCoef |  |
| } |  |
| } |  |
| } |  |
| } |  |

scaling_list_copy_mode_flag[ id ] equal to 1 specifies that the values of the scaling list are the same as the values of a reference scaling list. The reference scaling list is specified by scaling_list_pred_id_delta[ id ]. scaling_list_copy_mode_flag[ id ] equal to 0 specifies that scaling_list_pred_mode_flag is present. When not present, the value of scaling_list_copy_mode_flag[id] is inferred to be equal to 0.

# Conclusion

When scaling list ID is equal to 0, 2 and 8, which are the start IDs of 2x2, 4x4 and 8x8 QM, respectively, scaling list copy mode flag is not signaled. It is recommended to adopt this proposal into next version of VVC.

# Reference

- B. Bross, J. Chen, S. Liu, Y.K. Wang, “Versatile Video Coding (Draft 7)” Document of Joint Video Experts Team (JVET), JVET-P2001-vE, Geneva, Switzerland, 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).