| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0422-v1 |
| --- | --- |

| Title: | CE4 related: cleanup for signaling maximum number of triangle merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ling Li, Xiang Li, Guichun Li, Shan Liu | Tel: Email: | {aurali, xlxiangli, guichunli, shanl }@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Adoption of JVET-O0238 allows signaling some of the picture header (PH) parameters in PPS which stay constant for all slices associated with the PH. As one of the parameters, the syntax elements used to derive the maximum number of triangle merge candidate could be signaled either in PPS or PH. However, the current design introduces some issues.

- When MaxNumMergeCand is 2, it is redundant to signal pic_max_num_merge_cand_minus_max_num_triangle_cand regardless of the value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1.

- When sps_triangle_enabled_flag is 1 and MaxNumMergeCand is larger than or equal to 2, TPM could not be turned off for certain pictures. Therefore, it loses picture level flexibility.

- Regardless of sps_triangle_enabled_flag, pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 could be signaled and have a value which is never been used.

This contribution proposes a cleanup for signaling maximum number of triangle merge candidates to resolve above issues.

# Introduction

Current signaling of maximum number of triangle merge candidate in VVC Draft 7 [1] is described below.

Table 1 – Specification of the number of maximum TPM merge candidate signaled in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| … |  |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | ue(v) |
| } |  |
| … |  |
| } |  |

Table 2 – Specification of the number of maximum TPM merge candidate signaled in PH

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| … |  |
| } |  |

pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 equal to 0 specifies that pic_max_num_merge_cand_minus_max_num_triangle_cand is present in PHs of slices referring to the PPS. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 greater than 0 specifies that pic_max_num_merge_cand_minus_max_num_triangle_cand is not present in PHs referring to the PPS. The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 shall be in the range of 0 to MaxNumMergeCand − 1. When pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 is not present, it is inferred to be equal to 0.

pic_max_num_merge_cand_minus_max_num_triangle_cand specifies the maximum number of triangular merge mode candidates supported in the slices associated with the pictur header subtracted from MaxNumMergeCand.

When pic_max_num_merge_cand_minus_max_num_triangle_cand is not present, and sps_triangle_enabled_flag is equal to 1 and MaxNumMergeCand is greater than or equal to 2, pic_max_num_merge_cand_minus_max_num_triangle_cand is inferred to be equal to pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 − 1.

The maximum number of triangular merge mode candidates, MaxNumTriangleMergeCand is derived as follows:

MaxNumTriangleMergeCand=  
MaxNumMergeCand -  pic_max_num_merge_cand_minus_max_num_triangle_cand

When pic_max_num_merge_cand_minus_max_num_triangle_cand is present, the value of MaxNumTriangleMergeCand shall be in the range of 2 to MaxNumMergeCand, inclusive.

When pic_max_num_merge_cand_minus_max_num_triangle_cand is not present, and (sps_triangle_enabled_flag is equal to 0 or MaxNumMergeCand is less than 2), MaxNumTriangleMergeCand is set equal to 0.

When MaxNumTriangleMergeCand is equal to 0, triangle merge mode is not allowed for the slices associated with the PH.

# Problem Statement

To resolve following issues, several solutions are proposed.

- When MaxNumMergeCand is 2, it is redundant to signal pic_max_num_merge_cand_minus_max_num_triangle_cand regardless of the value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1.

- When sps_triangle_enabled_flag is 1 and MaxNumMergeCand is larger than or equal to 2, TPM could not be turned off for certain pictures. Therefore, it loses picture level flexibility.

- Regardless of sps_triangle_enabled_flag, pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 could be signaled and have a value which is never been used.

# Solution 1

Since current design does not support picture level TPM off when sps_triangle_enabled_flag is 1 and MaxNumMergeCand is larger than or equal to 2, the MaxNumTriangleMergeCand is in the range of 2 to MaxNumMergeCand, inclusive.

Furthermore, when sps_triangle_enabled_flag is 1 and MaxNumMergeCand is equal to 2, the MaxNumTriangleMergeCand is always 2 regardless of the value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 and pic_max_num_merge_cand_minus_max_num_triangle_cand.

To cleanup, it is suggested to only signal pic_max_num_merge_cand_minus_max_num_triangle_cand in following conditions.

Table 3 – Proposed the number of maximum TPM merge candidate signaled in PH

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >=3 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| … |  |
| } |  |

# Solution 2

Solution 2 proposes to support functionality of picture level turning TPM off when sps_triangle_enabled_flag is 1 and MaxNumMergeCand is larger than or equal to 2. This functionality is supported in original proposal JVET-N0400 which proposed to have MaxNumTriangleMergeCand. However, with adoption of JVET-O0238, the functionality no longer exists, which is believed as a bug.

Please note that solution 2 and solution 1 are not orthogonal.

## Solution 2.a

The functionality is supported by simply changing the ranges of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 and MaxNumTriangleMergeCand.

Table 4 – Proposed the ranges

|  | VVC Draft 7 | Proposed |
| --- | --- | --- |
| pps_max_num_merge_cand_minus_max_ num_triangle_cand_plus1 | 0 to MaxNumMergeCand − 1 | 0 to MaxNumMergeCand-1, and MaxNumMergeCand+1 |
| MaxNumTriangleMergeCand | 2 to MaxNumMergeCand | 0, 2 to MaxNumMergeCand |

## Solution 2.b

The functionality is supported by adding pps_max_num_triangle_merge_cand_present_flag specifying presence of pps_max_num_merge_cand_minus_max_num_triangle_cand syntax element in the bitstream. The proposed flag can decouple the presence and the actual value of maximum number of triangle merge candidate in PPS. The syntax and semantics are described in below.

Table 5 – Proposed number of maximum TPM merge candidate signaled in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| pps_dep_quant_enabled_idc | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] | u(2) |
| pps_mvd_l1_zero_idc | u(2) |
| pps_collocated_from_l0_idc | u(2) |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_max_num_triangle_merge_cand_present_flag | u(1) |
| if (pps_max_num_triangle_merge_cand_present_flag) |  |
| pps_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| … |  |
| } |  |

pps_max_num_triangle_merge_cand_present_flag specifies the presence of pps_max_num_merge_cand_minus_max_num_triangle_cand syntax element in the bitstream. When pps_max_num_triangle_merge_cand_present_flag is equal to 1, pps_max_num_merge_cand_minus_max_num_triangle_cand is present. When pps_max_num_triangle_merge_cand_present_flag is equal to 0, pps_max_num_merge_cand_minus_max_num_triangle_cand is not present. When pps_max_num_triangle_merge_cand_present_flag is not present, it is inferred to be 0. When the value of sps_triangle_enabled_flag is 0, it is requirement of bitstream conformance that the value of pps_max_num_triangle_merge_cand_present_flag shall equal to 0.

pps_max_num_merge_cand_minus_max_num_triangle_cand specifies maximum number of triangular merge mode candidates supported in the pictures refered to the PPS subtraced from MaxNumMergeCand.

When pps_max_num_merge_cand_minus_max_num_triangle_cand is not present, it is inferred to be equal to 0.

Table 6 – Proposed number of maximum TPM merge candidate signaled in PH

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && ! pps_max_num_triangle_merge_cand_present_flag) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| … |  |
| } |  |

pic_max_num_merge_cand_minus_max_num_triangle_cand specifies the maximum number of triangular merge mode candidates supported in the slices associated with the picture header subtracted from MaxNumMergeCand.

When pic_max_num_merge_cand_minus_max_num_triangle_cand is not present, it is inferred to be equal to the value of pps_max_num_merge_cand_minus_max_num_triangle_cand.

The maximum number of triangular merge mode candidates, MaxNumTriangleMergeCand is derived as follows:

If sps_triangle_enabled_flag is equal to 1 and MaxNumMergeCand is larger than or equal to 2,

MaxNumTriangleMergeCand=  
MaxNumMergeCand -  pic_max_num_merge_cand_minus_max_num_triangle_cand

Otherwise, MaxNumTriangleMergeCand is set to 0.

When MaxNumTriangleMergeCand is equal to 0, triangle merge mode is not allowed for the slices associated with the PH.

When pic_max_num_merge_cand_minus_max_num_triangle_cand is present, the value of MaxNumTriangleMergeCand shall be in the range of 0, 2 to MaxNumMergeCand, inclusive.

## Solution 2.c

The functionality is supported by simply removing pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 syntax element.

Table 7 – Proposed number of maximum TPM merge candidate signaled in PPS

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| constant_slice_header_params_enabled_flag | u(1) |
| if( constant_slice_header_params_enabled_flag ) { |  |
| pps_dep_quant_enabled_idc | u(2) |
| for( i = 0; i < 2; i++ ) |  |
| pps_ref_pic_list_sps_idc[ i ] | u(2) |
| pps_mvd_l1_zero_idc | u(2) |
| pps_collocated_from_l0_idc | u(2) |
| pps_six_minus_max_num_merge_cand_plus1 | ue(v) |
| pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 | ue(v) |
| } |  |
| … |  |
| } |  |

Table 8 – Proposed number of maximum TPM merge candidate signaled in PH

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| … |  |
| } |  |

pic_max_num_merge_cand_minus_max_num_triangle_cand specifies the maximum number of triangular merge mode candidates supported in the slices associated with the pictur header subtracted from MaxNumMergeCand.

When pic_max_num_merge_cand_minus_max_num_triangle_cand is not present, and sps_triangle_enabled_flag is equal to 1 and MaxNumMergeCand greater than or equal to 2, pic_max_num_merge_cand_minus_max_num_triangle_cand is inferred to be equal to pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 − 1.

The maximum number of triangular merge mode candidates, MaxNumTriangleMergeCand is derived as follows:

MaxNumTriangleMergeCand=  
MaxNumMergeCand -  pic_max_num_merge_cand_minus_max_num_triangle_cand

When pic_max_num_merge_cand_minus_max_num_triangle_cand is present, the value of MaxNumTriangleMergeCand shall be in the range of 0, 2 to MaxNumMergeCand, inclusive.

When pic_max_num_merge_cand_minus_max_num_triangle_cand is not present, and (sps_triangle_enabled_flag is equal to 0 or MaxNumMergeCand is less than 2), MaxNumTriangleMergeCand is set equal to 0.

When MaxNumTriangleMergeCand is equal to 0, triangle merge mode is not allowed for the slices associated with the PH.

# Solution 3

Solution 3 proposes to constrain the value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 when sps_triangle_enabled_flag is 0.

Please note that solution 3 can be combined with solution 1 or 2.

pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 equal to 0 specifies that pic_max_num_merge_cand_minus_max_num_triangle_cand is present in PHs of slices referring to the PPS. pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 greater than 0 specifies that pic_max_num_merge_cand_minus_max_num_triangle_cand is not present in PHs referring to the PPS. The value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 shall be in the range of 0 to MaxNumMergeCand − 1. When pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 is not present, it is inferred to be equal to 0. It is a requirement of bitstream conformance the value of pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 shall be 0 when sps_triangle_enabled_flag is equal to 0.

# Conclusion

This contribution proposes a cleanup for several issues introduced by having pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1. It is recommended to adopt this proposal into next version of VVC.

# Reference

- B. Bross, J. Chen, S. Liu, Y.K. Wang, “Versatile Video Coding (Draft 7)” Document of Joint Video Experts Team (JVET), JVET-P2001-vE, Geneva, Switzerland, 2019.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).