| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0427-v1 |
| --- | --- |

| Title: | Non-CE2: picture/slice level deblocking control flag for IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu Guichun Li Xiang Li Shan Liu | Tel: Email: | xiaozhongxu@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, a picture/slice level deblocking on/off control flag is suggested for the IBC mode. The claimed benefit of this flag is to turn on/off the deblocking operation for IBC coded samples with an encoder side choice for different contents.

Problem Statement

The deblocking decision for IBC mode has been studied since HEVC SCC, where IBC mode was unified with inter mode. In that case, the BS decision making in deblocking process for IBC is naturally aligned with inter mode deblocking decision method, i.e., using MV magnitudes/reference picture to evaluate whether adjustment on the boundary is necessary. In current VVC specification [1], separate processing modules are considered for IBC mode and inter mode. But for historical reasons, the deblocking part for IBC has been basically the same as in HEVC SCC.

On the other hand, studies have been carried out in the past meetings [2][3], revealing that for screen content, deblocking may not be necessary as it will blur the boundary when sharp edges are present in the content. Some solutions are to disable the deblocking operations on the boundary with one or both sides being IBC coded. However, consensus on this issue cannot be made. One of the arguments for keeping IBC with deblocking operation is that, IBC mode can also be used for natural camera-captured content, where the boundary of two blocks needs to be filtered to remove blocking artifacts from quantization. Even for screen content, it is not always good to disable the deblocking around IBC coded sample boundaries.

Proposed solutions

In this proposal, a picture/slice level flag is suggested to control the use of deblocking filters at IBC coded sample boundaries. By doing this, the encoder will have the choice to turn on or off deblocking operations for IBC mode alone. Below is the list of solutions following this idea:

- Solution 1.1: a picture level flag to turn off IBC deblocking filter, turn off deblocking filter when either side of the edge is coded in IBC mode

- Solution 1.2: a picture level flag to turn off IBC deblocking filter, turn off deblocking filter when both sides of the edge are coded in IBC mode

- Solution 2.1: a slice level flag to turn off IBC deblocking filter, turn off deblocking filter when either side of the edge is coded in IBC mode

- Solution 2.2: a slice level flag to turn off IBC deblocking filter, turn off deblocking filter when both sides of the edge are coded in IBC mode

Proposed text changes

Solution 1.1

- 7.3.2.6 Picture header RBSP syntax

| … |  |
| --- | --- |
| if( deblocking_filter_override_enabled_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flag | u(1) |
| if( pic_deblocking_filter_override_flag ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( !pic_deblocking_filter_disabled_flag && sps_ibc_enabled_flag) |  |
| pic_ibc_deblocking_filter_disabled_flag | u(1) |
| … |  |

- 7.4.3.6 Picture header RBSP semantics

pic_ibc_deblocking_filter_disabled_flag equal to 1 specifies that the operation of the deblocking filter is not applied for edges with samples from either side are coded in IBC mode in the current slice. pic_ibc_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter for samples coded in IBC mode may be applied for the current slice. When pic_ibc_deblocking_filter_disabled_flag is not present, it is inferred to be equal to 0.

- 7.8.4.1 General slice header semantics

Variable SliceIbcDeblockingFilterDisabled is derived as follows:

- If pic_ibc_deblocking_filter_disabled_flag is equal to 1 and slice_deblocking_filter_disabled_flag is equal to 0, SliceIbcDeblockingFilterDisabled is set equal to 1;

- Otherwise, SliceIbcDeblockingFilterDisabled is set equal to 0.

- 8.8.3.5	Derivation process of boundary filtering strength

…

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with IBC prediction mode, and SliceIbcDeblockingFilterDisabled is equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - …

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

    - …

Solution 1.2

- 7.3.2.6 Picture header RBSP syntax

| … |  |
| --- | --- |
| if( deblocking_filter_override_enabled_flag ) { |  |
| pic_deblocking_filter_override_present_flag | u(1) |
| if( pic_deblocking_filter_override_present_flag ) { |  |
| pic_deblocking_filter_override_flag | u(1) |
| if( pic_deblocking_filter_override_flag ) { |  |
| pic_deblocking_filter_disabled_flag | u(1) |
| if( !pic_deblocking_filter_disabled_flag ) { |  |
| pic_beta_offset_div2 | se(v) |
| pic_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( !pic_deblocking_filter_disabled_flag && sps_ibc_enabled_flag) |  |
| pic_ibc_deblocking_filter_disabled_flag | u(1) |
| … |  |

- 7.4.3.6 Picture header RBSP semantics

pic_ibc_deblocking_filter_disabled_flag equal to 1 specifies that the operation of the deblocking filter is not applied for edges with samples from both sides are coded in IBC mode in the current slice. pic_ibc_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter for samples coded in IBC mode may be applied for the current slice. When pic_ibc_deblocking_filter_disabled_flag is not present, it is inferred to be equal to 0.

- 7.8.4.1 General slice header semantics

Variable SliceIbcDeblockingFilterDisabled is derived as follows:

- If pic_ibc_deblocking_filter_disabled_flag is equal to 1 and slice_deblocking_filter_disabled_flag is equal to 0, SliceIbcDeblockingFilterDisabled is set equal to 1;

- Otherwise, SliceIbcDeblockingFilterDisabled is set equal to 0.

- 8.8.3.5	Derivation process of boundary filtering strength

…

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if both the sample p0 and q0 are in the coding block of a coding unit coded with IBC prediction mode, and SliceIbcDeblockingFilterDisabled is equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - …

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and or the coding subblock containing the sample q0 is are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

    - …

…

Solution 2.1

- 7.3.7.1	General slice header syntax

| … |  |
| --- | --- |
| if( deblocking_filter_override_enabled_flag && !pic_deblocking_filter_override_present_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( !slice_deblocking_filter_disabled_flag && sps_ibc_enabled_flag) |  |
| slice_ibc_deblocking_filter_disabled_flag | u(1) |
| … |  |

- 7.8.4.1 General slice header semantics

slice_ibc_deblocking_filter_disabled_flag equal to 1 specifies that the operation of the deblocking filter is not applied for edges with samples from either side are coded in IBC mode in the current slice. slice_ibc_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter for samples coded in IBC mode may be applied for the current slice. When slice_ibc_deblocking_filter_disabled_flag is not present, it is inferred to be equal to 0.

- 8.8.3.5	Derivation process of boundary filtering strength

…

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with IBC prediction mode, and slice_ibc_deblocking_filter_disabled_flag is equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - …

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and the coding subblock containing the sample q0 are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

    - …

…

Solution 2.2

- 7.3.7.1	General slice header syntax

| … |  |
| --- | --- |
| if( deblocking_filter_override_enabled_flag && !pic_deblocking_filter_override_present_flag ) |  |
| slice_deblocking_filter_override_flag | u(1) |
| if( slice_deblocking_filter_override_flag ) { |  |
| slice_deblocking_filter_disabled_flag | u(1) |
| if( !slice_deblocking_filter_disabled_flag ) { |  |
| slice_beta_offset_div2 | se(v) |
| slice_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| if( !slice_deblocking_filter_disabled_flag && sps_ibc_enabled_flag) |  |
| slice_ibc_deblocking_filter_disabled_flag | u(1) |
| … |  |

- 7.8.4.1 General slice header semantics

slice_ibc_deblocking_filter_disabled_flag equal to 1 specifies that the operation of the deblocking filter is not applied for edges with samples from either side are coded in IBC mode in the current slice. slice_ibc_deblocking_filter_disabled_flag equal to 0 specifies that the operation of the deblocking filter for samples coded in IBC mode may be applied for the current slice. When slice_ibc_deblocking_filter_disabled_flag is not present, it is inferred to be equal to 0.

- 8.8.3.5	Derivation process of boundary filtering strength

…

- The variable bS[ xDi ][ yDj ] is derived as follows:

    - If cIdx is equal to 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_luma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if cIdx is greater than 0 and both samples p0 and q0 are in a coding block with intra_bdpcm_chroma_flag equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if both the sample p0 and q0 are in the coding block of a coding unit coded with IBC prediction mode, and slice_ibc_deblocking_filter_disabled_flag is equal to 1, bS[ xDi ][ yDj ] is set equal to 0.

    - Otherwise, if the sample p0 or q0 is in the coding block of a coding unit coded with intra prediction mode, bS[ xDi ][ yDj ] is set equal to 2.

    - …

    - Otherwise, if cIdx is equal to 0, edgeFlags[ xDi ][ yDj ] is equal to 2, and one or more of the following conditions are true, bS[ xDi ][ yDj ] is set equal to 1:

      - The coding subblock containing the sample p0 and or the coding subblock containing the sample q0 is are both coded in IBC prediction mode, and the absolute difference between the horizontal or vertical component of the block vectors used in the prediction of the two coding subblocks is greater than or equal to 8 in units of 1/16 luma samples.

    - …

…

Conclusion

In this contribution, a high-level control flag for IBC mode deblocking operation is proposed. With this control flag, an encoder can choose to turn off the deblocking for IBC if that can help to improve visual quality.

References

[1]	B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019

[2] B. Ray, G. Van der Auwera, M. Kaczewicz, “Non-CE5/AHG-11: Boundary strength harmonization for BDPCM, TS, Palette and IBC”, JVET-P0611, Oct. 2019

[3] H.Jang, J.Nam, S.Kim, J.Lim, “Non-CE11: Considering boundary strength on CPR coded block boundary”, JVET-M0336, Jan. 2018

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).