| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0428-v3 |
| --- | --- |

| Title: | AHG 9: Picture header syntax cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xiaozhong Xu, Byeongdoo Choi, Ling Li, Xiang Li, Stephan Wenger, Shan Liu | Email: | xiaozhongxu@tencent.com bdchoi@tencent.com aurali@tencent.com |
| Source: | Tencent |  |  |

_____________________________

- Abstract

In this contribution, some syntax cleanups/grouping are suggested for the picture header syntax design. By the suggested cleanups, some syntax elements do not need to be signaled for the picture associated with the PH if they are completely not used in that picture. For example, if only I slice is used/allowed in a picture associated with the PH, syntax elements related to inter coding tools in the PH can be skipped.

In the first revision (v2), solution 3 with two independent flags controlling the presence of intra- /inter- related syntax was provided.

In the second revision (v3), a revised text of solution 2 was provided in the separate file.

Problem Statement

In the current VVC design [1], some syntax elements are moved from previously slice header to the picture header. In order to support all possible scenarios, syntax elements that are related to I slice and P/B slices are both signaled in each PH. In general use cases, there is only one slice in a picture (such as in CTC) or slices in the same picture are set to be with the same type. The current design in PH will signal a group of unused syntax elements.

Proposed solutions

In this contribution, it is suggested that unused syntax elements do not need to be signaled if only certain slice types are used in the picture associated with the PH. In particular, for a picture with only I slice allowed, syntax elements related to inter coding tools do not need to be signaled. Further, for a picture with only P/B slices allowed, syntax elements related to dual-tree structure do not need to be signaled.

In solution 1, a syntax element is used to indicate if all the slices in the picture associated with the PH has only I slice type or has only P or B slice types. If this is the case, a second syntax element is signaled to indicate whether all the slices the picture are I slice (with flag value 1) or P/B slices (with flag value 0).

In solution 2, two syntax elements at PH are signaled. One syntax element is signaled to indicate if at least one slice in the picture uses inter slice types; the other syntax element is conditionally signaled (when the first syntax is equal to 1) to indicate if at least one slice in the picture uses intra slice type.

In solution 3, two syntax elements at PH are signaled independently. In particular, one syntax element is signaled to indicate if at least one slice in the picture uses intra slice type; the other syntax element is signaled to indicate if at least one slice in the picture uses inter slice types.

Proposed text changes

Below are the related text changes.

Solution 1:

- 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( sps_poc_msb_flag ) { |  |
| ph_poc_msb_present_flag | u(1) |
| if( ph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| pic_single_coding_type_flag | u(1) |
| if( pic_single_coding_type_flag ) |  |
| pic_intra_picture_pred_only_flag | u(1) |
| … |  |

| … |  |
| --- | --- |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| if( !pic_single_coding_type_flag ¦ ¦ pic_intra_picture_pred_only_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| if( !pic_intra_picture_pred_only_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } } |  |
| } |  |
| } |  |
| if( !pic_single_coding_type_flag ¦ ¦ pic_intra_picture_pred_only_flag ) { |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| } |  |
| if( !pic_intra_picture_pred_only_flag ) { |  |
| if( cu_qp_delta_enabled_flag ) |  |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| if ( sps_ibc_enabled_flag ) |  |
| pic_six_minus_max_num_ibc_merge_cand | ue(v) |
| … |  |

- 7.4.3.6 Picture header RBSP semantics

pic_single_coding_type_flag specifies whether all the slices in the picture associated with the PH use only I slice type or only P/B slice types. If pic_single_coding_type_flag is equal to 1, all the slices associated with the PH shall choose slice_type all equal to I_slice, or all equal to inter slice types (P_slice or B_slice). Otherwise (pic_single_coding_type_flag is equal to 0), all slice types may be used in the slices of the picture associated with the PH. When not present, the value of pic_single_coding_type_flag is inferred to be equal to 0.

pic_intra_picture_pred_only_flag equal to 1 specifies that slice_type equal to I shall be true for all slices in the picture associated with the PH. pic_intra_picture_pred_only_flag equal to 0 specifies that slice_type equal to P or B shall be true for all the slices associated with the PH. When not present, the value of pic_intra_picture_pred_only_flag is inferred to be equal to 0.

Solution 2:

Please refer to the attached file for the updated text of solution 2, per meeting discussion. Below text is outdated.

- 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( sps_poc_msb_flag ) { |  |
| ph_poc_msb_present_flag | u(1) |
| if( ph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| pic_inter_ slice_only_flag | u(1) |
| if( !pic_inter_slice_only_flag ) |  |
| pic_intra_slice_only_flag | u(1) |
| … |  |

| … |  |
| --- | --- |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| if( !pic_inter_slice_only_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| if( !pic_intra_slice_only_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } } |  |
| } |  |
| } |  |
| if( ! pic_inter_slice_only_flag ) { |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| } |  |
| if( !pic_intra_slice_only_flag ) { |  |
| if( cu_qp_delta_enabled_flag ) |  |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| if ( sps_ibc_enabled_flag ) |  |
| pic_six_minus_max_num_ibc_merge_cand | ue(v) |
| … |  |

- 7.4.3.6 Picture header RBSP semantics

pic_inter_slice_only_flag equal to 1 specifies that a slice with slice_type equal to I cannot exist in the picture associated with the PH. pic_intra_slice_only_flag is equal to 0 specifies that a slice with slice_type equal to I may exist in the picture associated with the PH. When not present, the value of pic_inter_slice_only_flag is inferred to be equal to 0.

pic_intra_slice_only_flag equal to 1 specifies that a slice with slice_type equal to P or B cannot exist in the picture associated with the PH. pic_intra_slice_only_flag is equal to 0 specifies that a slice with slice_type equal to P or B may exist in the picture associated with the PH. When not present, the value of pic_intra_slice_only_flag is inferred to be equal to 0.

Solution 3:

- 7.3.2.6 Picture header RBSP syntax

| picture_header_rbsp( ) { | Descriptor |
| --- | --- |
| non_reference_picture_flag | u(1) |
| gdr_pic_flag | u(1) |
| no_output_of_prior_pics_flag | u(1) |
| if( gdr_pic_flag ) |  |
| recovery_poc_cnt | ue(v) |
| ph_pic_parameter_set_id | ue(v) |
| if( sps_poc_msb_flag ) { |  |
| ph_poc_msb_present_flag | u(1) |
| if( ph_poc_msb_present_flag ) |  |
| poc_msb_val | u(v) |
| } |  |
| if( sps_subpic_id_present_flag && !sps_subpic_id_signalling_flag ) { |  |
| ph_subpic_id_signalling_present_flag | u(1) |
| if( ph_subpics_id_signalling_present_flag ) { |  |
| ph_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |
| ph_subpic_id[ i ] | u(v) |
| } |  |
| } |  |
| pic_inter_ slice_present_flag | u(1) |
| pic_intra_slice_present_flag | u(1) |
| … |  |

| … |  |
| --- | --- |
| if( partition_constraints_override_enabled_flag ) { |  |
| partition_constraints_override_flag | u(1) |
| if( partition_constraints_override_flag ) { |  |
| if( pic_intra_slice_present_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| if( qtbtt_dual_tree_intra_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| pic_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( pic_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| pic_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| if( pic_inter_slice_present_flag ) { |  |
| pic_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| pic_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( pic_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| pic_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| pic_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } } |  |
| } |  |
| } |  |
| if( pic_intra_slice_present_flag ) { |  |
| if( cu_qp_delta_enabled_flag ) { |  |
| pic_cu_qp_delta_subdiv_intra_slice | ue(v) |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| } |  |
| if( pic_inter_slice_present_flag ) { |  |
| if( cu_qp_delta_enabled_flag ) |  |
| pic_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) { |  |
| pic_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| pic_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) |  |
| pic_temporal_mvp_enabled_flag | u(1) |
| if(!pps_mvd_l1_zero_idc ) |  |
| mvd_l1_zero_flag | u(1) |
| if( !pps_six_minus_max_num_merge_cand_plus1 ) |  |
| pic_six_minus_max_num_merge_cand | ue(v) |
| if( sps_affine_enabled_flag ) |  |
| pic_five_minus_max_num_subblock_merge_cand | ue(v) |
| if( sps_fpel_mmvd_enabled_flag ) |  |
| pic_fpel_mmvd_enabled_flag | u(1) |
| if( sps_bdof_pic_present_flag ) |  |
| pic_disable_bdof_flag | u(1) |
| if( sps_dmvr_pic_present_flag ) |  |
| pic_disable_dmvr_flag | u(1) |
| if( sps_prof_pic_present_flag ) |  |
| pic_disable_prof_flag | u(1) |
| if( sps_triangle_enabled_flag && MaxNumMergeCand >= 2 && !pps_max_num_merge_cand_minus_max_num_triangle_cand_plus1 ) |  |
| pic_max_num_merge_cand_minus_max_num_triangle_cand | ue(v) |
| } |  |
| if ( sps_ibc_enabled_flag ) |  |
| pic_six_minus_max_num_ibc_merge_cand | ue(v) |
| … |  |

- 7.4.3.6 Picture header RBSP semantics

pic_inter_slice_present_flag equal to 1 specifies that a slice with slice_type equal to B or P may be present in the picture associated with the PH. pic_inter_slice_present_flag is equal to 0 specifies that no slice with slice_type equal to B or P may be present in the picture associated with the PH. When not present, the value of pic_inter_slice_present_flag is inferred to be equal to 1.

pic_intra_slice_present_flag equal to 1 specifies that a slice with slice_type equal to I may be present in the picture associated with the PH. pic_intra_slice_present_flag is equal to 0 specifies that no slice with slice_type equal to I may be present in the picture associated with the PH. When not present, the value of pic_intra_slice_present_flag is inferred to be equal to 1.

Conclusion

In this contribution, cleanups on the PH syntax design are suggested. Only when I slice may be presented in the picture, should the PH include intra slice related syntax elements; only when P/B slices may be presented in the picture, should the PH include inter slice related syntax elements. It is proposed to include suggested solution 2 as a cleanup solution into next version of VVC spec.

The authors would like to thank experts from ByteDance, LGE, Qualcomm, Canon, Sharp, MediaTek, Broadcom, HHI, Nokia for valuable suggestions and extensive involvement of discussion during the meeting.

References

[1]	B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001, Oct. 2019

Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).