| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0429 |
| --- | --- |

| Title: | CE4-related: Geometric partitioning signaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | T. Poirier, F. Le Léannec, F. Urban A. Robert | Tel: Email: | tangi.poirier@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to replace look-up table based signaling of merge geo partition index with a merge geo partition distance and merge geo partition angle indexes. Test results reportedly show -0.30% and -0.68% BD rate reduction over VTM-7.0 for RA and LB configurations, respectively.

# Introduction

In geometric merge mode, a CU is split into 2 PUs using one of the 82 supported partitions. A partition is a combination of an angle and a distance. A total of 4 distances and 24 angles are used. For distance equal to 0, only 10 angles are used.  A look-up Table is used to derive angle and distance from geo partition index:

| geo_partition_idx | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 | 13 | 14 | 15 | 16 | 17 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| angleIdx | 0 | 0 | 0 | 1 | 1 | 1 | 1 | 2 | 2 | 2 | 2 | 3 | 3 | 3 | 3 | 4 | 4 | 4 |
| distanceIdx | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 |
| geo_partition_idx | 18 | 19 | 20 | 21 | 22 | 23 | 24 | 25 | 26 | 27 | 28 | 29 | 30 | 31 | 32 | 33 | 34 | 35 |
| angleIdx | 4 | 5 | 5 | 5 | 5 | 6 | 6 | 6 | 7 | 7 | 7 | 7 | 8 | 8 | 8 | 8 | 9 | 9 |
| distanceIdx | 3 | 0 | 1 | 2 | 3 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 |
| geo_partition_idx | 36 | 37 | 38 | 39 | 40 | 41 | 42 | 43 | 44 | 45 | 46 | 47 | 48 | 49 | 50 | 51 | 52 | 53 |
| angleIdx | 9 | 9 | 10 | 10 | 10 | 10 | 11 | 11 | 11 | 11 | 12 | 12 | 12 | 13 | 13 | 13 | 14 | 14 |
| distanceIdx | 2 | 3 | 0 | 1 | 2 | 3 | 0 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 |
| geo_partition_idx | 54 | 55 | 56 | 57 | 58 | 59 | 60 | 61 | 62 | 63 | 64 | 65 | 66 | 67 | 68 | 69 | 70 | 71 |
| angleIdx | 14 | 15 | 15 | 15 | 16 | 16 | 16 | 17 | 17 | 17 | 18 | 18 | 18 | 19 | 19 | 19 | 20 | 20 |
| distanceIdx | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 |
| geo_partition_idx | 72 | 73 | 74 | 75 | 76 | 77 | 78 | 79 | 80 | 81 |  |  |  |  |  |  |  |  |
| angleIdx | 20 | 21 | 21 | 21 | 22 | 22 | 22 | 23 | 23 | 23 |  |  |  |  |  |  |  |  |
| distanceIdx | 3 | 1 | 2 | 3 | 1 | 2 | 3 | 1 | 2 | 3 |  |  |  |  |  |  |  |  |

# Proposed method

In this contribution, the look-up table is removed and instead of combining the angle and distance indexes into a single value, 2 values are encoded into the bitstream:

- The geo distance index is encoded using 2 bypass bins.

- The geo angle index is encoded using the method described in next section 2.1, with a maximum value of 24 (only 10 when distance is equal to 0).

Proposed syntax modification is provided in the table below:

| if( !ciip_flag[ x0 ][ y0 ] && MaxNumGeoMergeCand > 1 ) { |  |
| --- | --- |
| merge_geo_partition_idx[ x0 ][ y0 ] | ae(v) |
| merge_geo_partition_distance_idx | ae(v) |
| merge_geo_partition_angle_idx | ae(v) |
| merge_geo_idx0[ x0 ][ y0 ] | ae(v) |
| if( MaxNumGeoMergeCand > 2 ) |  |
| merge_geo_idx1[ x0 ][ y0 ] | ae(v) |
| } |  |

## Coding tree used for signaling the angle

In this variant, the merge_geo_partition_angle_idx syntax element is coded using a tree depicted in the following figure:

Figure 1: Proposed coding for merge_geo_partition_angle_idx

# Test results

The proposed method was implemented on top of VTM-7.0_CE4_common_base software and tested under common test conditions.

The following Table 1 shows the performance obtained, compared to VTM-7.0. These results are to be compared with the performances of the GEO common base, which are recalled in the following Table 2. It can be observed that the RA performances are being slightly improved in every class compared to the GEO common base.

With respect to low-delay B performances, it can be seen that class B and class C performances are being slightly improved, while the overall average coding efficient is almost unchanged.

Table 1: performances of the proposed method compared to VTM-7.0.

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.20% | -0.18% | 102% | 101% |
| Class A2 | -0.23% | -0.17% | -0.08% | 104% | 100% |
| Class B | -0.17% | -0.13% | -0.15% | 103% | 101% |
| Class C | -0.63% | -0.94% | -0.86% | 103% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.30% | -0.37% | -0.33% | 103% | 100% |
| Class D | -0.45% | -0.68% | -0.61% | 104% | 101% |
| Class F | -0.32% | -0.37% | -0.29% | 101% | 99% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.37% | -0.45% | -0.51% | 103% | 100% |
| Class C | -0.81% | -1.26% | -0.91% | 104% | 100% |
| Class E | -1.02% | -0.18% | -0.93% | 102% | 99% |
| Overall | -0.68% | -0.65% | -0.75% | 103% | 100% |
| Class D | -0.68% | -0.51% | -0.63% | 103% | 100% |
| Class F | -0.57% | -1.35% | 0.00% | 102% | 100% |

Table 2: performance of the of the GEO common base, i.e. VTM-7.0_CE4_common_base software, over VTM-7.0

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.09% | -0.18% | 103% | 101% |
| Class A2 | -0.22% | -0.19% | -0.02% | 103% | 100% |
| Class B | -0.14% | -0.10% | -0.17% | 103% | 101% |
| Class C | -0.59% | -0.99% | -0.90% | 103% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.28% | -0.35% | -0.34% | 103% | 100% |
| Class D | -0.43% | -0.67% | -0.62% | 103% | 100% |
| Class F (optional) | -0.32% | -0.38% | -0.33% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.36% | -0.51% | -0.60% | 104% | 99% |
| Class C | -0.79% | -1.25% | -0.89% | 104% | 100% |
| Class E | -1.17% | -0.19% | -0.88% | 101% | 101% |
| Overall | -0.70% | -0.67% | -0.77% | 103% | 100% |
| Class D | -0.68% | -0.44% | -1.04% | 103% | 99% |
| Class F (optional) | -0.51% | -0.90% | -0.30% | 102% | 100% |

# Conclusion

This contribution proposed to decompose the syntax element merge_geo_partition_idx of VTM-7.0 into two separate syntax elements merge_geo_partition_distance_idx and merge_geo_partition_angle_idx.

It is claimed that the proposed change makes the GEO specification cleaner and removes the use of some lookup table. Moreover, the propsosed modification slightly improves the coding efficiency of the GEO common base in RA in every class, and LDB performances in classes B and C.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).