| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0433-v1 |
| --- | --- |

| Title: | Encoder only: On unbalanced luma/chroma gains for dependent quantization |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Heiko Schwarz | Email: | heiko.schwarz@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution discusses the unbalanced luma/chroma gains for dependent quantization in recent versions of the VTM reference software.  The contribution suggests a modification of the chroma distortion weight, which was reportedly already integrated in VTM-2, but later became ineffective when LFNST is enabled.  For measuring the overall coding efficiency, two YUV metrics are additionally evaluated: YUVa uses a weighted PSNR measure with the weighting factors 6, 1, and 1 for luma, Cb, and Cr, respectively; YUVb (which is also used by AHG13) represents a weighted BD rate measure with the weighting factors 8, 1, and 1 for luma, Cb, and Cr, respectively.

It is reported that the proposed encoder modification yields the following results over VTM-7.0:

AI:		0.26%, -1.96%, -1.85%  (Y, Cb, Cr)  and  -0.17% (YUVa), -0.17% (YUVb)
	RA:	0.16%, -1.24%, -1.21%  (Y, Cb, Cr)  and  -0.14% (YUVa), -0.12% (YUVb)
	LB:		0.00%,  0.00%,   0.00%  (Y, Cb, Cr)  and   0.00% (YUVa),  0.00% (YUVb)
	LP:		0.00%,  0.00%,   0.00%  (Y, Cb, Cr)  and   0.00% (YUVa),  0.00% (YUVb)

In comparison to VTM-7.0 with dependent quantization disabled and sign data hiding enabled, the following results are reported:

AI:		-1.69%, -1.30%, -1.13%  (Y, Cb, Cr)  and  -1.62% (YUVa), -1.59% (YUVb)
	RA:	-1.57%, -0.98%, -0.69%  (Y, Cb, Cr)  and  -1.40% (YUVa), -1.42% (YUVb)
	LB:		-1.53%, -0.28%,  0.06%  (Y, Cb, Cr)  and  -1.27% (YUVa), -1.24% (YUVb)
	LP:		-1.54%, -0.55%,  0.03%  (Y, Cb, Cr)  and  -1.31% (YUVa), -1.29% (YUVb)

It is asserted that the proposed encoder modification yields more balanced luma/chroma gains for dependent quantization; moreover, the overall coding efficiency for two often used YUV metrics is slightly improved.

# Discussion

In recent AHG13 tool-off tests for dependent quantization, unbalanced luma/chroma gains are reported. While rather large (> 1.5%) bit-rate savings for luma are observed, there are losses for chroma. Reasons for this behavior and an encoder-only adjustment for obtaining more balanced luma/chroma gains are briefly discussed in the following.

Table 1:  Summary of tool-off tests for dependent quantization. The data are taken from the AHG13 reports. The last two lines report additional low QP results (QPs 2, 7, 12, 17 and 100 frames) for VTM-6.0 and VTM-7.0.

| AHG13 tool-off test | All Intra |  |  | Random Access |  |  | Low Delay B |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | Cb | Cr | Y | Cb | Cr | Y | Cb | Cr |
| VTM-2.0 (vs SDH=0) | 2.44% | 2.03% | 1.75% | 2.18% | 1.20% | 0.82% | 1.82% | 1.70% | 0.88% |
| VTM-3.0 | 1.91% | 1.15% | 0.86% | 1.66% | 0.51% | 0.06% | 1.48% | 0.86% | -0.31% |
| VTM-4.0 | 1.84% | 1.04% | 0.73% | 1.57% | 0.78% | 0.25% | 1.33% | 1.47% | 0.57% |
| VTM-5.0 | 1.90% | -0.94% | -0.95% | 1.71% | -0.51% | -0.72% | 1.37% | 1.78% | 1.42% |
| VTM-6.0 | 2.03% | -0.84% | -0.91% | 1.76% | -0.24% | -0.61% | 1.33% | 1.49% | 1.45% |
| VTM-7.0 | 1.99% | -0.66% | -0.73% | 1.76% | -0.25% | -0.52% | 1.56% | 0.29% | -0.05% |
| add. data for low QP |  |  |  |  |  |  |  |  |  |
| VTM-6.0 (low QP) | 1.45% | 2.66% | 2.71% | 2.52% | 3.78% | 3.10% | 2.51% | 5.75% | 4.92% |
| VTM-7.0 (low QP) | 1.45% | 2.54% | 2.55% | 2.58% | 2.40% | 1.25% | 3.01% | 4.30% | 3.25% |

Table 1 summarizes the tool-off test results for dependent quantization (as reported by AHG13) for different versions of the VTM.  As can be seen, up to VTM-4, there were gains in both luma and chroma for all configurations.  Starting from VTM-5, the behavior changed for the all intra and random access configurations.  While the luma gains increased, there were losses for chroma. The reason for this is a change in the chroma distortion weight (or Lagrange parameter for chroma), as will be further discussed below.

Dependent quantization has the property that the obtainable coding gains depend on the number of significant transform coefficients in a transform block.  In fact, dependent quantization can only result in coding gains if a transform block has at least two significant coefficients.  The achievable gain increases with the number of significant transform coefficients.  For the bit rate range that corresponds to the common test conditions, the coding gains for luma are significantly higher than for chroma.  Due to the way the luma and chroma BD rates are calculated in JVET experiments, this often results in savings in luma BD rate and losses in chroma BD rates.  In order to reduce this bit rate shift from chroma to luma, the original implementation of dependent quantization in VTM-2 included a slight increase of the chroma distortion weights (which corresponded to 1/10-th or 1/5-th of a QP).

When LFNST was integrated into VTM-5, the increase of the chroma distortion weight became ineffective for configurations in which LFNST is enabled (as tool-off tests of AHG13 for dependent quantization).  This is eventually a results of the addition of the yellow-marked code part:

void

EncSlice::setUpLambda( Slice* slice, const double dLambda, int iQP)

{

...

if( m_pcCfg->getDepQuantEnabledFlag() && !( m_pcCfg->getLFNST() ) )

{

tmpWeight *= ( m_pcCfg->getGOPSize() >= 8 ? pow( 2.0, 0.1/3.0 ) : pow( 2.0, 0.2/3.0 ) );  
    // increase chroma weight for dependent quantization 
    // (in order to reduce bit rate shift from chroma to luma)

}

...

}

As a result, starting from VTM-5, the tool-off tests for dependent quantization show losses in chroma (and slightly increased gains in luma).

# Proposed Encoder Modification

In order to obtain better balanced luma/chroma gains for dependent quantization as well as a slight improvement in overall coding efficiency, we suggest to basically revert the change introduced in VTM-5.  The increase of the chroma distortion weight should depend only on the usage of dependent quantization and not on the usage of LFNST.  Since for low QP values, no bit rate shift from luma to chroma is observed, the increase of the chroma distortion weight is disabled for low QPs.

Eventually, the following one-line encoder change is suggested:

void

EncSlice::setUpLambda( Slice* slice, const double dLambda, int iQP)

{

...

#if PROPOSED_ENCODER_MODIFICATION

if( m_pcCfg->getDepQuantEnabledFlag() && iQP > 18 )

#else

if( m_pcCfg->getDepQuantEnabledFlag() && !( m_pcCfg->getLFNST() ) )

#endif

{

tmpWeight *= ( m_pcCfg->getGOPSize() >= 8 ? pow( 2.0, 0.1/3.0 ) : pow( 2.0, 0.2/3.0 ) );  
    // increase chroma weight for dependent quantization 
    // (in order to reduce bit rate shift from chroma to luma)

}

...

}

# Experimental Results

The coding efficiency impact of the proposed encoder modification is tested on top of VTM-7.0.

In order to judge the overall coding efficiency, two commonly used YUV metrics are additionally measured:

- YUVa represents the weighted PSNR measure that is used in several coding efficiency comparison papers (e.g., Ohm, et. al., “Comparison of the Coding Efficiency of Video Coding Standards – Including High Efficiency Video Coding (HEVC), IEEE-TCSVT, 2012).  Using this measure, the YUV-PSNR is calculated according to

YUV-PSNR = ( 6 * Y-PSNR + Cb-PSNR + Cr-PSNR ) / 8,

where Y-PSNR, Cb-PSNR, and Cr-PSNR represent the PSNR values for luma, Cb, and Cr, respectively. Given the average YUV-PSNR values for the bitstreams, the corresponding BD rates are calculated as usual.

- YUVb represent a weighted BD rate measure that is used in reports of AHG13.  Given the BD rates for luma, Cb, and Cr (BDR-Y, BDR-Cb, and BDR-Cr), the YUV-BD rate is calculated according to

BDR-YUV = ( 8 * BDR-Y + BDR-Cb + BDR-Cr ) / 10.

Complete simulation results for SDR CTC, SDR low QP, and HDR CTC can be found in the accompanying Excel documents.

Table 2 summarizes the results of the proposed encoder modification relative to VTM-7 (with dependent quantization enabled) for SDR common test conditions.  Table 3 summarizes the results of the proposed encoder modification with dependent quantization enabled relative to VTM-7 with dependent quantization disabled and sign data hiding enabled for SDR common test conditions.  It can be observed that the proposed encoder modification yields more balanced luma/chroma gains for dependent quantization.  In addition, both evaluated YUV metrics indicate a slight overall coding gain over VTM-7 for the all intra and random access configurations.

Table 4 and Table 5 summarize the simulation results for the low QP configuration (QP values 2, 7, 12, 17 and 100 frames).  Here, the effect is negligible for all intra and random access.  For the low delay configuration, a very minor overall coding gain is observed (when evaluating the YUV metrics).  For the low QP scenario, there does not exist the problem of highly unbalanced luma/chroma gains.

Finally, Table 6 and Table 7 summarize the simulation results for the HDR common test conditions.  Similarly as for SDR, the proposed encoder modification results in more balanced coding gains for luma and chroma.  And both YUV metrics indicate an overall increase in coding efficiency.

Table 2:  Proposed encoder modification (DQ=1) vs VTM-7.0 (DQ=1) under common test conditions.

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class A1 | 0.31% | -2.14% | -1.65% | -0.28% | -0.13% | 101% | 100% |
| Class A2 | 0.51% | -2.19% | -2.17% | 0.02% | -0.03% | 101% | 102% |
| Class B | 0.20% | -2.16% | -2.18% | -0.22% | -0.27% | 101% | 97% |
| Class C | 0.19% | -1.57% | -1.53% | -0.14% | -0.16% | 101% | 96% |
| Class E | 0.16% | -1.71% | -1.63% | -0.23% | -0.21% | 100% | 102% |
| Overall | 0.26% | -1.96% | -1.85% | -0.17% | -0.17% | 101% | 99% |
| Class D | 0.18% | -1.60% | -1.52% | -0.13% | -0.17% | 100% | 100% |
| Class F | 0.19% | -1.15% | -1.01% | -0.07% | -0.06% | 101% | 101% |
|  | Random access Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class A1 | 0.25% | -1.24% | -1.01% | -0.16% | -0.03% | 100% | 97% |
| Class A2 | 0.26% | -1.32% | -1.25% | -0.08% | -0.05% | 102% | 96% |
| Class B | 0.11% | -1.32% | -1.40% | -0.18% | -0.19% | 100% | 99% |
| Class C | 0.09% | -1.07% | -1.09% | -0.14% | -0.14% | 100% | 99% |
| Overall | 0.16% | -1.24% | -1.21% | -0.14% | -0.12% | 101% | 98% |
| Class D | 0.07% | -1.27% | -1.17% | -0.18% | -0.19% | 101% | 103% |
| Class F | 0.09% | -0.59% | -0.78% | -0.08% | -0.07% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 97% | 99% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 98% |
|  | Low delay P Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 94% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 96% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 96% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 101% | 100% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 96% |

Table 3:  Proposed encoder modification (DQ=1) vs VTM-7.0 (DQ=0, SDH=1) under common test conditions.

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with DQ=0, SDH=1 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class A1 | -2.18% | -1.24% | -0.97% | -1.92% | -1.96% | 100% | 91% |
| Class A2 | -2.87% | -1.14% | -1.27% | -2.74% | -2.53% | 103% | 90% |
| Class B | -1.51% | -1.13% | -0.94% | -1.42% | -1.42% | 105% | 92% |
| Class C | -1.07% | -1.82% | -1.51% | -1.18% | -1.19% | 111% | 96% |
| Class E | -1.12% | -1.11% | -0.97% | -1.10% | -1.11% | 105% | 99% |
| Overall | -1.69% | -1.30% | -1.13% | -1.62% | -1.59% | 105% | 94% |
| Class D | -1.12% | -1.65% | -1.45% | -1.20% | -1.20% | 111% | 101% |
| Class F | -0.51% | -1.38% | -1.31% | -0.67% | -0.68% | 106% | 99% |
|  | Random access Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 with DQ=0, SDH=1 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class A1 | -1.66% | -0.77% | 0.32% | -1.30% | -1.37% | 98% | 97% |
| Class A2 | -1.70% | -0.87% | -0.94% | -1.51% | -1.54% | 102% | 99% |
| Class B | -1.80% | -0.56% | -0.52% | -1.56% | -1.55% | 100% | 96% |
| Class C | -1.11% | -1.73% | -1.48% | -1.20% | -1.21% | 104% | 98% |
| Overall | -1.57% | -0.98% | -0.69% | -1.40% | -1.42% | 101% | 97% |
| Class D | -1.23% | -1.72% | -1.26% | -1.28% | -1.28% | 104% | 98% |
| Class F | -0.61% | -1.71% | -1.76% | -0.85% | -0.84% | 103% | 98% |
|  | Low delay B Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 with DQ=0, SDH=1 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class B | -2.07% | -0.01% | 0.51% | -1.68% | -1.60% | 101% | 98% |
| Class C | -1.28% | -1.21% | -0.95% | -1.24% | -1.24% | 106% | 102% |
| Class E | -0.97% | 0.52% | 0.68% | -0.64% | -0.65% | 97% | 99% |
| Overall | -1.53% | -0.28% | 0.06% | -1.27% | -1.24% | 101% | 99% |
| Class D | -1.37% | -1.75% | -1.34% | -1.42% | -1.41% | 101% | 98% |
| Class F | -0.82% | -1.77% | -1.90% | -1.03% | -1.02% | 102% | 98% |
|  | Low delay P Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 with DQ=0, SDH=1 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class B | -2.05% | -0.58% | 0.50% | -1.72% | -1.64% | 100% | 99% |
| Class C | -1.32% | -1.76% | -1.00% | -1.32% | -1.33% | 105% | 98% |
| Class E | -1.00% | 1.13% | 0.64% | -0.60% | -0.62% | 98% | 99% |
| Overall | -1.54% | -0.55% | 0.03% | -1.31% | -1.29% | 101% | 98% |
| Class D | -1.50% | -1.94% | -1.53% | -1.55% | -1.54% | 105% | 97% |
| Class F | -0.77% | -1.51% | -2.50% | -1.03% | -1.02% | 103% | 97% |

Table 4:  Proposed encoder modification (DQ=1) vs VTM-7.0 (DQ=1) under low QP test conditions.

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 99% |
| Class B | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 103% | 98% |
| Class C | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 101% |
| Class E | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 97% |
| Overall | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 102% | 99% |
| Class D | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 100% | 102% |
| Class F | 0.00% | 0.00% | 0.00% | 0.00% | 0.00% | 99% | 100% |
|  | Random access Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class A1 | 0.01% | -0.08% | -0.09% | -0.01% | -0.01% | 100% | 100% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.02% | -0.06% | -0.04% | 0.00% | 0.00% | 103% | 98% |
| Class C | 0.01% | -0.05% | -0.05% | 0.00% | 0.00% | 99% | 100% |
| Overall |  |  |  |  |  |  |  |
| Class D | 0.01% | -0.03% | -0.04% | 0.00% | 0.00% | 103% | 97% |
| Class F | 0.00% | -0.01% | -0.04% | 0.00% | 0.00% | 101% | 99% |
|  | Low delay B Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class B | -0.19% | 0.51% | 0.56% | -0.03% | -0.04% | 101% | 97% |
| Class C | -0.19% | 0.57% | 0.51% | -0.03% | -0.04% | 101% | 96% |
| Class E | -0.26% | 1.21% | 1.01% | -0.03% | 0.01% | 101% | 98% |
| Overall | -0.21% | 0.71% | 0.66% | -0.03% | -0.03% | 101% | 97% |
| Class D | -0.18% | 0.55% | 0.46% | -0.04% | -0.04% | 101% | 93% |
| Class F | -0.23% | 0.51% | 0.44% | -0.07% | -0.09% | 102% | 98% |
|  | Low delay P Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class B | -0.20% | 0.49% | 0.51% | -0.05% | -0.06% | 99% | 96% |
| Class C | -0.18% | 0.51% | 0.47% | -0.04% | -0.05% | 103% | 100% |
| Class E | -0.25% | 1.04% | 0.92% | -0.04% | 0.00% | 101% | 98% |
| Overall | -0.21% | 0.63% | 0.60% | -0.05% | -0.04% | 101% | 98% |
| Class D | -0.14% | 0.54% | 0.51% | 0.00% | -0.01% | 104% | 93% |
| Class F | -0.25% | 0.35% | 0.41% | -0.11% | -0.12% | 104% | 99% |

Table 5:  Proposed encoder modification (DQ=1) vs VTM-7.0 (DQ=0, SDH=1) under low QP test conditions.

|  | All Intra Main10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with DQ=0, SDH=1 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class A1 | -0.51% | -1.86% | -1.85% | -0.81% | -0.78% | 120% | 83% |
| Class A2 | -1.72% | -3.08% | -3.16% | -2.04% | -2.00% | 119% | 84% |
| Class B | -1.76% | -2.29% | -2.36% | -1.90% | -1.87% | 115% | 90% |
| Class C | -1.04% | -2.54% | -2.51% | -1.41% | -1.34% | 125% | 96% |
| Class E | -1.99% | -2.69% | -2.59% | -2.14% | -2.12% | 117% | 89% |
| Overall | -1.42% | -2.47% | -2.48% | -1.67% | -1.63% | 119% | 89% |
| Class D | -1.05% | -2.18% | -2.15% | -1.33% | -1.27% | 121% | 94% |
| Class F | -0.63% | -1.72% | -1.63% | -0.84% | -0.84% | 111% | 97% |
|  | Random access Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 with DQ=0, SDH=1 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class A1 | -2.12% | -2.25% | -1.50% | -2.11% | -2.07% | 130% | 88% |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -2.42% | -1.44% | 1.42% | -1.96% | -1.94% | 115% | 90% |
| Class C | -2.99% | -3.37% | -2.48% | -3.00% | -2.98% | 116% | 95% |
| Overall |  |  |  |  |  |  |  |
| Class D | -2.55% | -3.16% | -2.66% | -2.64% | -2.62% | 115% | 96% |
| Class F | -0.84% | -2.38% | -2.08% | -1.09% | -1.12% | 108% | 94% |
|  | Low delay B Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 with DQ=0, SDH=1 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class B | -2.55% | -2.96% | -2.16% | -2.57% | -2.55% | 117% | 93% |
| Class C | -2.98% | -3.71% | -2.85% | -3.06% | -3.04% | 115% | 95% |
| Class E | -4.22% | -3.74% | -2.46% | -4.05% | -4.00% | 115% | 89% |
| Overall | -3.11% | -3.41% | -2.47% | -3.10% | -3.08% | 116% | 93% |
| Class D | -2.55% | -3.51% | -3.19% | -2.73% | -2.71% | 115% | 99% |
| Class F | -1.91% | -3.02% | -2.44% | -2.10% | -2.07% | 106% | 99% |
|  | Low delay P Main10 |  |  |  |  |  |  |
|  | Over VTM-7.0 with DQ=0, SDH=1 |  |  |  |  |  |  |
|  | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class B | -2.47% | -3.23% | -2.35% | -2.55% | -2.53% | 117% | 94% |
| Class C | -2.59% | -4.38% | -3.10% | -2.85% | -2.82% | 118% | 96% |
| Class E | -3.63% | -3.97% | -2.50% | -3.57% | -3.55% | 114% | 88% |
| Overall | -2.80% | -3.80% | -2.64% | -2.91% | -2.88% | 117% | 93% |
| Class D | -2.27% | -4.05% | -3.64% | -2.61% | -2.59% | 118% | 99% |
| Class F | -1.86% | -3.46% | -2.66% | -2.13% | -2.10% | 106% | 98% |

Table 6:  Proposed encoder modification (DQ=1) vs VTM-7.0 (DQ=1) under common HDR test conditions.

|  | All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  |  |  | PSNR |  |  |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | YUVa | YUVb | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class H1 | -1.71% | 0.24% | 0.24% | -3.33% | -4.34% | -0.14% | -0.57% | 0.24% | -3.51% | -4.66% | -0.16% | -0.63% | 101% | 98% |
| Class H2 |  |  |  |  |  |  |  | 0.16% | -1.79% | -2.11% | -0.11% | -0.26% | 100% | 103% |
| Overall | -1.71% | 0.24% | 0.24% | -3.33% | -4.34% | -0.14% | -0.57% | 0.21% | -2.88% | -3.73% | -0.14% | -0.49% | 101% | 100% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  |  |  | PSNR |  |  |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | YUVa | YUVb | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class H1 | -0.95% | 0.11% | 0.11% | -2.12% | -2.43% | -0.15% | -0.37% | 0.10% | -2.25% | -2.56% | -0.16% | -0.40% | 101% | 99% |
| Class H2 |  |  |  |  |  |  |  | 0.06% | -1.53% | -1.67% | -0.16% | -0.27% | 101% | 100% |
| Overall | -0.95% | 0.11% | 0.11% | -2.12% | -2.43% | -0.15% | -0.37% | 0.09% | -1.99% | -2.24% | -0.16% | -0.35% | 101% | 99% |

Table 7:  Proposed encoder modification (DQ=1) vs VTM-7.0 (DQ=0, SDH=1) under common HDR test conditions.

|  | All Intra |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 with DQ=0, SDH=1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  |  |  | PSNR |  |  |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | YUVa | YUVb | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class H1 | -0.05% | -1.34% | -1.43% | -0.11% | 0.08% | -1.29% | -1.15% | -1.39% | -0.32% | -0.25% | -1.29% | -1.17% | 104% | 97% |
| Class H2 |  |  |  |  |  |  |  | -1.65% | -0.50% | -0.67% | -1.54% | -1.44% | 101% | 98% |
| Overall | -0.05% | -1.34% | -1.43% | -0.11% | 0.08% | -1.29% | -1.15% | -1.49% | -0.38% | -0.41% | -1.38% | -1.27% | 103% | 98% |
|  | Random Access |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Over VTM-7.0 with DQ=0, SDH=1 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  | wPSNR |  |  |  |  | PSNR |  |  |  |  |  |  |
|  | DE100 | PSNR-L100 | Y | U | V | YUVa | YUVb | Y | U | V | YUVa | YUVb | EncT | DecT |
| Class H1 | 0.68% | -1.44% | -1.42% | 1.37% | 1.92% | -1.10% | -0.80% | -1.36% | 1.32% | 1.72% | -1.07% | -0.79% | 101% | 98% |
| Class H2 |  |  |  |  |  |  |  | -1.70% | -1.08% | -0.90% | -1.60% | -1.55% | 99% | 100% |
| Overall | 0.68% | -1.44% | -1.42% | 1.37% | 1.92% | -1.10% | -0.80% | -1.48% | 0.44% | 0.77% | -1.26% | -1.07% | 100% | 98% |

# Summary

We propose a one-line encoder modification, which results in more balanced luma/chroma gains for dependent quantization.  Furthermore, the evaluated YUV metrics indicate that the proposed encoder modification yields a slight overall coding gain.

We suggest to integrate this change into the next VTM software.

# Patent rights declaration(s)

Fraunhofer HHI does not have current or pending patent rights relating to the technology described in this contribution.