| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0436 |
| --- | --- |

| Title: | CABAC zero word threshold |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information / Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Karl Sharman Steve Keating | Email: | adrian.browne@sony.com karl.sharman@sony.com steve.keating@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

This document presents an analysis of the need for padding with cabac_zero_word. Following on from JVET-P0395, where CTU-level statistics were analysed, in this contribution statistics are provided for tiles of CTUs, to examine the impact of averaging. It is concluded that padding is required on tiles for some sequences within both “High” and “Main” tiers of the current specification.  In addition, figures are given which indicate that these results are not “averaged out” as tiles become larger.  Two possible changes to the cabac_zero_word threshold are proposed if the impact of padding is to be reduced.

# Introduction

Section 7.4.3.13 of the specification [1] states the following:

Let NumBytesInVclNalUnits be the sum of the values of NumBytesInNalUnit for all VCL NAL units of a coded picture.

Let BinCountsInNalUnits be the number of times that the parsing process function DecodeBin( ), specified in clause TBD, is invoked to decode the contents of all VCL NAL units of a coded picture.

Let the variable RawMinCuBits be derived as follows:

RawMinCuBits = MinCbSizeY * MinCbSizeY *
				( BitDepthY + 2 * BitDepthC / ( SubWidthC * SubHeightC ) )	(7-39)

The value of BinCountsInNalUnits shall be less than or equal to ( 32 ÷ 3 ) * NumBytesInVclNalUnits + ( RawMinCuBits * PicSizeInMinCbsY )  32.

NOTE – The constraint on the maximum number of bins resulting from decoding the contents of the coded slice NAL units can be met by inserting a number of cabac_zero_word syntax elements to increase the value of NumBytesInVclNalUnits. Each cabac_zero_word is represented in a NAL unit by the three-byte sequence 0x000003 (as a result of the constraints on NAL unit contents that result in requiring inclusion of an emulation_prevention_three_byte for each cabac_zero_word).

From the specification it can be seen that the threshold consists of a static component computed from the picture size “( RawMinCuBits * PicSizeInMinCbsY )  32”, and a second dynamic component based on the number of bytes in the stream “( 32 ÷ 3 ) * NumBytesInVclNalUnits”.

For the purposes of this submission this is rewritten as:

n = β * b + c ÷ α

- n is the number of bins allowed

- b is the number of bits coded

- c is the number of bits in the raw image or image region

- α and β are two constants with default values of 32 and 4 ÷ 3 respectively

In section 7.3.2.13 of the specification it states that cabac_zero_word may be added at the end of each slice to maintain a ratio of bits to bins, such that a decoder has a limit with which to work.

During the last Geneva meeting JVET-P0395[3] was presented and contained statistics for the cabac_zero_word threshold on a per CTU basis.  This submission presents statistics on a per tile basis, using tiles consisting of 10, 60 and 120 complete (128x128) CTU.  The largest of these represents the number of complete CTU in a standard HD picture.

# Analysis

The pictures were split into tiles of a specific size, and for consistency, only image sequences from the CTC were selected that had at least 120 CTUs, i.e. HD. i.e. classes A, B and TGM.

The tile arrangements for the two formats tested are shown in the table below.

| Number of complete CTUs in tile | HD format | 4K format |
| --- | --- | --- |
| 10 | 3 tile columns, 4 tile rows; each tile is 5x2 CTUs | 6 tile columns, 8 tile rows; each tile is 5x2 CTUs |
| 60 | 1 tile column, 2 tile rows; each tile is 15x4 CTUs | 2 tile columns, 4 tile rows; each tile is 15x4 CTUs |
| 120 | Single tile of 15x8 CTUs; | 2 tile columns, 2 tile rows; each tile is 15x8 CTUs |

However, the first analysis using 10 CTU tiles showed that class TGM (figure 1) did not have any tiles over the cabac_zero_word threshold, and therefore class TGM was excluded from further analysis.

Figure 1. Bin to bit ratio vs bits in encoded stream for tiles of 10 CTU for the TGM class.  Note that the static component of the cabac_zero_word threshold (c ÷ α) has been removed to normalise the graph.  The colour of each point represents the slice QP.

Statistics were generated for 10 CTU tiles by running simulations at every QP within the range 12-37.  Each of these simulations was then categorized to either tier “Main” or “High” based on their luma sample rate and sequence bit rate using Table A.2 from the specification[1]. An extract is shown in Table 1.  Results with bit rates beyond “High” tier have been excluded.

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| .. | .. | .. | .. | .. | .. |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| .. | .. | .. | .. | .. | .. |

Table 1. Extract from Table A.2 – Tier and level limits for the video profiles

Figures 2 and 3 show results for these two tiers for 10 CTU tiles.  Figures 4 and 5 show results for 60 CTU tiles.  Although the number of results is much lower for 60 CTU tiles, the same pattern of bin to bit ratio vs bits in encoded stream exists.

Figure 2. Bin to bit ratio vs bits in encoded stream for tiles of 10 CTU for “Main” tier.

Figure 3. Bin to bit ratio vs bits in encoded stream for tiles of 10 CTU for “High” tier.

Figure 4. Bin to bit ratio vs bits in encoded stream for tiles of 60 CTU for “Main” tier.

Figure 5. Bin to bit ratio vs bits in encoded stream for tiles of 60 CTU for “High” tier.

Further analysis identified that the source of the threshold exceeding tiles was from the following sequences CatRobot and Campfire for “High” profile and DaylightRoad2 for both “High” and “Main” profile. More specifically the only tiles effected in “Main” profile were I frames and may thus be considered less significant for realistic use-cases. Table 2 shows the results for DaylightRoad2 in more depth for the high tier cases:

|  |  | I / B Slice QP |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 18 | 19 | 20 | 21 | 22 |
| Tile size in CTU | 10 | 22.9% | 50.0% | 62.5% | 75.0% | 43.75% |
|  | 60 | 25.0% | 25.0% | 50.0% | 75.0% | 43.75% |
|  | 120 | 0.0% | 50.0% | 50.0% | 62.5% | 50.0% |

Table 2. Percentage of tiles over cabac_zero_world threshold in “High” tier for DaylightRoad2 (Combined I and B frames).

It can be seen that percentage of tiles is not reduced as tile size increases, i.e. there is no evidence of any “averaging out” effect. Furthermore Table 3 shows the average percentage these tiles are over the cabac_zero_word threshold.  This represents, in effect, the cost to the coding efficiency imposed by adding cabac_zero_word.

|  |  | I / B Slice QP |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | 18 | 19 | 20 | 21 | 22 |
| Tile size in CTU | 10 | 0.93% | 1.70% | 3.14% | 3.81% | 1.85% |
|  | 60 | 1.74% | 3.61% | 3.52% | 3.73% | 2.23% |
|  | 120 | 0.0% | 1.38% | 3.26% | 4.30% | 1.53% |

Table 3. Average percentage tiles are over cabac_zero_word threshold in “High” tier for DaylightRoad2 (Combined I and B frames).

# Observation

As described above the addition of cabac_zero_word effectively introduces a loss to coding efficiency.  This must either be accepted, or an alternative threshold for cabac_zero_word must be defined.  Using the figures presented here, this could be done in one of three ways.

- Increase the static element of the cabac_zero_word threshold.

- Increase the dynamic element of the cabac_zero_word threshold

- Increase both.

To use the first technique, the maximum bit to bin ratio needs to be increased from 4 ÷ 3 (32 ÷ 3 on a byte to bin basis) to 3 ÷ 2 as in Figure 6.

Figure 6. Alternative bin to bit ratio for tiles of 10 CTU for “Main” tier.

For the second technique the divisor for the number of bits in the image can be changed to 24 from 32 as shown in Figure 7.

Figure 7. Required bin offset vs bits in encoded stream for tiles of 10 CTU for “Main” tier.  The lower line represents the current divisor in the cabac_zero_word threshold. The upper line represents a potential divisor which removes the need for cabac_zero_word padding.

To allow comparison of these two alternatives consider Figure 8 which shows a graph of bins vs bits for tiles of 10 CTU.  The red line indicates the existing cabac_zero_word formula and clearly a number of CTU exceed this threshold.  The green line indicates the first alternative (β = 3 ÷ 2), and the blue line the second alternative (α = 24).  Both these alternatives formulas prevents CTU exceeding the threshold, one with a higher threshold for CTU with many bits, the second with a higher threshold for CTU with few bits.

Figure 8. Bins vs Bits for tiles of 10 CTU for existing cabac_zero_word formula and two alternatives.

# Conclusion

It can be shown that the current CABAC zero word threshold is exceeded, but principally only for the high tier.

It is therefore proposed to change the CABAC zero word threshold constants for the high tier, to prevent the currently imposed bit rate losses, which can be as high as 4.30% for Daylight Road at slice QP 21.

# References

- B. Bross, J. Chen, S. Liu, and Y.-K. Wang, “Versatile Video Coding (Draft 7)”, JVET-P2001-vE the 16th JVET meeting, October 2019, Geneva, CH

[2] A. Browne, S.Keating, and K. Sharman, “CABAC zero word thresholds”, JVET-P0395, the 16th JVET meeting, October 2019, Geneva, CH

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).