| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0437 |
| --- | --- |

| Title: | Non-CE4: Disabling MTS for Geo |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Fabrice LE LEANNEC Tangi POIRIER Franck GALPIN | Tel: Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In this contribution, it is proposed disable MTS for Geometric Inter Prediction. It is reported that this achieves 0.00% loss in RA and 0.02% in gain in LDB if inter MTS is allowed, while the encoder time is decreased to 98% and 99% respectively.

# Introduction

Geometric Inter Prediction (Geo) is a new inter prediction tool under study in the core experiment of VVC (CE4). It can be considered as an extension of the existing triangular partitioning mode (TPM), where split line between the two prediction areas is extended from diagonal and anti-diagonal direction to more flexible angles and displacements. Due to this flexible split, the residual statistics associated with Geo predicted blocks are less homogeneous compared to regular one. Therefore, this contribution proposes disallowing MTS with Geo. That is, only DCT2 is allowed as a transform for Geo predicted residuals.

# Specification Changes

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| … |  |
| if( cu_cbf ) { |  |
| … |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && tu_cbf_luma[ x0 ][ y0 ] && MergeGeoFlag [ x0 ][ y0 ] = 0 ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |

# Experimental Results

The proposal is implemented on top of CE4 common base software, and simulation results were generated according to the commons testing conditions for RA and LDB configuration. Inter MTS for both anchor and test is activated (--MTS=3). The results are provided in the table below:

Table 1 Test results

|  |  |  | Random access Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over CE4 common base and --MTS=3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.01% | 0.03% | 98% | 100% |
| Class A2 | 0.00% | 0.03% | -0.08% | 98% | 100% |
| Class B | 0.01% | -0.04% | 0.04% | 98% | 100% |
| Class C | 0.00% | -0.17% | -0.03% | 99% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.05% | 0.00% | 98% | 100% |
| Class D | 0.04% | 0.10% | 0.03% | 100% | 99% |
| Class F | 0.02% | 0.00% | 0.01% | 101% | 100% |
|  |  |  | Low delay B Main10 |  |  |
|  |  |  | Over CE4 common base and --MTS=3 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | -0.15% | 0.06% | 100% | 100% |
| Class C | -0.04% | -0.09% | -0.23% | 97% | 99% |
| Class E | -0.12% | -0.33% | -0.18% | 100% | 101% |
| Overall | -0.02% | -0.18% | -0.10% | 99% | 100% |
| Class D | 0.07% | -0.23% | -0.34% | 98% | 98% |
| Class F | -0.02% | -0.19% | -0.48% | 100% | 100% |

As seen, the proposed modification provides no loss in RA and slight gain in in LDB. Further, the encoder time is reduced because of less RD checks.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).