| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0438 |
| --- | --- |

| Title: | Monochrome processing |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information / Proposal |  |  |
| Author(s) or Contact(s): | Adrian Browne Karl Sharman Steve Keating | Email: | adrian.browne@sony.com karl.sharman@sony.com steve.keating@sony.com |
| Source: | Sony Europe BV |  |  |

_____________________________

# Abstract

The current profiles for VTM permit the coding of monochrome (4:0:0) video formats. However, when an attempt was made to test monochrome processing on VTM 7.0, several fixes were required both to the encoder and the decoder software.

This contribution includes both the changes made to permit monochrome processing and results comparing VTM 7.0 monochrome performance with HM 16.21.

It is proposed that monochrome testing is added to the tests completed by AHG-13, to ensure that future changes to the VTM software model that conflict with monochrome processing are identified rapidly.

# Work Completed

Monochrome profiles were added to HEVC in version 2[1], with an example use cases being medical imagery.

The current profiles for VTM permit the coding of monochrome (4:0:0) video formats. Since monochrome was originally part of the VTM code base (as the original software was backwards compatible with the HEVC test model, HM), VTM had the controls present to configure this mode of operation.

However, when testing was attempted it was found that the executable failed due to segmentation failures.  Once these failures were resolved a number of other issues were identified in both inter and intra prediction which caused encoder / decoder mismatches.  Following a period of testing a patch was generated which allowed successful 4:0:0 processing, the results of which are given in the next section.

It is suggested that, in order to maintain 4:0:0 compatibility, a merge request is completed for the current patch.

# Results

The following results were achieved by comparing monochrome processing using the modified version of VTM 7.0 and monochrome processing using HM 16.21.  These results were produced using CTC[2], but with ChromaFormatIDC=400 set on the command line.

|  |  | All Intra |  |
| --- | --- | --- | --- |
|  |  | Over HM16.21_400 |  |
|  | Y | EncT | DecT |
| Class A1 | -20.87% | 2076% | 187% |
| Class A2 | -20.75% | 3053% | 204% |
| Class B | -18.35% | 3630% | 217% |
| Class C | -17.47% | 4602% | 233% |
| Class E | -23.05% | 2881% | 214% |
| Overall | -19.76% | 3259% | 212% |
| Class D | -13.67% | 4873% | 217% |
| Class F | -32.01% | 6049% | 213% |
| Class TGM | -56.72% | 4905% | 172% |

|  |  | Random Access |  |
| --- | --- | --- | --- |
|  |  | Over HM16.21_400 |  |
|  | Y | EncT | DecT |
| Class A1 | -32.70% | 581% | 135% |
| Class A2 | -38.00% | 673% | 146% |
| Class B | -33.19% | 665% | 144% |
| Class C | -26.96% | 883% | 166% |
| Class E |  |  |  |
| Overall | -32.39% | 700% | 148% |
| Class D | -25.06% | 1001% | 200% |
| Class F | -36.72% | 526% | 126% |
| Class TGM | -56.36% | 461% | 125% |

These contrast with the following results using CTC for 420 processing.

|  |  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over HM16.21_420 |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -28.20% | -33.34% | -33.64% | -27.99% | 1559% | 153% |
| Class A2 | -27.97% | -20.64% | -12.90% | -23.19% | 2456% | 162% |
| Class B | -21.25% | -20.05% | -27.33% | -20.82% | 2710% | 169% |
| Class C | -21.98% | -19.85% | -23.89% | -21.74% | 3777% | 184% |
| Class E | -25.35% | -21.78% | -26.48% | -25.19% | 2222% | 158% |
| Overall | -24.37% | -22.61% | -25.07% | -23.34% | 2532% | 166% |
| Class D | -17.79% | -14.00% | -15.60% | -17.26% | 4253% | 185% |
| Class F | -38.85% | -39.17% | -41.58% | -38.77% | 4468% | 162% |
| Class TGM | -62.28% | -60.46% | -60.51% | -61.61% | 3681% | 137% |

|  |  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Over HM16.21_420 |  |  |
|  | Y | U | V | YUV | EncT | DecT |
| Class A1 | -37.29% | -40.06% | -45.62% | -37.50% | 725% | 149% |
| Class A2 | -41.49% | -39.21% | -33.48% | -39.31% | 818% | 159% |
| Class B | -33.98% | -43.17% | -44.27% | -34.97% | 776% | 154% |
| Class C | -28.58% | -34.77% | -36.71% | -29.46% | 1005% | 180% |
| Class E |  |  |  |  |  |  |
| Overall | -34.70% | -39.52% | -40.37% | -34.87% | 829% | 160% |
| Class D | -26.33% | -30.81% | -31.42% | -26.87% | 1128% | 234% |
| Class F | -40.33% | -46.13% | -47.21% | -41.36% | 609% | 152% |
| Class TGM | -59.92% | -60.56% | -60.57% | -60.00% | 523% | 136% |

The results show that for AI, luma gains are overall 5% down with a significant increase in relative decoder times (212% vs 166%), but for RA the overall luma gain is 2% down whilst the relative decoder time is better for 400 processing (148% of HM) than 420 processing (160% of HM).

# Conclusion

It is suggested that, in order to maintain 4:0:0 compatibility, a merge request is completed for the current patch. In addition, it is proposed that AHG-13 add monochrome testing to its mandates, in order to quickly identify any tool changes that may inadvertently break this mode of operation, and to ascertain whether the 5% (AI) 2% (RA) BD-rate discrepancies observed between the 4:2:0 and 4:0:0 results are correct.

# References

[1] “High efficiency video coding”, ITU-T H.265 (V2), October 2014.

[2] J. Boyce, K. Suehring, X. Li and V. Seregin, “JVET common test conditions and software reference configurations for SDR video”, JVET-N1010, the 14th JVET meeting, March 2019, Geneva, CH

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).