| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0442 |
| --- | --- |

| Title: | Non-CE: Transform Selection for MIP in Implicit MTS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Fabrice LE LEANNEC Tangi POIRIER Franck GALPIN | Tel: Email: | karam.naser@InterDigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

Implicit MTS is a special configuration of transform selection where mts_idx is not signaled and the transform pairs are implicitly selected. Unlike regular intra prediction mode, the implicit transform selection for MIP is the default DCT2 for both horizontal and vertical direction.

This contribution proposes an alternative method of selecting the  transform pairs depending on block dimensions with the same condition as for regular intra prediction mode. This method achieves a luma coding gain of 0.03% and 0.02% in AI and RA configurations.

# Introduction

In vtm7.0, the transform selection can be configured with SPS level flags in three ways:

- All DCT2

- Implicit transform selection (Implicit MTS)

- Explicit transform selection (Explicit MTS)

All DCT2 is useful for encoders that do not support DST7 or DCT8 transforms, where the transform is set to DCT2 for all types of residual blocks. Explicit transform selection is the more powerful configuration where the residual blocks can be further transformed with DST7 and DCT8 transforms. Implicit transform selection is a good trade of between DCT2 and explicit transform selection as some blocks are allowed to use non-DCT2 transforms, while no RD checks are required to obtain the best transform pair. Implicit transform selection can achieve about 80% of the coding gain of the explicit transform selection, while having the same coding time as All DCT2.

In implicit MTS,  the transform selection for intra predicted residual is derived from the block dimensions as follows:

trTypeHor = ( nTbW  >=  4  &&  nTbW  <=  16 ) ? DST7 : DCT2

trTypeVer = ( nTbH  >=  4  &&  nTbH  <=  16 ) ? DST7 : DCT2

That is, if the considered dimension is between 4 and 16, DST7 is selected, otherwise DCT2 is selected. The same selection criterion is used for ISP. However, for MIP, only DCT2 is allowed since no current selection method is proposed.

# Proposed Method

Similar to regular intra prediction mode, implicit transform selection method is proposed for MIP. It is also based on the block dimensions. Specifically, the following is proposed.

If  ( nTbW  <=  16) && (nTbH  <=  16) ){

if (intra_mip_transposed[x0][y0] == 0){

trTypeHor = DCT2

trTypeVer = DST7

}

else{

trTypeHor = DST7

trTypeVer = DCT2

}

}

That is, if both width and height are between 4 and 16 and MIP transpose flag is false, the horizontal transform is DCT2 and the vertical is DST7, otherwise (MIP transpose flag is true), the horizontal transform is DST7 and the vertical is DCT2. It should be noted that there is no need to check if the block dimension is greater than or equal to 4, since MIP is not used for blocks less than 4x4.

The corresponding specification changes are as follows:

8.7.4.1	General

…

The variable implicitMtsEnabled is derived as follows:

- If sps_mts_enabled_flag is equal to 1 and one of the following conditions is true, implicitMtsEnabled is set equal to 1:

- IntraSubPartitionsSplitType is not equal to ISP_NO_SPLIT

- cu_sbt_flag is equal to 1 and Max( nTbW, nTbH ) is less than or equal to 32

- sps_explicit_mts_intra_enabled_flag is equal to 0 and CuPredMode[ 0 ][ xTbY ][ yTbY ] is equal to MODE_INTRA and lfnst_idx[ x0 ][ y0 ] is equal to 0 and ( ! intra_mip_flag[ x0 ][ y0 ] is equal to 0 or (nTbW  <=  16 and nTbH  <=  16) )

- Otherwise, implicitMtsEnabled is set equal to 0.

…

- Otherwise, if implicitMtsEnabled is equal to 1, the following applies:

- If cu_sbt_flag is equal to 1, trTypeHor and trTypeVer are specified in Table 38 depending on cu_sbt_horizontal_flag and cu_sbt_pos_flag.

- Otherwise, if intra_mip_flag[ x0 ][ y0 ] is equal to 0 and (cu_sbt_flag is equal to 0), trTypeHor and trTypeVer are derived as follows:

if ! intra_mip_flag[ x0 ][ y0 ]

trTypeHor = ( nTbW  >=  4  &&  nTbW  <=  16 ) ? 1 : 0	(1159)

trTypeVer = ( nTbH  >=  4  &&  nTbH  <=  16 ) ? 1 : 0	(1160)

Otherwise, if (intra_mip_flag[ x0 ][ y0 ] is equal to 1)

if (intra_mip_transposed[x0][y0] == 0 )

trTypeHor = 0

trTypeVer = 1

Otherwise (intra_mip_transposed[x0][y0] == 1)

trTypeHor = 1

trTypeVer = 0

…

# Experimental Results

The proposal is implemented on top of vtm7.0, and simulation results were generated according to the commons testing conditions for AI and RA configurations. Both anchor and test are configured to implicit MTS using the following parameters setting: --MTS=0 --MTSImplicit=1. The results are shown in the table below:

Table 1 Test results

|  |  |  | All Intra Main10 |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 with --MTS=0 --MTSImplicit=1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.06% | 0.01% | 0.05% | 100% | 101% |
| Class A2 | -0.05% | 0.03% | -0.01% | 100% | 102% |
| Class B | 0.00% | -0.06% | -0.05% | 100% | 104% |
| Class C | -0.01% | 0.08% | -0.15% | 100% | 100% |
| Class E | -0.06% | 0.03% | 0.03% | 102% | 100% |
| Overall | -0.03% | 0.01% | -0.04% | 100% | 101% |
| Class D | -0.01% | 0.00% | -0.03% | 101% | 100% |
| Class F | -0.04% | 0.11% | 0.03% | 102% | 101% |
|  |  |  | Random access Main10 |  |  |
|  |  |  | Over VTM-7.0 with --MTS=0 --MTSImplicit=1 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.06% | 0.09% | 98% | 99% |
| Class A2 | 0.01% | -0.03% | 0.00% | 102% | 102% |
| Class B | -0.04% | 0.04% | 0.06% | 103% | 103% |
| Class C | -0.03% | 0.01% | -0.18% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.02% | 0.00% | -0.01% | 101% | 101% |
| Class D | -0.05% | -0.07% | -0.19% | 100% | 101% |
| Class F | -0.01% | -0.18% | -0.06% | 100% | 101% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).