| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0466-v1 |
| --- | --- |

| Title: | MIP with constant shifts and offsets |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Björn Stallenberger, Michael Schäfer, Philipp Merkle, Tobias Hinz, Philipp Helle, Heiko Schwarz, Detlev Marpe, Thomas Wiegand (Fraunhofer HHI) Kenji Kondo, Masaru Ikeda (Sony) Junyan Huo, Haixin Wang, Yanzhuo Ma, Fuzheng Yang (Xidian University) Shuai Wan (NPU) Yuanfang Yu (OPPO) | Tel: Email: | +49 30 31002-642 jonathan.pfaff@hhi.fraunhofer.de, kenji.kondo@sony.com, Masaru.Ikeda@sony.com, jyhuo@mail.xidian.edu.cn, swan@nwpu.edu.cn, yuyuanfang@oppo.com |
| Source: | Fraunhofer HHI, Sony Corporation, Xidian University, NPU, OPPO |  |  |

_____________________________

# Abstract

In this document, it is proposed to unify the shift and the offset used in the matrix-vector multiplication of matrix based intra prediction (MIP). It is proposed to always set the right shift sW to 6 and to always set the offset fO to 32. Thus, it is proposed to remove the dependency of sW and fO on the MIP mode and the MIP-sizeId. Experimental results of 0.00% BD rate change for the AI configuration and of -0.01% BD rate change for the RA configuration are reported.  The measured encoding- and decoding-times are  99% and 99% for the AI configuration and 99% and 101% for the RA configuration, respectively.

# Description of the proposed technology

In the MIP prediction process, a matrix-vector multiplication is performed in order to compute a prediction signal which is to be clipped and upsampled afterwards in order to obtain the final MIP prediction signal. This matrix-vector multiplication is specified in equations (257) and (258) of 8.4.5.2.1 in [1]. For these two equations, the variable sW representing the right shift and the variable fO representing the offset of the MIP matrix are needed. The variables sW and fO depend on mipSizeId and modeId and are given in Table 8-5 and Table 8-6 of [1].

In this document, it is proposed to remove Table 8-5 and Table 8-6 of [1] and to always set:

- sW = 6

- fO  = 32 .

It is asserted that the proposed change, in particular the fixed shift value 6, is beneficial for an efficient and compact SIMD-implementation of MIP in software. Next, it is asserted that setting fO=32 enables to implement the multiplication by fO in (257) of [1] by shifts. Finally, it is asserted that the proposed change can be regarded as a cleanup since it removes the dependency of sW and fO on mipSizeId and modeId.

Moreover, it is proposed to change the sign of the first entry of the input vector for the matrix-vector multiplication of MIP in the case that mipSizeI=0 or mipSizeId=1. Thus, in the first line of equation (256) of [1],  it is proposed to replace the definition

p[ 0 ] = pTemp[ 0 ] − ( 1  <<  ( BitDepth − 1 ) )

by

p[ 0 ] =  ( 1  <<  ( BitDepth − 1 ) ) − pTemp[ 0 ].

It is asserted that such a change leads to a smaller dynamic range of the matrix coefficients of the matrices needed for the MIP-modes of mipSizeId=0 and mipSizeId=1 and is thus beneficial in order to achieve the proposed change sW=6 and fO=32. This observation respectively the proposed change were already suggested in [2].

In order to achieve sW=6 and fO=32, the MIP-matrices of mipSizId=0 and mipSizeId=2 were retrained. In this retraining, the constraint on the shift and the offset were incorporated. Otherwise, the training was unchanged in comparison to the training run that led to the MIP-matrices of VTM-7.0. In the appendix of this document, a description of the aforementioned modification of the training algorithm is given.

# Experimental results

In this section, experimental results are reported according to the common test conditions [3] and the test conditions of the CE for intra, [4]. In Table 1 and Table 2, results are reported for the proposed change compared to the VTM-7.0 anchor for the AI and the RA configuration, respectively. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler.

Table 1. Result of the incorporated MIP changes: Reference is VTM-7.0 anchor, test is VTM-7.0 with the proposed changes, AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,02% | -0,19% | 0,15% | 100% | 103% |
| Class A2 | 0,00% | 0,01% | -0,03% | 99% | 99% |
| Class B | 0,01% | 0,00% | -0,05% | 99% | 97% |
| Class C | 0,00% | 0,09% | 0,02% | 97% | 99% |
| Class E | 0,00% | -0,03% | 0,05% | 100% | 100% |
| Overall | 0,00% | -0,02% | 0,02% | 99% | 99% |
| Class D | -0,04% | 0,16% | 0,14% | 101% | 99% |
| Class F | -0,02% | -0,04% | 0,05% | 99% | 95% |

Table 2. Result of the incorporated MIP changes: Reference is VTM-7.0 anchor, test is VTM-7.0 with the proposed changes, RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,01% | -0,19% | 0,00% | 99% | 101% |
| Class A2 | -0,02% | 0,05% | 0,07% | 100% | 102% |
| Class B | -0,01% | 0,09% | 0,08% | 100% | 105% |
| Class C | 0,01% | 0,03% | -0,04% | 99% | 97% |
| Class E |  |  |  |  |  |
| Overall | -0,01% | 0,01% | 0,03% | 99% | 101% |
| Class D | -0,02% | -0,25% | -0,23% | 99% | 99% |
| Class F | 0,00% | 0,11% | 0,01% | 96% | 99% |

# Conclusion

In this contribution, MIP with constant shifts and offsets for the involved matrix-vector multiplications is proposed with no impact on the coding performance. If such constant shifts and offsets for MIP are regarded as desirable, it is proposed to adopt the technology of the present document into the next working draft of the VVC.

# References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 7), Document JVET-P2001, Version 14 , Geneva, Switzerland, October 2019

[2] J. Huo, H. Wang, Y. Sun, Y. Ma, S. Wan, F. Yang, Y. Yu, Y. Liu,  Non-CE 3: MIP simplification, Document JVET-P0136, Geneva, Switzerland, October 2019

[3] F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, Document  JVET-N1010, Geneva, Switzerland, March 2019

[4] G. Van der Auwera et al.,“Description of Core Experiment 3: Intra Prediction and Mode Coding”, Document JVET –O2023, Gothenbrug, Sweden, July 2019

# Appendix: Description of the training modifications

In order to achieve a fixed shift 6 and a fixed offset 32, an interval [a,b] was fixed such that every xϵ[a,b] satisifies

26*x+3226ϵ-0.5,27-0.5,

i.e. one puts a=-32.564 ~-0.5 and  b=95.564~1.5. Then, a function f was fixed which is smooth, whose image is (a,b), which approximates the identiy around a+b2~0.5 and whose derivative vanishes nowhere. In the training, the function 
fx=x-63128101+x-6312810+63128
was used, see Figure 1.

Figure 1: Smooth clipping function used in the training

The function f was applied to each entry of each MIP matrix during training which means that it was part of the optimization function that the training algorithm attempts to minimize. Otherwise, the training was unchanged. The function f instead of the true clipping to [a,b] was used since the training algorithm is based on a stochastic gradient descent method and since the true clipping has derivative 0 outside [a,b]. Thus, if one used the true clipping, matrix entries that fall outside [a,b] would never receive a gradient update during training.

# Patent rights declaration

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Sony Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

NPU may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

OPPO may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).