| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0448-v1 |
| --- | --- |

| Title: | MTS dependent coefficient subblock scanning for zero-out |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago de Luxán Hernández Tung Nguyen Benjamin Bross Heiko Schwarz Detlev Marpe Thomas Wiegand | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution proposes two fixes to replace the bitstream restriction that requires the MTS index to be equal to 0 (DCT-II in both directions) in case there are non-zero coefficients outside the 16x16 zero-out area, by a syntax change. The first fix A checks for every coefficient subblock whether there are non-zero coefficients outside the 16x16 area and does not signal the MTS index if there are any and infers it to zero instead. The second fix B avoids checking each subblock by moving the MTS index signaling before the coefficient parsing in subblocks and adapting the subblock scan to cover only the 16x16 area in case the MTS index is greater than 0. Fix A results in 0.0% BD-rate Y for AI and RA, fix B results in -0.01% BD-rate Y for AI and 0.02% for RA with no measurable change in runtimes.

# Introduction

In the current VVC draft specification [1], the MTS index, is signaled after the transform coefficients. Consequently, the position of the last significant coefficient is known. When this coefficient is outside the 16x16 non-zero area, there is no need to signal the transform selection because the only allowed transform in this case is the DCT-II, which corresponds to an MTS index equal to 0. However, in this current design, it cannot be guaranteed that all coefficients outside the 16x16 area are equal to zero. This can be seen in the example in Figure 1, where the last position is located inside the 16x16 non-zero area but subsequent subblocks in scan order are outside this area. In case that these non-zero coefficients exist, an encoder constraint has to ensure that the signaled transform is the DCT-II, e.g. by always signaling an MTS index equal to 0. This produces a signaling overhead because the DCT-II is explicitly signaled although always required to be equal to 0. In the following two fixes are described to tackle this aspect.

Figure 1. Diagonal scanning of coefficient subblocks in VVC Draft 7 for MTS index 0 (left) and MTS index > 0 (right)

# Proposed Fixes

Two fixes are proposed to replace the encoder constraint for the MTS index by a syntax change.

## Fix A

For every coefficient subblock, check whether there are non-zero coefficients outside the 16x16 area, do not signal the MTS index if there are any and infer it to zero instead.

## Fix B

Instead of checking each subblock, move the MTS index signaling before the coefficient parsing in subblocks and adapt the subblock scan to cover only the 16x16 area in case the MTS index is greater than 0. This modified coefficient scan is illustrated in Figure 2. Moving the MTS index before the LFNST index removes the dependency on the LFNST index from MTS index parsing. Instead, the LFNST index parsing now depends on the MTS index. However, the dependency on the LFNST index can be partially kept by making the MTS index context dependent on two conditions that also steer the LFNST index parsing:

- If all conditions that allow the LFNST index to be signaled (and potentially be greater than 0) are true including if the current transform block has more non-zero coefficients than the DC and there are no more than 8 coefficients in scan order from the last significant position to the DC, select context 1.

- Otherwise, select context 0.

Figure 2 Modified scanning of coefficient subblocks for MTS index > 0 (Fix B)

# Draft Text

Both fixes remove the following bitstream restriction in the mts_idx semantics:

mts_idx specifies which transform kernels are applied along the horizontal and vertical direction of the associated luma transform blocks in the current coding unit.

When mts_idx is not present, it is inferred to be equal to 0.

It is a requirement of bitstream conformance that mts_idx shall be equal to 0 if in the current coding unit at least one coded_sub_block_flag[ xS ][ yS ] in the residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) syntax structure is not equal to 0 for cIdx equal to 0 and xS or yS greater than 3.

## Fix A

Add the text highlighted in yellow and remove the text highlighted in red:

7.3.9.11	Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| … |  |  |
| if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 ) |  |  |
| MtsZeroOutSigCoeffFlag = 0 |  |  |
| QState = 0 |  |  |
| for( i = lastSubBlock; i >= 0; i− − ) { |  |  |
| startQStateSb = QState |  |  |
| xS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 0 ] |  |  |
| yS = DiagScanOrder[ log2TbWidth − log2SbW ][ log2TbHeight − log2SbH ] [ i ][ 1 ] |  |  |
| inferSbDcSigCoeffFlag = 0 |  |  |
| if( i < lastSubBlock && i > 0 ) { |  |
| coded_sub_block_flag[ xS ][ yS ] |  |
| inferSbDcSigCoeffFlag = 1 |  |
| } |  |
| if( ( coded_sub_block_flag[ xS ][ yS ] ¦ ¦ i = = lastSubBlock ) && ( xS > 3 ¦ ¦ yS > 3 ) && cIdx = = 0 ) { |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| } |  |
| … |  |

## Fix B

Add the text highlighted in yellow and remove the text highlighted in red:

7.3.9.5	Coding unit syntax

| coding_unit( x0, y0, cbWidth, cbHeight, cqtDepth, treeType, modeType ) { | Descriptor |
| --- | --- |
| ... |  |
| LfnstDcOnly = 1 |  |
| LfnstZeroOutSigCoeffFlag = 1 |  |
| MtsZeroOutSigCoeffFlag = 1 |  |
| lfnstWidth = ( treeType = = DUAL_TREE_CHROMA ) ? cbWidth / SubWidthC : ( ( IntraSubPartitionsSplitType = = ISP_VER_SPLIT ) ? cbWidth / NumIntraSubPartitions : cbWidth ) |  |
| lfnstHeight = ( treeType = = DUAL_TREE_CHROMA ) ? cbHeight / SubHeightC : ( ( IntraSubPartitionsSplitType = = ISP_HOR_SPLIT) ? cbHeight / NumIntraSubPartitions : cbHeight ) |  |
| CanUseLfnst = Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY |  |
| transform_tree( x0, y0, cbWidth, cbHeight, treeType, chType ) |  |
| if( Min( lfnstWidth, lfnstHeight ) >= 4 && sps_lfnst_enabled_flag = = 1 && CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && ( treeType = = DUAL_TREE_CHROMA ¦ ¦ !intra_mip_flag[ x0 ][ y0 ] ¦ ¦ Min( lfnstWidth, lfnstHeight ) >= 16 ) && Max( cbWidth, cbHeight ) <= MaxTbSizeY ) { |  |
| if( ( IntraSubPartitionsSplitType != ISP_NO_SPLIT ¦ ¦ LfnstDcOnly = = 0 ) && LfnstZeroOutSigCoeffFlag = = 1 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && && mts_idx = = 0 && CanUseLfnst) |  |
| lfnst_idx | ae(v) |
| } |  |
| if( treeType != DUAL_TREE_CHROMA && lfnst_idx = = 0 && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( cbWidth, cbHeight ) <= 32 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && MtsZeroOutSigCoeffFlag = = 1 && tu_cbf_luma[ x0 ][ y0 ] ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| } |  |
| } |  |

7.3.9.11	Residual coding syntax

| residual_coding( x0, y0, log2TbWidth, log2TbHeight, cIdx ) { | Descriptor |  |
| --- | --- | --- |
| if( sps_mts_enabled_flag && cu_sbt_flag && cIdx = = 0 && log2TbWidth = = 5 && log2TbHeight < 6 ) |  |  |
| log2ZoTbWidth = 4 |  |  |
| else |  |  |
| log2ZoTbWidth = Min( log2TbWidth, 5 ) |  |  |
| if( sps_mts_enabled_flag && cu_sbt_flag && cIdx = = 0 && log2TbWidth < 6 && log2TbHeight = = 5 ) |  |  |
| log2ZoTbHeight = 4 |  |  |
| else |  |  |
| log2ZoTbHeight = Min( log2TbHeight, 5 ) |  |  |
| if( log2TbWidth > 0 ) |  |  |
| last_sig_coeff_x_prefix | ae(v) |
| if( log2TbHeight > 0 ) |  |
| last_sig_coeff_y_prefix | ae(v) |
| if( last_sig_coeff_x_prefix > 3 ) |  |
| last_sig_coeff_x_suffix | ae(v) |
| if( last_sig_coeff_y_prefix > 3 ) |  |
| last_sig_coeff_y_suffix | ae(v) |
| if( cIdx = = 0 && treeType != DUAL_TREE_CHROMA && transform_skip_flag[ x0 ][ y0 ][ 0 ] = = 0 && Max( log2TbWidth, log2TbHeight ) <= 5 && IntraSubPartitionsSplit[ x0 ][ y0 ] = = ISP_NO_SPLIT && cu_sbt_flag = = 0 && ( LastSignificantCoeffX < = 15 && LastSignificantCoeffY < = 15 ) ) { |  |
| if( ( ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTER && sps_explicit_mts_inter_enabled_flag ) ¦ ¦ ( CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_INTRA && sps_explicit_mts_intra_enabled_flag ) ) ) |  |
| mts_idx | ae(v) |
| if( mts_idx > 0 && log2TbWidth = = 5 ) |  |
| log2ZoTbWidth = 4 |  |
| if( mts_idx > 0 && log2TbHeight = = 5 ) |  |
| log2ZoTbHeight = 4 |  |
| ... |  |
| if( ( lastSubBlock > 0 && log2TbWidth >= 2 && log2TbHeight >= 2 ) ¦ ¦ ( lastScanPos > 7 && ( log2TbWidth = = 2 ¦ ¦ log2TbWidth = = 3 ) && log2TbWidth = = log2TbHeight ) ) |  |
| LfnstZeroOutSigCoeffFlagCanUseLfnst = 0 |  |
| if( ( LastSignificantCoeffX > 15 ¦ ¦ LastSignificantCoeffY > 15 ) && cIdx = = 0 ) |  |
| MtsZeroOutSigCoeffFlag = 0 |  |
| ... |  |
| } |  |  |

9.3.4.2	Derivation process for ctxTable, ctxIdx and bypassFlag

...

Table 128 – Assignment of ctxInc to syntax elements with context coded bins

| Syntax element | binIdx |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | >= 5 |
| ... |  |  |  |  |  |  |
| mts_idx | ( LfnstDcOnly = = 0 && CanUseLfnst ) ? 1 : 0 | 2 | 3 | 4 | na | na |
| ... |  |  |  |  |  |  |

# Results

The two proposed fixes A and B have been tested using common test conditions.

## Fix A

Table 1 - AI results for Fix A

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.03% | 0.01% | 100% | 100% |
| Class A2 | -0.01% | -0.01% | 0.00% | 99% | 101% |
| Class B | 0.00% | 0.01% | -0.03% | 102% | 97% |
| Class C | 0.00% | 0.00% | 0.00% | 101% | 98% |
| Class E | 0.00% | 0.00% | -0.03% | 100% | 102% |
| Overall | 0.00% | 0.00% | -0.01% | 100% | 99% |
| Class D | 0.00% | 0.03% | 0.00% | 100% | 98% |
| Class F | 0.00% | 0.01% | 0.00% | 99% | 101% |
| Class TGM | 0.00% | 0.00% | 0.00% | 100% | 100% |

Table 2 - RA results for Fix A

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | -0.07% | 0.00% | 100% | 100% |
| Class A2 | -0.05% | 0.02% | -0.02% | 99% | 100% |
| Class B | 0.02% | 0.02% | 0.06% | 97% | 100% |
| Class C | 0.00% | -0.04% | 0.03% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.02% | 99% | 100% |
| Class D | 0.00% | 0.01% | -0.03% | 99% | 100% |
| Class F | 0.00% | -0.02% | -0.02% | 99% | 97% |
| Class TGM | 0.00% | 0.00% | 0.00% | 99% | 101% |

## Fix B

Two results are reported. The following tables report results when only the syntax is changed, i.e. parsing mts_idx before lfnst_idx and making lfnst_idx conditioned on mts_idx.

Table 3 - AI results for Fix B without context change

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.13% | 0.38% | 0.64% | 102% | 101% |
| Class A2 | 0.06% | 0.34% | 0.29% | 101% | 102% |
| Class B | 0.09% | 0.39% | 0.35% | 101% | 101% |
| Class C | 0.06% | 0.33% | 0.34% | 101% | 100% |
| Class E | 0.13% | 0.42% | 0.56% | 101% | 103% |
| Overall | 0.09% | 0.37% | 0.42% | 101% | 101% |
| Class D | 0.08% | 0.34% | 0.17% | 101% | 101% |
| Class F | 0.04% | 0.21% | 0.39% | 101% | 101% |
| Class TGM | 0.00% | 0.04% | -0.02% | 100% | 100% |

Table 4 - RA results for Fix B without context change

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.12% | 0.12% | 0.19% | 100% | 99% |
| Class A2 | 0.05% | 0.22% | 0.23% | 99% | 100% |
| Class B | 0.10% | 0.22% | 0.35% | 100% | 97% |
| Class C | 0.07% | 0.22% | 0.15% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | 0.09% | 0.20% | 0.24% | 100% | 99% |
| Class D | 0.09% | -0.01% | 0.36% | 100% | 103% |
| Class F | 0.01% | 0.17% | 0.09% | 100% | 99% |
| Class TGM | 0.02% | -0.03% | -0.02% | 101% | 102% |

It can be seen that changing the syntax order introduces loss which can be reduced by adding the additional context for the first bin of mts_idx as reported in the following tables.

Table 5 - AI results for Fix B with context change

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.19% | 0.40% | 101% | 104% |
| Class A2 | -0.01% | 0.13% | 0.12% | 100% | 100% |
| Class B | 0.00% | 0.16% | 0.16% | 100% | 100% |
| Class C | -0.02% | 0.17% | 0.14% | 100% | 101% |
| Class E | 0.00% | 0.06% | 0.32% | 100% | 103% |
| Overall | -0.01% | 0.15% | 0.22% | 100% | 101% |
| Class D | 0.00% | 0.18% | 0.16% | 101% | 101% |
| Class F | -0.01% | 0.16% | 0.18% | 99% | 103% |
| Class TGM | 0.00% | 0.03% | -0.01% | 99% | 101% |

Table 6 - RA results for Fix B with context change

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-7.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.04% | 0.00% | 0.11% | 98% | 102% |
| Class A2 | -0.01% | 0.10% | 0.14% | 99% | 99% |
| Class B | 0.01% | 0.07% | 0.24% | 99% | 101% |
| Class C | 0.05% | 0.15% | 0.11% | 99% | 103% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.08% | 0.16% | 99% | 101% |
| Class D | 0.05% | -0.13% | 0.20% | 100% | 103% |
| Class F | 0.02% | 0.19% | 0.14% | 98% | 101% |
| Class TGM | -0.05% | -0.06% | -0.03% | 99% | 104% |

# References

- B. Bross et al., “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, Geneva, Switzerland, October 2019

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)