| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0449-v1 |
| --- | --- |

| Title: | On smoothing filter with RPR in VVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Benjamin Bross Martin Winken Yago Sanchez Robert Skupin Heiko Schwarz Detlev Marpe Thomas Wiegand | Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution asserts that application of the luma half-sample smoothing interpolation filter in case of RPR is undesirable, as samples of a reference block will be derived using different filter kernels thereby mixing interpolation filter kernels with smoothing and DCTIF (i.e., almost all-pass) characteristics in a reference block. This design aspect is asserted to be an oversight. The contribution proposes to change the interpolation filter selection by disabling the smoothing filter kernel for reference pictures either independently in horizontal and vertical if the scaling ratio in the respective direction is not equal to 1 (Fix A) or jointly in horizontal and vertical direction of the scaling ratio in at least one direction is not equal to 1 (Fix B). The coding efficiency results in BD-rate (Y, U, V) using the reference picture resampling CE test conditions with a scale factor of 2.0x are identical for A and B and different for different scale factors such as 2.0x in horizontal and 1.0x in vertical direction:

Hor./Ver.: 2.0x A, B: 	LB   0.05%, −0.05%, −0.05% (PSNR1)	  0.03%, −0.04%,−0.02% (PSNR2)

Hor: 2.0x / Ver: 1.0x A	LB −0.04%, −0.09%,   0.10% (PSNR1)	−0.03%, −0.11%,  0.09% (PSNR2)

Hor: 2.0x / Ver: 1.0x B	LB   0.00%,    0.04%,   0.23% (PSNR1)	   0.01%,   0.06%, 0.24% (PNSR2)

# Introduction

The current VVC draft specification [1] contains interpolation filter kernels that depend on the fractional sample position of the reference sample block. Furthermore, the filter kernels depend on the signalled motion vector (MV) resolution. For a particular case (half-sample MV resolution, no IBC, no affine), the interpolation filter kernel used for luma half-sample fractional sample positions is a 6-tap filter with smoothing characteristics. Otherwise, (other fractional sample positions and/or MV resolution) all filter kernels have close to all-pass characteristics. The following table from the current VVC draft specification contains the interpolation filter coefficients, wherein yellow highlighting indicates the smoothing filter.

| Fractional sample position p | interpolation filter coefficients |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | fL[ p ][ 0 ] | fL[ p ][ 1 ] | fL[ p ][ 2 ] | fL[ p ][ 3 ] | fL[ p ][ 4 ] | fL[ p ][ 5 ] | fL[ p ][ 6 ] | fL[ p ][ 7 ] |
| 1 | 0 | 1 | −3 | 63 | 4 | −2 | 1 | 0 |
| 2 | −1 | 2 | −5 | 62 | 8 | −3 | 1 | 0 |
| 3 | −1 | 3 | −8 | 60 | 13 | −4 | 1 | 0 |
| 4 | −1 | 4 | −10 | 58 | 17 | −5 | 1 | 0 |
| 5 | −1 | 4 | −11 | 52 | 26 | −8 | 3 | −1 |
| 6 | −1 | 3 | −9 | 47 | 31 | −10 | 4 | −1 |
| 7 | −1 | 4 | −11 | 45 | 34 | −10 | 4 | −1 |
| 8 (hpelIfIdx = = 0) | −1 | 4 | −11 | 40 | 40 | −11 | 4 | −1 |
| 8 (hpelIfIdx = = 1) | 0 | 3 | 9 | 20 | 20 | 9 | 3 | 0 |
| 9 | −1 | 4 | −10 | 34 | 45 | −11 | 4 | −1 |
| 10 | −1 | 4 | −10 | 31 | 47 | −9 | 3 | −1 |
| 11 | −1 | 3 | −8 | 26 | 52 | −11 | 4 | −1 |
| 12 | 0 | 1 | −5 | 17 | 58 | −10 | 4 | −1 |
| 13 | 0 | 1 | −4 | 13 | 60 | −8 | 3 | −1 |
| 14 | 0 | 1 | −3 | 8 | 62 | −5 | 2 | −1 |
| 15 | 0 | 1 | −2 | 4 | 63 | −3 | 1 | 0 |

The variable hpelIfIdx equal to 1 indicates whether the highlighted 6-tap smoothing filter is used for the luma half-sample fractional sample positions. hpelIfIdx is set to 1 when AmvrShift is equal to 3, which indicates half-sample MVD resolution is used without IBC mode. This is signalled in the bitstream using the syntax amvr_flag equal to 1 and amvr_precision_idx equal to 0. See also the following table from the current VVC draft specification wherein yellow highlighting indicates the respective value of AmvrShift.

| amvr_flag | amvr_precision_idx | AmvrShift |  |  |
| --- | --- | --- | --- | --- |
|  |  | inter_affine_flag = =1 | CuPredMode[ chType ][ x0 ][ y0 ] = = MODE_IBC ) | inter_affine_flag = =0 && CuPredMode[ chType ][ x0 ][ y0 ] ! = MODE_IBC |
| 0 | - | 2 (1/4 luma sample) | - | 2 (1/4 luma sample) |
| 1 | 0 | 0 (1/16 luma sample) | 4 (1 luma sample) | 3 (1/2 luma sample) |
| 1 | 1 | 4 (1 luma sample) | 6 (4 luma samples) | 4 (1 luma sample) |
| 1 | 2 | - | - | 6 (4 luma samples) |

Another feature in the VVC draft specification that was introduced later-on is reference picture resampling (RPR), which allows changing the coded picture size within a CVS by enabling inter-prediction references to previously coded pictures that are of a different size than the current picture. For this purpose, the referenced picture area is resampled to having the same sample array size as the current block. Thereby, a reference block usually contains multiple fractional sample positions, each obtained through different filter coefficients corresponding to the appropriate phase.

# Problem

In RPR, depending on the applied sampling ratio, the regular interpolation filter coefficients are re-used (<=1.25x) or additional filter kernels are applied (>1.25x). In case of the former, it may happen that part of the (resampled) reference block samples are obtained through a smoothing filter while neighbouring samples of the reference block are obtained through the DCTIF filters.

This contribution asserts that application of the smoothing interpolation filter in case of RPR with scaling ration equal to 1 is undesirable, as samples of a reference block will be derived using different filter kernels thereby mixing interpolation filter kernels with smoothing and close to all-pass characteristics in a single reference block. This design aspect is asserted to be an oversight during the RPR integration.

# Proposed Fix

In order to prevent the aforementioned mixture of smoothing and DCTIF filter kernels in case of reference picture resampling, two alternative fixes are proposed that change the interpolation filter selection by disabling the smoothing filter kernel for reference pictures:

- Fix A: independently in horizontal and vertical if the scaling ratio in the respective direction is not equal to 1

- Fix B: jointly in horizontal and vertical direction of the scaling ratio in at least one direction is not equal to 1

# Draft Text

## Fix A

Change the following highlighted in yellow:

8.5.6.3.2	Luma sample interpolation filtering process...

...

The horizontal and vertical half sample interpolation filter indices hpelHorIfIdx and hpelVerIfIdx are derived as follows:

hpelHorIfIdx = ( scalingRatio[ 0 ] = = 16384 )  ?  hpelIfIdx  :  0	(955)

hpelVerIfIdx = ( scalingRatio[ 1 ] = = 16384 )  ?  hpelIfIdx  :  0	(956)

The horizontal luma interpolation filter coefficients fLH[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLH[ p ] are specified in Table 32.

- Otherwise, if scalingRatio[ 0 ] is greater than 28672, luma interpolation filter coefficients fLH[ p ] are specified in Table 30.

- Otherwise, if scalingRatio[ 0 ] is greater than 20480, luma interpolation filter coefficients fLH[ p ] are specified in Table 31.

- Otherwise, the luma interpolation filter coefficients fLH[ p ] are specified in Table 29 depending on hpelIfIdx set equal to hpelHorIfIdx.

The vertical luma interpolation filter coefficients fLV[ p ] for each 1/16 fractional sample position p equal to yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLV[ p ] are specified in Table 32.

- Otherwise, if scalingRatio[ 1 ] is greater than 28672, luma interpolation filter coefficients fLV[ p ] are specified in Table 30.

- Otherwise, if scalingRatio[ 1 ] is greater than 20480, luma interpolation filter coefficients fLV[ p ] are specified in Table 31.

- Otherwise, the luma interpolation filter coefficients fLV[ p ] are specified in Table 29 depending on hpelIfIdx set equal to hpelVerIfIdx.

## Fix B

Change the following highlighted in yellow:

8.5.6.3.2	Luma sample interpolation filtering process...

...

The half sample interpolation filter indix hpelIfIdx is modified as follows:

hpelIfIdx = ( scalingRatio[ 0 ] = = 16384  &&  scalingRatio[ 1 ] = = 16384  )  ?  hpelIfIdx  :  0	(955)

The horizontal luma interpolation filter coefficients fLH[ p ] for each 1/16 fractional sample position p equal to xFracL or  yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLH[ p ] are specified in Table 32.

- Otherwise, if scalingRatio[ 0 ] is greater than 28672, luma interpolation filter coefficients fLH[ p ] are specified in Table 30.

- Otherwise, if scalingRatio[ 0 ] is greater than 20480, luma interpolation filter coefficients fLH[ p ] are specified in Table 31.

- Otherwise, the luma interpolation filter coefficients fLH[ p ] are specified in Table 29 depending on hpelIfIdx.

The vertical luma interpolation filter coefficients fLV[ p ] for each 1/16 fractional sample position p equal to yFracL are derived as follows:

- If MotionModelIdc[ xSb ][ ySb ] is greater than 0, and sbWidth and sbHeight are both equal to 4, the luma interpolation filter coefficients fLV[ p ] are specified in Table 32.

- Otherwise, if scalingRatio[ 1 ] is greater than 28672, luma interpolation filter coefficients fLV[ p ] are specified in Table 30.

- Otherwise, if scalingRatio[ 1 ] is greater than 20480, luma interpolation filter coefficients fLV[ p ] are specified in Table 31.

- Otherwise, the luma interpolation filter coefficients fLV[ p ] are specified in Table 29 depending on hpelIfIdx.

# Results

The two proposed fixes A and B have been tested using the test conditions from CE1 on reference picture resampling filters [2]. This includes the low delay B (LB) configuration, a resolution change by factor 2.0x and 1.5x in horizontal and vertical direction every 0.5s as well as encoding only the first half number of frames. However, for a scaling factor of 1.5x, the VTM-7.1 anchor produces an encoder / decoder mismatch for the ReaceHorseC sequences. This has been reported in ticket #787 and thus no results have been produced for this scaling factor.

| Fix A 2.0x |  | PSNR1 |  |  | PSNR2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LB | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0.05% | 0.04% | 0.02% | 0.04% | 0.03% | 0.03% | 99% | 98% |
| Class C | 0.04% | -0.02% | -0.14% | 0.03% | 0.12% | -0.05% | 99% | 95% |
| Class E | 0.06% | -0.26% | -0.02% | 0.01% | -0.36% | -0.05% | 100% | 100% |
| Overall | 0.05% | -0.05% | -0.05% | 0.03% | -0.04% | -0.02% | 99% | 98% |
| Class D | -0.08% | 0.35% | 0.41% | -0.10% | 0.26% | 0.39% | 100% | 99% |

| Fix B 2.0x |  | PSNR1 |  |  | PSNR2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LB | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0.05% | 0.04% | 0.02% | 0.04% | 0.03% | 0.03% | 100% | 99% |
| Class C | 0.04% | -0.02% | -0.14% | 0.03% | 0.12% | -0.05% | 99% | 97% |
| Class E | 0.06% | -0.26% | -0.02% | 0.01% | -0.36% | -0.05% | 101% | 102% |
| Overall | 0.05% | -0.05% | -0.05% | 0.03% | -0.04% | -0.02% | 100% | 99% |
| Class D | -0.08% | 0.35% | 0.41% | -0.10% | 0.26% | 0.39% | 101% | 101% |

As can be seen from the results above, fix A and B are identical in case the scaling factor is the same in horizontal and vertical direction. Hence, additional results have been produced with a scaling factor of 2.0x in horizontal direction and 1.0x (no scaling) in vertical direction.

| Fix A hor 2.0x / ver 1.0x |  | PSNR1 |  |  | PSNR2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LB | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | -0.05% | -0.15% | 0.04% | -0.05% | -0.16% | 0.05% | 99% | 99% |
| Class C | -0.03% | -0.03% | 0.25% | -0.01% | -0.06% | 0.21% | 99% | 99% |
| Class E | -0.05% | -0.10% | 0.00% | -0.04% | -0.07% | -0.01% | 100% | 100% |
| Overall | -0.04% | -0.09% | 0.10% | -0.03% | -0.11% | 0.09% | 100% | 99% |
| Class D | 0.06% | -0.19% | -0.58% | 0.02% | -0.24% | -0.57% | 100% | 98% |

| Fix B hor 2.0x / ver 1.0x |  | PSNR1 |  |  | PSNR2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LB | Y | U | V | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |
| Class B | 0.03% | 0.21% | 0.13% | 0.02% | 0.23% | 0.15% | 100% | 99% |
| Class C | -0.01% | -0.09% | 0.21% | -0.01% | -0.08% | 0.23% | 100% | 99% |
| Class E | -0.02% | -0.05% | 0.42% | 0.01% | -0.02% | 0.40% | 100% | 99% |
| Overall | 0.00% | 0.04% | 0.23% | 0.01% | 0.06% | 0.24% | 100% | 99% |
| Class D | 0.06% | -0.26% | -0.65% | 0.04% | -0.18% | -0.60% | 100% | 99% |

# References

- B. Bross et al., “Versatile Video Coding (Draft 7)”, JVET-P2001-vE, Geneva, Switzerland, October 2019

- D. Luo et al.,“Description of Core Experiment 1 (CE1): Reference picture resampling filters”,  JVET-O2021, Gothenburg, Sweden, July 2019

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)