| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 17th Meeting: Brussels, BE, 7–17 January 2020 | Document: JVET-Q0452-v1 |
| --- | --- |

| Title: | Modification of up-sampling in MIP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Junyan Huo, Xinwei Li, Qihong Ran, Yanzhuo Ma, Shuai Wan, Fuzheng Yang, Yuanfang Yu, Yang Liu | Email: | jyhuo@mail.xidian.edu.cn xinweili@stu.xidian.edu.cn qhRan@stu.xidian.edu.cn yzma@mail.xidian.edu.cn swan@nwpu.edu.cn fzhyang@mail.xidian.edu.cn yuyuanfang@oppo.com serena@oppo.com |
| Source: | Xidian University, Northwestern Polytechnical University, Guangdong OPPO Mobile Telecommunications Corp., Ltd |  |  |

_____________________________

- Abstract

In VVC Draft 7, a fixed up-sampling order is used to obtain the prediction signal in MIP. The up-sampling process is performed firstly in the horizontal direction and secondly in the vertical direction. In this contribution, it is proposed to perform the up-sampling process in two orders, horzontal-vertical and vertical-horizontal. Then the average of the prediction signals obtained by the two up-sampling orders is the final prediction signal of MIP. It is reported that, under the CTC, the overall performance impact for {Y, U, V} is {-0.02%, -0.04 %, -0.01 %} for AI and {-0.01 %, -0.04 %, 0.00 %} for RA over VTM 7.0. As for Class A1 sequences, the performance impact for {Y, U, V} is {-0.04%, -0.15 %, 0.13 %} for AI and {-0.02 %, -0.16 %, 0.02%} for RA over VTM 7.0. As for Class A2 sequences, the performance impact for {Y, U, V} is {-0.03%, -0.01 %, -0.11 %} for AI and {-0.04 %, 0.00%, -0.08%} for RA over VTM 7.0.

Introduction

In VVC Draft 7[1] matrix-based intra prediction (MIP) is performed as follow:

Figure 1. Matrix weighted intra prediction

Firstly, the reconstructed neighboring samples are averaged to generate a vector. Then, the vector is multiplied with a selected matrix to generate a reduced prediction signal. Finally, the reduced prediction signal is up-sampled to generate the prediction signal.

For a W×H block, the size of reduced prediction signal is predSize×pedSize, then the horizontal up-sampling factor upHor and the vertical up-sampling factor upVer are calculated as follows:

upHor = W / predSize

upVer = H / predSize

The up-sampling process is related to upHor and upVer:

  - if both upHor and upVer are equal to 1, no up-sampling is required

  - if upHor is greater than 1 and upVer is equal to 1, only horizontal up-sampling is required

  - if upHor is equal to 1 and upVer is greater than 1, only vertical up-sampling is required

  - if both upHor and upVer are greater than 1, up-sampling in both horizontal and vertical directions is required

In VVC7, when the up-sampling in both horizontal and vertical directions is required, it is performed firstly in the horizontal direction and secondly in the vertical direction as shown in Figure 2.

Figure 2. The up-sampling process in VVC7(The inputs are the above reference row, the left reference column, and the reduced prediction signal. Interpolation performed horizontally first and then vertically. The red points are the reference points for interpolation, and the yellow points are the points generated by interpolation. The output is the prediction signal of the current block.)

This method makes some points in the left reference column (marked with black circles in Figure 2) not used for the upsampling process.

Proposed Methods

It is proposed to perform the up-sampling process in two orders, respectively, when the up-sampling in both horizontal and vertical directions is required:

- Horizontal firstly and then vertical

- Vertical firstly and then horizontal

Then the two prediction signals obtained by the two orders up-sampling are averaged to generate the final prediction signal, as shown in Figure 3.

Figure 3. The up-sampling process in the proposed method

Experimental results

The experiments were performed based on VTM7.0 software [2] under JVET common test condition [3]. Table 1 present a summary of the results. According to the simulation results, it shows that there is a small gain and performs better on large resolution sequences (Class A1 and Class A2). Besides, there is no impact on both the encoding time and decoding time.

Table 1. Simulation results of the proposed method

|  |  |  | All Intra |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.15% | 0.13% | 100% | 100% |
| Class A2 | -0.03% | -0.01% | -0.11% | 100% | 100% |
| Class B | -0.02% | -0.03% | -0.10% | 100% | 100% |
| Class C | -0.01% | 0.03% | 0.01% | 100% | 100% |
| Class E | 0.00% | -0.05% | 0.07% | 100% | 101% |
| Overall | -0.02% | -0.04% | -0.01% | 100% | 100% |
| Class D | 0.01% | 0.00% | -0.07% | 101% | 100% |
| Class F | 0.00% | -0.02% | -0.02% | 100% | 100% |

|  |  |  | Random Access |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Over VTM-7.0 |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.16% | 0.02% | 100% | 100% |
| Class A2 | -0.04% | 0.00% | -0.08% | 101% | 100% |
| Class B | 0.01% | -0.03% | 0.02% | 100% | 100% |
| Class C | 0.01% | 0.02% | 0.01% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.04% | 0.00% | 100% | 100% |
| Class D | 0.00% | -0.14% | -0.07% | 100% | 100% |
| Class F | 0.00% | 0.09% | 0.01% | 100% | 99% |

Conclusions

In this contribution, it is proposed to use two orders up-sampling to average instead of fixed order upsampling. This method allows all reference pixels to be used in the up-sampling process and makes the upsampling process for larger blocks more robust. A slight gain was obtained with no impact on both the encoding time and decoding time .It is recommended to adopt into VVC.

- 5  References

[1] B. Bross, J. Chen, S. Liu, Y.-K. Wang, Verstatile Video Coding (Draft 7), Document JVET-P2001, Version 14 , Geneva, Switzerland, October 2019

[2] https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tree/VTM-7.0

[3] F. Bossen et al., “JVET common test conditions and software reference configurations for SDR video”, Document  JVET-N1010, Geneva, Switzerland, March 2019

- 6  Patent rights declaration(s)

Xidian University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Northwestern Polytechnical University may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Guangdong OPPO Mobile Telecommunications Corp., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).